| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G423 |
| --- | --- |

| Title: | Non-CE6: Remove potential duplicate modes from the candidate mode list for chroma intra prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Haitao Yang Jiantong Zhou Haoping Yu Section B1, Bantian Huawei Base, Longgang District, Shenzhen, China | Tel: Email: | +86 755 28788991 haitao.yang@huawei.com zhoujiantong@huawei.com haoping.yu@huawei.com |
| Source: | Huawei Technologies Co., Ltd. |  |  |

_____________________________

# Abstract

A simplified chroma intra prediction scheme by removing potential duplicate modes is proposed in this document. With the proposed scheme, only DM and LM modes are used as candidate chroma modes. The Y/U/V BD-Rate are 0.0%/0.2%/0.3% for AI-HE configurations. The BD-Rate using the averaged PSNR is 0.0%.

The four potential duplicate modes in HM4.0 design are planar, vertical horizontal and DC. The four modes may be duplicated with DM mode when the luma mode is one of them. By removing potential candidate modes, dynamic interpretation of the parsed codeword is removed, leading to a simpler decoder and neat WD description. Encoding can also be speed up because the number of candidate mode is reduced to 2.

# Problem Statement

In HM4.0, the candidate mode list for chroma intra prediction comprises six modes: DM, LM, Planar, Vertical, Horizontal, and DC. Among the six modes, DM and LM modes are specific to chroma intra prediction, while the other four modes are shared modes that also apply to luma intra prediction.

The DM mode can actually be considered as the Direct Mode for chroma intra prediction, using one flag to indicate whether the chroma intra prediction mode is the same as the luma intra prediction mode. Obviously, it is possible that DM mode duplicate with the four shared modes in the mode list. In this case, a substitute mode, mode Ver+8 in HM4.0, is used to replace the duplicated shared mode. Therefore, we denote the four shared modes, i.e. Planar, Vertical, Horizontal, and DC as potential duplicate modes since there is a chance one of them is the same as the luma mode and hence the same as the DM mode. The aforementioned classification of the six modes is also described in Table 1.

Table 1. Classification of the six candidate chroma intra prediction modes

| Specific modes (only for chroma prediction) |  | Potential duplicate modes (also for luma prediction) |  |  |  |
| --- | --- | --- | --- | --- | --- |
| DM | LM | Planar | Ver | Hor | DC |

In principle, the two specific modes, both DM and LM, exploit the correlation of luma component and chroma component. DM is supposed to be selected if the texture directionality of luma and chroma components looks similar. On the other hand, LM is supposed to be selected if the sample intensity of luma and chroma components is highly correlated. Because the type of correlation exploited by DM and LM are different, the two modes are supposed to complement with each other in terms of coding performance.

The four potential duplicate modes add additional complexity to both encoder and decoder. Because we need to determine whether it duplicate DM mode based on the luma mode of the same block, and replace it by the substitute mode if it is the duplicate mode. The introduced dependency requires dynamic interpretation of the parsed codeword.

# Basic idea

Here comes the question, it is possible that the potential duplicate modes are removed from the candidate mode list without significant performance drop?

On one hand, remove candidate modes would harm PSNR. On the other hand, the codeword length of remaining modes is decreased, so the bit rate would also be decreased. In the end, a new working point with slightly worse chroma BD-Rate and better luma BD-Rate can be obtained.

# Proposed scheme

We proposed to remove duplicate mode from the candidate mode list for chroma intra prediction. There are two cases according to the configuration parameter chroma_pred_from_luma_enabled_flag.

Case 1:

When chroma_pred_from_luma_enabled_flag is set to 1, there are totally two candidate modes for chroma intra prediction, i.e. DM and LM. In this case, only a flag is used to signalling the selected mode to the decoder.

Case 2:

When chroma_pred_from_luma_enabled_flag is set to 0, there is only one mode, DM mode available for chroma intra prediction. In this case, luma and chroma components always share the same prediction mode, so an encoder does not need to send separate mode information for chroma components.

# Simulation

For Case 2, since only DM mode is enabled, the chroma components always share the same mode with luma component. So it is natural for an encoder to perform joint mode decision (JMD) for luma and chroma when searching for the best mode. For Case 1, since only one luma mode is evaluated for chroma components, it is also reasonable to perform the joint mode decision to select the best luma mode. Then the best luma mode is used as DM mode and competes with LM mode for the best chroma mode. So simulations with JMD on are done to get the primary results. Results with JMD off are also provided for information.

All intra configurations specified in the common test condition [1] are used. HM4.0 is used to obtain the anchor data for comparison.

Table 2. Case 1 with JMD on

Table 3. Case 2 with JMD on

Table 4. Case 1 with JMD off

Table 5. Case 2 with JMD off

# Conclusion

The proposed simplified chroma intra prediction scheme by removing potential duplicate mode has following advantages

- Remove dynamic interpretation of the parsed codeword: simpler decoding and neat WD text

- Totally one or two candidate mode for chroma intra prediction: fast encoding

- New working point with BD-Rate gain for luma component, RD-Rate loss for chroma components

- No obvious color degradation in subjective viewing

Therefore, we recommend adopting the proposed simplified chroma intra prediction scheme to HM.

# References

- F. Bossen, "Common test conditions and software reference configurations," JCT-VC document JCTVC-F900, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP3 and ISO/IEC JTC1/SC29/WG11 6th meeting, Torino, IT, 14-22 July, 2011

# WD text changes

7.2.3 Prediction unit syntax

| prediction_unit( x0, y0, log2CUSize ) { | Descriptor |
| --- | --- |
| if( skip_flag[ x0 ][ y0 ] ) { |  |
| merge_idx[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| } else if( PredMode = = MODE_INTRA ) { |  |
| if( PartMode == PART_2Nx2N && log2CUSize >= Log2MinIPCMCUSize ) |  |
| pcm_flag | u(1) ¦ ae(v) |
| if( pcm_flag ) { |  |
| … |  |
| } else { |  |
| prev_intra_luma_pred_flag[ x0 ][ y0 ] | u(1) ¦ ae(v) |
| if( prev_intra_luma_pred_flag[ x0 ][ y0 ] ) |  |
| mpm_idx[ x0 ][ y0 ] | u(1) ¦ ae(v) |
| else |  |
| rem_intra_luma_pred_mode[ x0 ][ y0 ] | ce(v) ¦ ae(v) |
| if( chroma_pred_from_luma_enabled_flag ) |  |
| intra_chroma_pred_mode[ x0 ][ y0 ] | u(1) ¦ ae(v) |
| } |  |
| } else { /* MODE_INTER */ |  |
| … |  |
| } |  |
| } |  |

8.3.2 Derivation process for chroma intra prediction mode

Input to this process is a luma location ( xB, yB ) specifying the top-left luma sample of the current block relative to the top-left luma sample of the current picture.

Output of this process is the variable IntraPredModeC.

Depending on chroma_pred_from_luma_enabled_flag, IntraPredMode[ xB ][ yB ], and intra_chroma_pred_mode, the variable IntraPredModeC is derived as follows:

- If chroma_pred_from_luma_enabled_flag is equal to 1 and intra_chroma_pred_mode  is equal to 1, IntraPredModeC is set to be LM.

- Otherwise, IntraPredModeC is set to be IntraPredMode[ xB ][ yB ].

# Patent rights declaration(s)

Huawei may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).