| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G424 |
| --- | --- |

| Title: | CE13: Cross-check report of JCTVC-G240 (section 3.5 test 1 and test 4) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Sung-Chang Lim, Jinho Lee, Hui Yong Kim ETRI, 218 Gajeongno, Yuseong-gu, Daejeon, 305-700, Korea | Tel: Email: | +82-42-860-5772 sclim@etri.re.kr jinosoul@etri.re.kr hykim5@etri.re.kr |
| Source: | ETRI |  |  |

_____________________________

# Abstract

This document presents a cross-check report of CE13 section 3.5 test 1 and test 4 proposed by Canon.

# Cross-check result

For CE13 [1] activity, Canon provided the source code and simulation results of section 3.5 test 1 and test 4. Cross-check of Canon’s proposal [2] was carried out by setting the macro definitions in the source code as in Table I and II.

Table I. Macro definition of CE13 section 3.5 test 1

#define CE13_3_5_CANON_TEST_1 1 #if CE13_3_5_CANON_TEST_1 #define CE13_3_5_NO_ZERO_PREDICTORS 1 // No add zero predictors for Merge #define CE13_3_5_NO_SCALED_PREDICTORS 0 // No add scaled predictors for Merge #define CE13_3_5_MAX_NUM_COMBINED 0 // Maximum number of combined predictors #define CE13_3_5_ADD_NON_REDUNDANT_PREDICTORS 1 // Add non-redundant predictors for AMVP/Merge #endif

Table II. Macro definition of CE13 section 3.5 test 4

#define CE13_3_5_CANON_TEST_4 1 #if CE13_3_5_CANON_TEST_4 #define CE13_3_5_NO_ZERO_PREDICTORS 1 // No add zero predictors for Merge #define CE13_3_5_NO_SCALED_PREDICTORS 1 // No add scaled predictors for Merge #define CE13_3_5_MAX_NUM_COMBINED 2 // Maximum number of combined predictors #define CE13_3_5_ADD_NON_REDUNDANT_PREDICTORS 1 // Add non-redundant predictors for AMVP/Merge #endif

The provided source code is studied and the change in the source code of each test is summarized as follows:

- CE13 section 3.5 test 1

- The existing zero merge candidate addition method is removed. And if there is no merge candidate in the merge candidate list, zero merge candidate is added to the merge candidate list.

- Two kinds of merge candidate addition method are implemented according to the direction of each motion vector of the first merge candidate. When motion vectors of the first merge candidate refer to same direction or the first merge candidate contains only one motion vector, a fixed offset value is added to the first merge candidate and the offset-added merge candidate is appended to the merge candidate list. The offset is selected from the offset set, { (-8, 0), (8, 0), (0, -8), (0, 8) }, in order. When motion vectors of the first merge candidate refer to different direction, a fixed offset value is added to the list 0 motion vector of first merge candidate and an inverted offset value is added to the list 1 motion vector of first merge candidate. i.e., when (-8, 0) is added to the list 0 motion vector, then (8, 0) is added to the list 1 motion vector.

- CE13 section 3.5 test 4

- The existing zero merge candidate addition method is removed. And if there is no merge candidate in the merge candidate list, zero merge candidate is added to the merge candidate list.

- Maximum number of combined-bi merge candidate is set equal to 2.

- The existing non-scaled merge candidate addition method is removed.

- Same as CE13 section 3.5 test 1 B.

Simulations were conducted under four conditions (RA-HE, RA-LC, LB-HE, and LB-LC) as specified in common test condition [3]. All the simulation results are exactly matched with the ones provided by Canon. ETRI's cross-check result compared to the anchor are summarized in Table III and IV. Detailed cross-check results of section 3.5 test 1 and test 4 are included in accompanying Excel files.

Table III. Cross-check results of CE13 section 3.5 test 1

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | -0.4% | -0.5% | -0.1% | -0.1% | -0.3% |
| Class B | -0.2% | -0.2% | -0.2% | -0.3% | -0.2% | -0.2% |
| Class C | -0.1% | -0.2% | -0.1% | -0.1% | -0.3% | -0.2% |
| Class D | -0.1% | -0.1% | -0.2% | -0.1% | -0.2% | -0.2% |
| Class E |  |  |  |  |  |  |
| Overall | -0.1% | -0.2% | -0.2% | -0.2% | -0.2% | -0.2% |
|  | -0.1% | -0.2% | -0.2% | -0.2% | -0.2% | -0.2% |
| Enc Time[%] | 105% |  |  | 103% |  |  |
| Dec Time[%] | 101% |  |  | 101% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.3% | -0.3% | -0.4% | -0.4% | -0.4% | -0.3% |
| Class C | -0.2% | -0.3% | -0.3% | -0.2% | -0.2% | -0.4% |
| Class D | -0.2% | -0.1% | -0.7% | -0.2% | 0.0% | -0.5% |
| Class E | 0.1% | 0.2% | 0.7% | 0.1% | 1.7% | 0.9% |
| Overall | -0.2% | -0.2% | -0.2% | -0.2% | 0.2% | -0.1% |
|  | -0.2% | -0.1% | -0.2% | -0.2% | 0.1% | -0.1% |
| Enc Time[%] | 104% |  |  | 102% |  |  |
| Dec Time[%] | 101% |  |  | 101% |  |  |

Table IV. Cross-check results of CE13 section 3.5 test 4

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.1% | -0.3% | -0.3% | 0.0% | -0.1% | -0.3% |
| Class B | -0.2% | -0.1% | -0.2% | -0.2% | -0.2% | -0.2% |
| Class C | 0.1% | -0.1% | 0.0% | 0.0% | -0.1% | 0.0% |
| Class D | 0.1% | 0.0% | 0.0% | 0.1% | -0.1% | -0.2% |
| Class E |  |  |  |  |  |  |
| Overall | 0.0% | -0.1% | -0.1% | 0.0% | -0.1% | -0.2% |
|  | 0.0% | -0.1% | -0.1% | -0.1% | -0.1% | -0.2% |
| Enc Time[%] | 107% |  |  | 103% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.2% | -0.3% | -0.4% | -0.2% | -0.3% | -0.2% |
| Class C | 0.1% | -0.1% | -0.1% | 0.1% | -0.1% | -0.2% |
| Class D | 0.0% | -0.1% | -0.3% | 0.0% | -0.4% | -0.3% |
| Class E | 0.2% | 1.0% | 1.1% | 0.4% | 1.3% | 0.8% |
| Overall | 0.0% | 0.1% | 0.0% | 0.0% | 0.1% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% |
| Enc Time[%] | 105% |  |  | 103% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

# Conclusions

The result of CE13 section 3.5 test 1 and test 4 presented by Canon is confirmed by ETRI.

# References

[1]	J. Jung, B. Bross, J. Chen, P. Onno, and M. Zhou, "Description of Core Experiment CE13: Motion data parsing robustness and throughput," JCTVC-F913, Jul. 2011.

[2]	G. Laroche, T. Poirier, and P. Onno, "CE13: Experiment regarding section 3.5," JCTVC-G240, Jul. 2011.

[3]	Frank Bossen, "Common test conditions and software reference configurations," JCTVC-F900, Jul. 2011.