| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G428_r1 WG11 Number: m21992 |
| --- | --- |

| Title: | CE3: Cross-check report for Sony's tool 7 (JCTVC-G390) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Takeshi Chujoh 1, Komukai-Toshiba-cho, Saiwai-ku, Kawasaki, 212-8582, Japan | Tel: Email: | +81-44-549-2152 takeshi.chujoh@toshiba.co.jp |
| Source: | TOSHIBA Corporation |  |  |

_____________________________

# Abstract

Cross-checking results of Sony's tool 7 in CE3 (JCTVC-G390) are shown.  Three source codes were distributed by Sony and they were run on an independent platform.  All local decoded images by encoder matched decoded images by decoder perfectly.  All bitrate and PSNR were identical with proponent's results perfectly.  All values of complexity analyses of decoder side were identical with proponent’s values.  The results of complexity analyses of Excel sheet were reasonable.  Detailed experimental results show attached Excel files.  Sony’s tool 7 in CE3 (JCTVC-G390) has been cross-verified independently.

# Software check

Totally three source codes that were different filters for luminance and chrominance block boundaries were distributed.  Although there are several changes from original proposal, JCTVC-F100, they seem to be acceptable level.

# Experimental results

The simulation was conducted for all sequences of WQVGA, WVGA, 720p, 1080p and cropped 4kx2k based on the recommended simulation common conditions, JCTVC-F800.  Error: Reference source not found indicates the average results of coding efficiency compared to each anchor.  According to JCTVC-F900, the interpolation filter of anchor is 8-tap DCT-IF.  Used platform is that the OS is Windows 7 64-bit; the CPU is Core™i7-2600, 3.4GHz and the compiler is Visual C++ 2008.  All local decoded images by encoder match decoded images by decoder perfectly.  All bitrate and PSNR are identical with proponent's results perfectly.  All values of complexity analyses of decoder side are identical with proponent’s values.

Table 1	Summary of results

|  | a |  |  | b |  |  | c |  |  | d |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V | Y | U | V |
| Gain Ave.[%] | 0.23% | 0.59% | 0.54% | 0.25% | 0.55% | 0.60% | 0.00% | 0.43% | 0.48% | 0.41% | 0.79% | 0.85% |
| Gain Best [%] | -0.23% | -0.35% | -0.99% | -0.15% | -0.94% | -0.69% | -0.80% | -0.83% | -0.98% | -0.05% | -0.64% | -0.94% |
| Gain Worst [%] | 1.43% | 3.10% | 3.10% | 1.51% | 3.40% | 3.97% | 0.35% | 1.81% | 3.44% | 0.88% | 2.62% | 3.81% |
| Gain Ave.[%] (with LDP) | 0.23% | 0.59% | 0.56% | 0.27% | 0.57% | 0.64% | 0.03% | 0.43% | 0.48% | 0.30% | 0.67% | 0.72% |
| Gain Best [%] (with LDP) | -0.23% | -0.69% | -0.99% | -0.15% | -0.94% | -0.69% | -0.80% | -2.00% | -1.36% | -0.40% | -2.00% | -1.36% |
| Gain Worst [%] (with LDP) | 1.52% | 3.45% | 3.10% | 1.92% | 3.59% | 3.97% | 0.41% | 2.43% | 3.44% | 0.88% | 2.62% | 3.81% |
| Enc Time[%] | 101.79% |  |  | 113.37% |  |  | 102.26% |  |  | 97.79% |  |  |
| Dec Time[%] | 100.02% |  |  | 98.80% |  |  | 99.88% |  |  | 100.20% |  |  |
| Mult Ave.[%] | -8.98% |  |  | -8.89% |  |  | -4.94% |  |  | -5.13% |  |  |
| Mult Worst[%] | -27.59% |  |  | -27.59% |  |  | -20.69% |  |  | -32.76% |  |  |
| Adds Ave.[%] | -9.46% |  |  | -9.38% |  |  | -5.03% |  |  | -5.22% |  |  |
| Adds Worst[%] | -27.51% |  |  | -27.51% |  |  | -20.11% |  |  | -32.54% |  |  |
| Access[%] | -16.88% |  |  | -16.84% |  |  | -11.74% |  |  | -12.21% |  |  |
| MemBand(2D) Ave.[%] | -17.00% |  |  | -17.00% |  |  | -12.44% |  |  | -13.18% |  |  |
| MemBand(2D) Max.[%] | -8.84% |  |  | -8.01% |  |  | -5.74% |  |  | -5.51% |  |  |
| MemBand(2D) Worst[%] | -43.75% |  |  | -43.75% |  |  | -37.50% |  |  | -57.81% |  |  |
| MemBand(1D) Ave.[%] | -17.29% |  |  | -17.29% |  |  | -12.61% |  |  | -13.29% |  |  |
| MemBand(1D) Max.[%] | -8.38% |  |  | -7.87% |  |  | -5.69% |  |  | -5.44% |  |  |
| MemBand(1D) Worst[%] | -33.33% |  |  | -33.33% |  |  | -25.00% |  |  | -59.09% |  |  |

# Subjective quality

Regarding subjective quality, in order to check ringing noises of interpolation filter, CE3 (JCTVC-F903) [1] recommends that three configurations should be checked and attached the specified still images on the report.  The decoded still images are attached.

It seems to be difficult to find the difference of subjective quality between proposals and HM40 anchor.

# Conclusions

Sony’s tool 7 in CE3 (JCTVC-G390) has been cross-checked independently.

# References

[1] 	T. Chujoh and E. Alshina, “CE3: Motion compensation,” Joint Collaborative Team on Video Coding, JCTVC-F903, Torino, July 2011.

# Patent rights declaration(s)

TOSHIBA Corporation may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).