| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G430 WG11 Number: m21994 |
| --- | --- |

| Title: | An experimental comparison of memory bandwidth between 8-bit and 10-bit coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Takeshi Chujoh TOSHIBA Corporation Keiichi Chono NEC Corporation | Email: Email: | takeshi.chujoh@toshiba.co.jp chono@ct.jp.nec.com |
| Source: | TOSHIBA Corporation and NEC Corporation |  |  |

_____________________________

# Abstract

An experimental comparison between 8-bit and 10-bit coding is reported.  Although the coding efficiency of 10-bit coding has been discussed at previous several meetings, the quantitative discussion of memory bandwidth has not been sufficient.  By using the measurement module of memory bandwidth that has been developed by AHG8 of reference memory compression and has been distributed to CE3 of motion compensation, the numerical result of memory bandwidth is shown.  As experimental results, the memory bandwidth increase of 10-bit coding compared to 8-bit coding is an average of 37% and a maximum of 47%.  Since the reductions of memory bandwidth have been discussed in AHG8 and CE3, those discussions should be considered.

# Memory bandwidth measurement

Regarding memory bandwidth measurement, in order to discuss reference memory compression and motion compensation, AHG8 of reference memory compression [1] has developed a module and it has been distributed to CE3 of motion compensation [2].  The detailed specification of memory architecture is described in JCTVC-C007 [3].  This module can measure an average of memory access frame by frame and a maximum of memory access frame by frame.  Mainly three parameters that are access block size for luminance and chrominance respectively, alignment number of bits and burst length are provided.  CE3 defines typical two parameter sets.

Table 1	Parameter set for memory bandwidth measurement

|  | Access block size for luma | Access block size for chroma | Alignment | Burst length |
| --- | --- | --- | --- | --- |
| Case 1 (2D) | 4x2 | 4x2 | 64-bit | 128-bit |
| Case 2 (1D) | 8x1 | 8x1 | 32-bit | 64-bit |

# Experimental results

Error: Reference source not found indicates the results of 10-bit coding compared to 8-bit coding.  Since the high efficiency test condition of JCTVC-F903 [4] uses 10-bit coding, it is compared to 8-bit coding of inter coding with high efficiency test condition.  Used platform is that the OS is Windows 7 64-bit; the CPU is Core™i7-2600, 3.4GHz and the compiler is Visual C++ 2008.  The detailed numerical results are shown in the attached excel sheet.

Table 2	Summary of results (10-bit compared to 8-bit)

|  | Random Access HE |  |  | Low delay B HE |  |  | Low delay P HE |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -3.7% | -19.6% | -20.8% |  |  |  |  |  |  |
| Class B | -2.9% | -9.2% | -8.8% | -3.0% | -10.1% | -11.1% | -2.8% | -9.8% | -11.3% |
| Class C | -1.6% | -3.6% | -4.3% | -1.7% | -4.6% | -5.0% | -1.5% | -4.1% | -4.3% |
| Class D | -1.1% | -3.2% | -3.5% | -1.1% | -5.9% | -5.7% | -0.9% | -4.9% | -4.9% |
| Class E |  |  |  | -6.6% | -17.2% | -13.7% | -5.9% | -17.7% | -14.1% |
| Overall | -2.4% | -8.9% | -9.3% | -2.9% | -9.0% | -8.7% | -2.6% | -8.7% | -8.5% |
|  | -2.4% | -8.9% | -9.3% | -2.8% | -9.0% | -8.7% | -2.6% | -8.7% | -8.5% |
| Enc Time[%] | 100% |  |  | 100% |  |  | 100% |  |  |
| Dec Time[%] | 111% |  |  | 111% |  |  | 113% |  |  |
| MemBand(2D) Ave.[%] | 34% |  |  | 34% |  |  | 36% |  |  |
| MemBand(2D) Max.[%] | 48% |  |  | 49% |  |  | 45% |  |  |
| MemBand(1D) Ave.[%] | 34% |  |  | 34% |  |  | 36% |  |  |
| MemBand(1D) Max.[%] | 48% |  |  | 48% |  |  | 44% |  |  |

# Conclusions

From the quantitative viewpoint, the results of memory bandwidth measurement of 8-bit and 10-bit coding have been shown.  Since the reductions of memory bandwidth have been discussed in AHG8 and CE3, those discussions should be considered.

# References

[1] 	K. Chono, T. Chujoh, C.S. Lim, A. Tabatabai and M. Zhou, “JCT-VC AHG report: Reference pictures memory compression (AHG 8),” Joint Collaborative Team on Video Coding, JCTVC-F008, Torino, July 2011.

[2] 	T. Chujoh and E. Alshina, “CE3: Motion Compensation,” Joint Collaborative Team on Video Coding, JCTVC-F903, Torino, July 2011.

[3] 	K. Chono, T. Chujoh and C.S. Lim, “JCT-VC AHG report: Memory compression,” Joint Collaborative Team on Video Coding, JCTVC-C008, Daegu, Oct. 2010.

[4] 	F. Bossen, “Common test conditions and software reference configurations,” Joint Collaborative Team on Video Coding, JCTVC-F900, Torino, July 2011.

# Patent rights declaration(s)

TOSHIBA Corporation may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

NEC Corporation may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).