| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G431 WG11 Number: m21995 |
| --- | --- |

| Title: | Non-CE9: Cross-verification of JVC Kenwood proposal on AMVP syntax for bi-prediction (JCTVC-G182) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Takeshi Chujoh 1, Komukai-Toshiba-cho, Saiwai-ku, Kawasaki, 212-8582, Japan | Tel: Email: | +81-44-549-2152 takeshi.chujoh@toshiba.co.jp |
| Source: | TOSHIBA Corporation |  |  |

_____________________________

# Abstract

Cross-checking results of JVC Kenwood’s proposal on AMVP syntax for bi-prediction (JCTVC-G182) are shown.  The software that is provided by JVC Kenwood is used and run on an independent platform.  All local decoded images by encoder match decoded images by decoder perfectly.  All bitrate and PSNR are identical with proponent's results perfectly.  The results of encoding and decoding time are almost the same as anchor.  Detailed experimental results show attached Excel files.  JVC Kenwood’s proposal on AMVP syntax for bi-prediction (JCTVC-G182) has been cross-verified independently.

# Software check

Three parts have been changed.  The modification seems to impact on high efficiency condition only.

# Experimental results

The simulation was conducted for all sequences of WQVGA, WVGA, 720p, 1080p and cropped 4kx2k based on the recommended simulation common conditions, JCTVC-F900.  Error: Reference source not found indicates the average results of coding efficiency compared to random access and low delay bidirectional structure anchor on both high efficiency and low complexity condition.  Used platform is that the OS is Windows 7 64-bit; the CPU is Core™i7-2600, 3.4GHz and the compiler is Visual C++ 2008.  All local decoded images by encoder match decoded images by decoder perfectly.  All bitrate and PSNR are identical with proponent's results perfectly.

Table 1 Results of coding efficiency and Enc/Dec times

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% |
| Class B | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% |
| Class E |  |  |  |  |  |  |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 101% |  |  | 101% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.3% | -0.2% | 0.0% | 0.0% | 0.0% |
| Class E | 0.0% | -0.1% | 0.6% | 0.0% | 0.0% | 0.0% |
| Overall | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 101% |  |  | 101% |  |  |
| Dec Time[%] | 100% |  |  | 101% |  |  |

# Conclusions

We have cross-checked JVC Kenwood’s proposal on AMVP syntax for bi-prediction (JCTVC-G182) independently.

# Patent rights declaration(s)

TOSHIBA Corporation may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).