| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G432 |
| --- | --- |

| Title: | CE2: Cross-verification of Qualcomm’s Overlapped Block Motion Compensation(JCTVC-G749) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Cross-check |  |  |
| Author(s) or Contact(s): | Sukho Lee, Seunghyun Cho, Nakwoong Eum 218 Gajeongno Yuseong-gu Daejeon Korea | Tel: Email: | +81-42-860-6171 shlee99@etri.re.kr |
| Source: | ETRI |  |  |

_____________________________

# Abstract

This is a cross verification of contribution JCTVC-G749 (Qualcomm’s Overlapped Block Motion Compensation) for six test conditions. The source code was provided by Qualcomm and was based on HM-4.0. We compiled, inspected, and ran the code with Random Access, Low delay B and Low delay P for high efficiency and low complexity configurations. Results of BD-rate were identical to those provided by Qualcomm.

# Test condition

Our computing platform used for cross-verification tests is a single computer, which contains:

- CPU: quad-core Intel i7 3.2 GHz

- Memory: 24 GB RAM

- OS: Centos 5.6 64-bit Linux

The encoder and decoder executable files were generated with g++ 4.1.2. For encode, several machines of above identical platform were used. For decode, only one computer was used.

# Result

## BD-rate

The BD-Rates for Qualcomm’s Overlapped Block Motion Compensation are summarized as following:

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-Rate | U BD-Rate | V BD-Rate | Y BD-Rate | U BD-Rate | V BD-Rate |
| Class A | -0.4% | -1.5% | -1.0% | -0.5% | -0.6% | -0.7% |
| Class B | -0.5% | -0.9% | -0.8% | -0.5% | -0.8% | -0.7% |
| Class C | -0.9% | -1.3% | -1.2% | -0.8% | -1.1% | -1.3% |
| Class D | -0.7% | -1.0% | -1.0% | -0.6% | -0.9% | -0.8% |
| Class E |  |  |  |  |  |  |
| Overall | -0.6% | -1.2% | -1.0% | -0.6% | -0.8% | -0.8% |
|  | -0.6% | -1.2% | -1.0% | -0.6% | -0.8% | -0.8% |
| Enc Time[%] | 93% |  |  | 101% |  |  |
| Dec Time[%] | 101% |  |  | 102% |  |  |
| Table2-1: Results for Random Access |  |  |  |  |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y BD-Rate | U BD-Rate | V BD-Rate | Y BD-Rate | U BD-Rate | V BD-Rate |
| Class A |  |  |  |  |  |  |
| Class B | -0.7% | -1.1% | -0.9% | -0.6% | -0.6% | -0.5% |
| Class C | -1.0% | -1.4% | -1.4% | -0.9% | -0.8% | -1.3% |
| Class D | -1.1% | -1.1% | -0.7% | -0.8% | -0.6% | -0.9% |
| Class E | -1.1% | -0.7% | -0.8% | -1.0% | -0.7% | -0.7% |
| Overall | -0.9% | -1.1% | -0.9% | -0.8% | -0.7% | -0.8% |
|  | -0.9% | -1.1% | -1.0% | -0.8% | -0.7% | -0.8% |
| Enc Time[%] | 97% |  |  | 98% |  |  |
| Dec Time[%] | 101% |  |  | 101% |  |  |
| Table2-2: Results for Low delay B |  |  |  |  |  |  |
|  | Low delay P HE |  |  | Low delay P LC |  |  |
|  | Y BD-Rate | U BD-Rate | V BD-Rate | Y BD-Rate | U BD-Rate | V BD-Rate |
| Class A |  |  |  |  |  |  |
| Class B | -1.4% | -1.6% | -1.4% | -1.7% | -1.2% | -1.3% |
| Class C | -2.0% | -2.0% | -2.2% | -1.9% | -1.5% | -1.9% |
| Class D | -2.7% | -2.3% | -2.2% | -2.5% | -1.7% | -2.2% |
| Class E | -2.2% | -1.7% | -1.5% | -2.3% | -1.7% | -1.6% |
| Overall | -2.0% | -1.9% | -1.8% | -2.0% | -1.5% | -1.7% |
|  | -2.1% | -1.9% | -2.0% | -2.1% | -1.5% | -1.7% |
| Enc Time[%] | 98% |  |  | 105% |  |  |
| Dec Time[%] | 101% |  |  | 102% |  |  |

Table2-3: Results for Low delay P

# Conclusions

The code and results are verified and are conformant to the results stated by Qualcomm.

# References

- L. Guo, P. Chen, I. Chong, “CE2: Overlapped Block Motion Compensation for 2NxN and Nx2N Motion Partitions”, JCTVC-F299, Torino, July 2011.

- L. Guo, I.S. Chong, X. Wang, M. Karczewicz, “CE2: Overlapped Block Motion Compensation”, JCTVC-G749, Geneva, November 2011

- F. Bossen, “Common test conditions and software reference configurations” JCTVC-F900, Torino, July 2011

- X. Zheng, I.Chong, I. Kim, M. Narroschke “Core Experiment 2: Motion Partitioning and OBMC”, JCTVC-F902, Torino, July 2011

# Patent rights declaration(s)

ETRI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).