| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G434 |
| --- | --- |

| Title: | CE4 subtest 3: Quantization matrix for HEVC based on JCTVC-F362 and F475 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yoshitaka Morigami Junichi Tanaka Teruhiko Suzuki 2-10-1, Osaki, Shinagawa-ku , Tokyo, 141-8610 JAPAN | Tel: Email: | +81-50-3750-2740 Yoshitaka.Morigami@jp.sony.com Tanaka.Junichi@jp.sony.com teruhikos@jp.sony.com |
| Source: | Sony Corporation |  |  |

_____________________________

# Abstract

This contribution reports the results of CE4 subtest 3, Quantization matrix. In this subtest, three methods, F362 (AVC method), F085 (TI method), and F475 (Sony method) are compared. This contribution reports the results of F362 and F475. The coding efficiency of matrix coding is evaluated in test 1 and the overhead to support quantization matrix among bitstream is evaluated in test 2. The default matrix of F362 was included in the Excel sheet attached to the CE description JCTVC-F904. The subjective quality improvement using the default matrix is evaluated during JCTVC Geneva meeting. The subjective quality is improved by using quantization matrix compared with HM4. Compared with F362, F475 can improve compression efficiency up to 10 times better. Based on this results, this contribution propose to include quantization methods in HM5.

# Introduction

The requirements and the current practices of quantization matrix are summarized in JCTVC-E056. In existing coding standard, e.g. MPEG2 and AVC, quantization matrix is introduced to improve subjective quality. Quantization matrix is widely used in consumer and professional video products, e.g. video camera, Blu-Ray disc, etc. Such well establish method in the existing standard should be supported by HEVC. The requirements and the current practices of quantization matrix are reported in JCTVC-E056.

Data size of quantization matrix, especially for large CU, is large and efficient encoding method to support quantization matrix is desired.

If compression mode is DPCM mode only, as in AVC, it needs to enormous bits. The efficient quantization matrix to the JCTVC-E073[1] was proposed in Geneva meeting. In Torino, three contribution proposed quantization matrix for HEVC.

JCTVC-F362: AVC method (DPCM in zigzag scan + exp-Golumn coding)

JCTVC-F085: TI method (down sampling, symmetry processing, unsigned exp-Golomb coding)

JCTVC-F475: Sony method (prediction, quantization and residual coding)

JCTVC-F362 and F475 can support lossless coding of quantization matrix. Since NSQT was adopted in Torino, quantization matrix for NSQT was also included in each proposal.

## Quantization matrix for NSQT

The quantization matrices for NSQT are generated NxN matrix both encoder and decoder. Therefore matrices for NSQT are not transmitted. The generation method is illustrated in Figure 2.

- Vertically long rectangular: The quantization coefficient of mxN uses the value of the four columns of intervals in NxN.

- Horizontally long rectangular: The quantization coefficient of Nxm is uses with the value of the four lines of intervals in NxN.

Figure 1: Generation method of quantization matrix for NSQT

# Simulation results

## Evaluation of coding method of quantization matrices

The evaluation method of quantization matrix is summarized in 6.3 in JCTVC-F904. In this contribution, coding performance of proposed method (F475) is evaluated compared with AVC method (F362).

The following tests were conducted.

Test 1: Results of compression performance of matrix itself (matrix only)

Test 2: Its impact on overall HM performance

In case of the test 1, the coding efficiency of matrix only is investigated. The proposed method is compared with AVC method. In case of lossy coding, coding error of matrix coefficients is also evaluated. In the test 2, QMs are changed frame by frame, and are encoded at each frame. Then the impact on the overall performance can be studied. In this case, the ratio of the generated bits for matrix coefficients and the other remaining bits and BD rate/PSNR are measured.

The two matrices used for experiments are created. One is symmetry structure, another is asymmetry structure. The matrices are summarized in the attached Excel file. The same QM are used in both.

Different quantization matrices for each 4x4/8x8/16x16/32x32 size can be used for Intra/Inter of Luma/Chroma respectively.

## Test 1: Results of compression performance of matrix itself (matrix only)

The test condition is summarized as followings.

Five types of test conditions are used in the experiments. The conditions are summarized in Table 1. “Cond0” are AVC method. “Cond1” corresponds with lossless coding for multiplication representation. “Cond2-4” corresponds with lossy coding for multiplication representation. Region quantization 0-3 are shown in Table1. When larger quantizer is used, coding efficiency is improved but coding error is increased.

|  | mode | Region0 | Region1 | Region2 | Region3 |
| --- | --- | --- | --- | --- | --- |
| Cond0 | AVC | none | none | none | none |
| Cond1 | lossless | 1 | 1 | 1 | 1 |
| Cond2 | lossy | 2 | 2 | 2 | 2 |
| Cond3 | lossy | 2 | 2 | 7 | 7 |
| Cond4 | lossy | 6 | 6 | 8 | 8 |

Table 1: Test conditions (Setting of quantization)

The proposal is evaluated as follows.

- Generated bits for luma matrix coefficients for 4x4/8x8/16x16/33x32, and Intra/Inter

- Coding error of matrix coefficients (max error)

### Results

The Results of compression performance of matrix itself are shown in table 2 and 3. Generated bits of Sony method have smaller than generated bits of AVC method. The Sony method has a small average error in lossy coding too. (AVC method only supports lossless coding)

Additional different value of Luma and Chroma are experimented. (those matrices are attached in zip file as qumatrix_additional tested_symmetry1.txt and qumatrix_additional tested_asymmetry1.txt) Simulation results are shown in table 4 and 5. Generated bit of Sony method also has smaller than generated bit of AVC method.

Table 2: results of AVC method

Table 3: results of Sony’s method

Table 4: results of AVC method (additional tested)

Table 5: results of Sony’s method (additional tested)

## Test 2: Impact on overall performance

The impact to support quantization matrix on the overall performance is investigated. The test condition is summarized in Table 6.

| software | HM4.0 |
| --- | --- |
| frame number | Full frame |
| condition | All condtion |
| Quantization matrix transmit period | each frame |
| matrix | symmetry structure, asymmetry |
| Transmission | for each, frame, different matrices are sent. |
| coding method | AVC method and sony method |
| Region quantization | Cond1 |

Table 6: Test conditions

In this experiment, different quantization matrices for each 4x4/8x8/16x16/32x32 size are used for Intra/Inter of Luma/Chroma respectively. And the matrices are changed in each frame. This is a kind of worst case to transmit quantization matrix, however, such operational practices exist in the commercial products as reported in JCTVC-F475.

The ratio of the generated bits for matrix coefficients and the other remaining bits is measured. BD rate calculated by using AVC as anchor is also measured.

### Results

The ratio of the generated bits for matrix coefficients and the other remaining bits are shown in Error: Reference source not found-10. (Other data are attached zip file.) In case of symmetry matrix of cond1 of AVC method, the ratio is 5.0% for Nebuta and 616.2 % for Basket ball Pass. In case of asymmetry matrix of cond1 of AVC method, the ratio is 8.7% for Nebuta and 725.1 % for Basket ball Pass. Significant amount of bits are consumed to transmit quantization matrices in case of AVC method. Therefore efficient coding method of matrix coefficient is required.

In case of symmetry matrix of cond1 of proposed method, the ratio is 0.5% for Nebuta and 109.5 % for Basket ball Pass. In case of asymmetry matrix of cond1 of proposed method, the ratio is 0.9% for Nebuta and 388.0% for Basket ball Pass.

In case of symmetry matrix of cond4 of proposed method, the ratio is 0.4% for Nebuta and 87.2 % for Basket ball Pass. In case of asymmetry matrix of cond1 of proposed method, the ratio is 0.5% for Nebuta and 126.2% for Basket ball Pass.

Table 7: Result of symmetry matrix for Intra LC (cond1)

Table 8: Result of asymmetry matrix for Intra LC (cond1)

Table 9: Result of symmetry matrix for Intra LC (cond4)

Table 10: Result of asymmetry matrix for Intra LC (cond4)

Sony method can change compression ratio and the accuracy of the matrix optionally.

# WD test

WD test of AVC method is attached. Detail syntax and semantics are specified.

# Conclusion

This contribution reports the test result of the quantization matrix, described in JCTVC-F475 of Torino meeting. Compared to the AVC method of quantization matrix, the proposed method demonstrates 10x more compression for cond4. In additional, average errors are small. This contribution propose to include quantization methods in HM5.

# Patent rights declaration(s)

Sony Corporation may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Annex

## Transmission of quantization matrices by new NAL unit

New NAL unit, QMPS(Quantization Matrix Parameter Set), to transmit quantization matrices is proposed. The detail is summarized in JCTVC-E073. It is better to define new NAL unit to transmit the parameters which is changed dynamically in a bitstream. It is possible to transmit more than one QMPS NAL unit in a bitstream. Then the value of quantization matrices in QMPS NAL unit can be updated at each slice. To support this, an identifier, QMPS_ID (qmps_present_flag : 1bit + qmps_id ue()) is introduced in slice header. The detail syntax is summarized in Annex A.

## Coding method of quantization matrix

In AVC, quantization matrix is coded by DPCM. However, data size coded by DPCM will be large, because transform size is larger in HEVC. Data size of matrix coefficients was evaluated by experiments. It was found that data size is about 16 times larger than AVC (4x4 + 8x8), if matrix size is extended to 32x32. Therefore efficient coding method of quantization matrix is required in HEVC.

The diagram of decoder of quantization matrix of the proposed algorithm is shown in Error: Reference source not found. The block diagram of encoder is shown in Error: Reference source not found. At the first step, each matrix coefficient is predicted. Three prediction modes are proposed in this contribution. Then residue is coded. In each mode, lossless and lossy coding are supported. Quantization of matrix coefficient is introduced. If quantization is not applied, then matrix can be coded in lossless. If quantization is applied, matrix coefficients are coded by lossy coding and data size can be reduced.

Figure 2: Block diagram of decoder

Figure 3: Block diagram of encoder

## Prediction

Two prediction modes, copy mode and DPCM mode, are proposed. The detail of each prediction mode is described in this sub-clause.

### Copy mode

In this mode, current matrix coefficients are predicted from previously coded matrix coefficients. Previously coded matrix, which is referred by identifiers (QMPS_ID, SizeID, and MatrixID), is copied to the predictor. Then residue is calculated using this predictor. When the size of reference matrix is different from that of the current matrix, up-conversion or down-conversion of matrix is applied by linear interpolation. When the reference ID (QMPS_ID / SizeID / MatrixID) is equal to current ID, the default matrix is used for predictor. Example of copy mode is shown in Error: Reference source not found.

Figure 4: Example of copy mode

### DPCM mode

In this mode, matrix coefficients are coded by DPCM, like AVC. Matrix coefficients are coded by DPCM in zigzag scan order. Quantization can be applied after DPCM.

### Quantization

Quantization can be applied for the residual error signal of matrix coefficients or a DPCM signal. Matrix coefficients are classified into four domains as shown in Error: Reference source not found. Error: Reference source not found is an example for 8x8 matrix. Index is used to refer to each domain and different quantization scale can be used. The same quantization scale in each frequency domain is used for all transform size, e.g. 4x4, 8x8, 16x16 and 32x32, within one QMPS.

Figure 5: Quantization domain index (8x8)

### Residual coding

#### Coding of residual signal

In case of copy mode and axis mode, residual error of prediction can be coded. Residual error is quantized for lossy coding. Block diagram of residual coding is shown in Error: Reference source not found. Residual error of prediction is coded either DPCM coding or run length coding. In DPCM, residual error of prediction is coded using DPCM in raster scan order by VLC. In run length coding, residual error of prediction is converted to combination of zero run and level (the value of non-zero value) in zigzag scan order. Each zero run and level is coded by VLC respectively.

Figure 6: Block diagram of residual decoding

### VLC

DPCM coding signal and run length coding signal are coded by one of the 11 VLC tables defined in LCEC. The indicator to select VLC table is explicitly signaled in a bitstream.