| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G435 |
| --- | --- |

| Title: | CE1 subtest C: Cross-checking reports of PIPE/V2F for low complexity entropy coding (JCTVC-G458) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Akiyuki Tanizawa Tomoo Yamakage 1, Komukai-Toshiba-cho, Saiwai-ku, Kawasaki, 212-8582, JAPAN | Tel: Email: | +81-44-549-2152 akiyuki.tanizawa@toshiba.co.jp |
| Source: | TOSHIBA Corporation |  |  |

_____________________________

# Abstract

This contribution reports cross-checking results of PIPE/V2F for low complexity entropy coding in the subtest C on Core Experiment 1 of Entropy Coding. Two variations of Test8 and Test10 were tested on HM common coding conditions. The verification tasks have been done successfully and the results match those provided by proponent.

Experimental results show that BD-bitrate of Test8 is -2.9%, -3.8%, -3.2% and -3.1% for IO-LC, RA-LC, LB-LC and IO-LC compared with HM anchor, respectively. The negative value means gain. Average decoding times are 103%, 102%, 101% and 100% for IO-LC, RA-LC, LB-LC and IO-LC, respectively. BD-bitrate of Test10 is -2.8%, -3.4%, -2.9% and -2.9% for IO-LC, RA-LC, LB-LC and IO-LC compared with HM anchor, respectively. Average decoding times are 104%, 101%, 100% and 99% for IO-LC, RA-LC, LB-LC and IO-LC, respectively.

# Cross-checking

There are 17 kinds of configurations of tools in this subtest C of CE1 as follows;

Table 1. List of tests to be conducted as combinations of tools

| Test | V2F | V2V | BAC | LC mod. | MC mod. | PMU off | Multi bin | Low delay | 8-bit Init | Alt. PMU | RDOQ off | RDO PMU off | TBC | Config. tested |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  |  | x | x |  |  | n/a |  | x |  |  |  |  | LC |
| 2 |  | x |  | x |  |  | x |  | x |  |  |  | x | LC |
| 3 |  | x |  | x |  |  | x | x | x |  |  |  | x | LC |
| 4 |  | x |  |  |  |  |  |  | x |  |  |  | x | HE, LC |
| 5 |  | x |  |  |  |  |  | x | x |  |  |  | x | HE |
| 6 |  |  | x |  |  |  | n/a |  | x |  |  |  |  | HE, LC |
| 7 |  |  | x |  |  |  | n/a |  | x | x |  |  |  | HE |
| 8 | x |  |  |  |  |  |  |  | x |  |  |  |  | LC |
| 9 | x |  |  |  |  |  |  | x | x |  |  |  |  | LC |
| 10 | x |  |  |  | x |  |  | x | x |  |  |  |  | LC |
| 11 |  | x |  |  |  |  |  |  | x | x |  |  | x | HE |
| 12 |  |  | x |  |  |  |  |  |  |  | x | x |  | LC |
| 13 |  |  | x |  |  | x |  |  |  |  | x |  |  | LC |
| 14 |  |  | x | x |  |  | n/a |  | x |  | x |  |  | LC |
| 15 |  | x |  | x |  |  | x |  | x |  | x |  | x | LC |
| 16 | x |  |  | x |  |  | x |  | x |  | x |  |  | LC |
| 17 |  |  | x |  |  |  |  |  |  |  |  |  | x | HE, LC |

This contribution reports the cross-checking results of Test 8 and Test 10. The software based on HM version 4.0 was provided on Sep. 22 2011 by Mitsubishi. The software was complied fine and the cross-checking was performed on 64bit environments. The simulation has been conducted under CE common test conditions [1], [2] for only low complexity condition in the case of I-only, random access, low delay B and low delay P coding structures.

Table 1 shows that BD-rate and relative encoding/decoding time of Test8 compared with HM anchor and Table 2 shows BD-rate and relative encoding/decoding time of Test10 compared with HM anchor. The BD-rate for luma is 2.8% to 3.8%, however the BD-rate for chroma is considerably worse compared with the HM anchor. It is thought that the balance between luma and chroma is not optimal.

Table 2. BD-Rate[%] and relative encoding/decoding time[%] (Test8)

|  | All Intra LC |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -3.2% | 4.0% | 4.4% | -5.6% | 11.1% | 10.0% |
| Class B | -3.1% | 3.6% | 4.6% | -4.6% | 6.5% | 7.8% |
| Class C | -2.2% | 3.0% | 3.1% | -3.0% | 4.5% | 5.2% |
| Class D | -1.6% | 3.5% | 3.6% | -1.7% | 6.7% | 7.2% |
| Class E | -4.6% | 9.6% | 9.3% |  |  |  |
| Overall | -2.9% | 4.5% | 4.7% | -3.8% | 7.2% | 7.6% |
|  | -2.8% | 4.4% | 4.7% | -3.7% | 7.0% | 7.5% |
| Enc Time[%] | 103% |  |  | 102% |  |  |
| Dec Time[%] | 103% |  |  | 102% |  |  |
|  | Low delay B LC |  |  | Low delay P LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -4.5% | 9.1% | 13.3% | -4.1% | 10.6% | 14.6% |
| Class C | -2.9% | 6.9% | 8.0% | -2.9% | 7.4% | 8.7% |
| Class D | -1.8% | 11.7% | 11.9% | -1.9% | 12.2% | 11.5% |
| Class E | -3.2% | 10.3% | 11.5% | -3.4% | 11.3% | 13.2% |
| Overall | -3.2% | 9.4% | 11.3% | -3.1% | 10.3% | 12.1% |
|  | -3.1% | 9.3% | 11.1% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 102% |  |  | 103% |  |  |
| Dec Time[%] | 101% |  |  | 100% |  |  |

Table 3. BD-Rate[%] and relative encoding/decoding time[%] (Test10)

|  | All Intra LC |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -3.2% | 4.4% | 4.8% | -5.0% | 10.7% | 9.9% |
| Class B | -3.1% | 4.8% | 5.8% | -4.1% | 8.4% | 10.1% |
| Class C | -2.3% | 4.1% | 4.3% | -2.7% | 5.9% | 6.7% |
| Class D | -1.5% | 4.1% | 4.1% | -1.5% | 8.5% | 8.8% |
| Class E | -4.5% | 10.4% | 10.4% |  |  |  |
| Overall | -2.8% | 5.3% | 5.7% | -3.4% | 8.4% | 9.0% |
|  | -2.8% | 5.2% | 5.6% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 102% |  |  | 101% |  |  |
| Dec Time[%] | 104% |  |  | 101% |  |  |
|  | Low delay B LC |  |  | Low delay P LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -4.0% | 12.2% | 17.9% | -4.0% | 13.3% | 19.2% |
| Class C | -2.6% | 8.9% | 10.4% | -2.6% | 9.2% | 11.0% |
| Class D | -1.9% | 16.9% | 16.7% | -1.7% | 16.3% | 16.9% |
| Class E | -2.8% | 14.6% | 15.9% | -2.8% | 15.4% | 16.5% |
| Overall | -2.9% | 13.0% | 15.4% | -2.9% | 13.4% | 16.1% |
|  | -2.8% | 12.9% | 15.3% | -2.7% | 13.4% | 16.0% |
| Enc Time[%] | 102% |  |  | 103% |  |  |
| Dec Time[%] | 100% |  |  | 99% |  |  |

# References

[1]	F. Bossen, "Common test conditions and software reference configurations ", Joint Collaborative Team on Video Coding, JCTVC-F900, Torino, July 2011.

[2]	CE description, " Description of Core Experiment 1: Entropy Coding," Joint Collaborative Team on Video Coding, JCTVC-F901, Torino, July 2011.

# Patent rights declaration(s)

TOSHIBA Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).