| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G436 |
| --- | --- |

| Title: | CE6.d category 2: Cross-checking reports of intra prediction with secondary boundary (JCTVC-F172 and JCTVC-F456) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Akiyuki Tanizawa Taichiro Shiodera 1, Komukai-Toshiba-cho, Saiwai-ku, Kawasaki, 212-8582, JAPAN | Tel: Email: | +81-44-549-2152 akiyuki.tanizawa@toshiba.co.jp |
| Source: | TOSHIBA Corporation |  |  |

_____________________________

# Abstract

This contribution reports cross-checking results of “intra prediction with secondary boundary” in subtest d of category 2 (Filtering for other angular predictions) on Core Experiment 6 of Intra Coding Improvements. Tools of bi-direction intra angular prediction (JCTVC-F122) and directional intra prediction smoothing (JCTVC-F456) are tested on HM common coding conditions. The verification task has been done successfully and the results perfectly match those proposed in JCTVC-F172 and JCTVC-F456.

Experimental results show that BD-bitrate of JCTVC-F122 is -0.3% and -0.3% for IO-HE and IO-LC, respectively. The negative value means gain. Average decoding times are 101% and 101% for IO-HE and IO-LC, respectively. BD-bitrate of JCTVC-F456 is -0.3% and -0.3% for IO-HE and IO-LC, respectively.  Average decoding times are 101% and 100% for IO-HE and IO-LC, respectively.

# Cross-checking

The software based on HM version 4.0 and the experimental results were provided on Sep. 13 2011 by MediaTek. Both schemes were implemented in this software.

The bi-direction intra angular prediction of JCTVC-F122 adds some macros as follows;

#define CE6_D_F122 1 // (MediaTek) Smoothing for vertical, horizontal and diagonal modes (intraPredAngle>= 0)

#define F122_VH 0 // Smoothing for vertical and horizontal modes (intraPredAngle == 0)

#define F122_VHD 0 // Smoothing for modes ( 0 < intraPredAngle <= 9)

#define F122_DGLD 1 // Smoothing for modes ( 9 < intraPredAngle < 32)

#define F122_DGL 1 // Smoothing for modes ( intraPredAngle == 32)

#define MULTI_LINE_DGL 1 // 1: smoothing applied to four lines adjacent to the second boundary; 0: smoothing applied to one line adjacent to the boundary

The directional intra prediction smoothing of JCTVC-F456 add one macro as follows;

#define CE6_D_F456 1 // (Nokia)      Diagonal smoothing for modes with abs(intraPredAngle) > 9

The software was complied fine and the cross-checking was performed on 64bit environments. The simulation has been conducted under CE common test conditions [1], [2] for both high efficiency and low complexity conditions in the case of I-slice only coding structures. We reviewed the source code and confirmed that the schemes implemented in the software matches those described in JCTVC-F122 and JCTVC-F456, respectively. These methods are also consistent with CE description [2].

Table 1 shows the results of bi-directional intra angular prediction and Table 2 shows the results of directional intra prediction smoothing. Through the both results, it turns out that the gain is promising and both schemes do not affect encoding time and decoding time.

Table 1. BD-Rate[%] and relative encoding/decoding time[%] (JCTVC-F122)

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.4% | -0.3% | -0.3% | -0.3% | -0.4% | -0.4% |
| Class B | -0.3% | -0.2% | -0.1% | -0.3% | -0.3% | -0.3% |
| Class C | -0.3% | -0.2% | -0.2% | -0.3% | -0.3% | -0.3% |
| Class D | -0.3% | -0.2% | -0.2% | -0.3% | -0.3% | -0.3% |
| Class E | -0.3% | -0.3% | -0.4% | -0.3% | -0.4% | -0.3% |
| Overall | -0.3% | -0.3% | -0.2% | -0.3% | -0.3% | -0.3% |
|  | -0.3% | -0.3% | -0.3% | -0.3% | -0.3% | -0.3% |
| Enc Time[%] | 101% |  |  | 101% |  |  |
| Dec Time[%] | 101% |  |  | 101% |  |  |

Table 2. BD-Rate[%] and relative encoding/decoding time[%] (JCTVC-F456)

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.3% | -0.3% | -0.3% | -0.3% | -0.3% | -0.4% |
| Class B | -0.2% | -0.2% | -0.1% | -0.2% | -0.2% | -0.2% |
| Class C | -0.2% | -0.2% | -0.2% | -0.2% | -0.3% | -0.2% |
| Class D | -0.3% | -0.2% | -0.3% | -0.3% | -0.3% | -0.3% |
| Class E | -0.3% | -0.3% | -0.3% | -0.2% | -0.3% | -0.3% |
| Overall | -0.3% | -0.2% | -0.2% | -0.2% | -0.3% | -0.3% |
|  | -0.3% | -0.2% | -0.2% | -0.2% | -0.3% | -0.3% |
| Enc Time[%] | 101% |  |  | 101% |  |  |
| Dec Time[%] | 101% |  |  | 100% |  |  |

# References

[1]	F. Bossen, "Common test conditions and software reference configurations ", Joint Collaborative Team on Video Coding, JCTVC-F900, Torino, July 2011.

[2]	CE description, "Core Experiment 6: Intra Coding Improvements," Joint Collaborative Team on Video Coding, JCTVC-F906, Torino, July 2011.

# Patent rights declaration(s)

TOSHIBA Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).