| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G437 |
| --- | --- |

| Title: | CE9 subtest 2.6: Cross-checking reports of MRG_ID04 (ETRI/KHU’s proposal) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Akiyuki Tanizawa Taichiro Shiodera 1, Komukai-Toshiba-cho, Saiwai-ku, Kawasaki, 212-8582, JAPAN | Tel: Email: | +81-44-549-2152 akiyuki.tanizawa@toshiba.co.jp |
| Source: | TOSHIBA Corporation |  |  |

_____________________________

# Abstract

This contribution reports cross-checking results of Merge/Skip operations for identical motion in subtest 2.6 on Core Experiment 9 of MV Coding and Skip/Merge Operations. Two variations of MRG_ID04 were tested on HM common coding conditions. The verification task has been done successfully and the results match those provided by proponent.

Experimental results show that BD-bitrate of variation A in MRG_ID04 is 0.0%, 0.0%, 0.2% and 0.2% for RA-HE, RA-LC, LB-HE and LB-LC, respectively. The negative value means gain. Average decoding times are 100%, 100%, 98% and 98% for RA-HE, RA-LC, LB-HE and LB-LC, respectively. BD-bitrate of variation B in MRG_ID04 is 0.0%, 0.0%, 0.1% and 0.2% for RA-HE, RA-LC, LB-HE and LB-LC, respectively. Average decoding times are 99%, 100%, 98% and 98% for RA-HE, RA-LC, LB-HE and LB-LC, respectively.

# Cross-checking

There are four kinds of configurations of approaches, when the collocated motion information of a bi-predicted Merge/SKIP PU is identical (i.e., mvL0==mvL1 and refPOCL0==refPOCL1 for the collocated merge candidate) in CE9. This contribution shows the results of MRG_ID04 case.

Table 1. Combinations of merge/skip operations for identical motion in CE9

| Test | Description | Related Doc. | Variation | Tester1 | Tester2 |
| --- | --- | --- | --- | --- | --- |
| MRG_ID01 | MV from different collocated picture | JCTVC-F325 | A: When refPOCs are identical | Toshiba | ETRI/KHU |
|  |  |  | B: When both MVs and refPOCs are identical | Toshiba | ETRI/KHU |
| MRG_ID02 | Non-zero MV from spatial neighbor | JCTVC-F712 | A: Keep the original refIdxL1Col | ETRI/KHU | Samsung |
|  |  |  | B: Derive refIdxL1Col from the one related to the found MV | ETRI/KHU | Samsung |
| MRG_ID03 | Different MV from spatial neighbor | JCTVC-F712 | A: Keep the original refIdxL1Col | ETRI/KHU | LGE |
|  |  |  | B: Derive refIdxL1Col from the one related to the found MV | ETRI/KHU | LGE |
| MRG_ID04 | Combination of MRG_ID02 and MRG_ID03 (Non-zero and different MV from spatial neighbor) |  | A: Keep the original refIdxL1Col | ETRI/KHU | Toshiba |
|  |  |  | B: Derive refIdxL1Col from the one related to the found MV | ETRI/KHU | Toshiba |

The software based on HM version 4.0 (bug-fix version) and the experimental results were provided on . Sep. 16 2011 by ETRI/KHU. The software was complied fine and the cross-checking was performed on 64bit environments. The simulation has been conducted under CE common test conditions [1], [2] for both high efficiency and low complexity conditions in the case of random access and low delay B coding structures. We reviewed the source code and confirmed that the schemes implemented in the software matches those described in JCTVC-F712 and CE description [2].

Proponent adds two macros as follows;

#define CE9_MRG_ID04_A                    0           ///< CE9: Test MEG_ID04 Variation A

#define CE9_MRG_ID04_B                    1           ///< CE9: Test MEG_ID04 Variation B

The first macro is to keep the original refIdxL1Col for identical motion. The second macro is to derive refIdxL1Col from the one related to the found MV for identical motion.

Table 1 shows that BD-rate and relative encoding/decoding time of variation A in MRG_ID04 and Table 2 shows BD-rate and relative encoding/decoding time of variation B in MRG_ID04. Through the results, it turns out that the gain is a little worse than the results of JCTVC-F712.

Table 2. BD-Rate[%] and relative encoding/decoding time[%] (variation A)

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% |
| Class C | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% |
| Class E |  |  |  |  |  |  |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 105% |  |  | 105% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.1% | -0.2% | 0.0% | 0.1% | 0.0% | -0.1% |
| Class C | 0.1% | 0.0% | 0.2% | 0.1% | 0.2% | -0.1% |
| Class D | 0.2% | -0.1% | 0.5% | 0.2% | 0.5% | 0.1% |
| Class E | 0.3% | 0.7% | 1.2% | 0.3% | 0.8% | 0.4% |
| Overall | 0.2% | 0.0% | 0.4% | 0.1% | 0.3% | 0.0% |
|  | 0.2% | 0.0% | 0.3% | 0.1% | 0.3% | 0.0% |
| Enc Time[%] | 106% |  |  | 106% |  |  |
| Dec Time[%] | 98% |  |  | 98% |  |  |

Table 3. BD-Rate[%] and relative encoding/decoding time[%] (variation B)

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | -0.1% | -0.1% | 0.0% | 0.1% | 0.1% |
| Class B | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | -0.1% |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class E |  |  |  |  |  |  |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 104% |  |  | 104% |  |  |
| Dec Time[%] | 99% |  |  | 100% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.1% | -0.2% | -0.1% | 0.0% | 0.0% | -0.2% |
| Class C | 0.1% | 0.2% | 0.1% | 0.1% | 0.1% | -0.2% |
| Class D | 0.1% | 0.2% | 0.2% | 0.1% | 0.3% | 0.0% |
| Class E | 0.4% | 0.8% | 0.9% | 0.2% | 0.5% | 0.0% |
| Overall | 0.2% | 0.2% | 0.2% | 0.1% | 0.2% | -0.1% |
|  | 0.2% | 0.2% | 0.2% | 0.1% | 0.1% | -0.1% |
| Enc Time[%] | 105% |  |  | 105% |  |  |
| Dec Time[%] | 98% |  |  | 98% |  |  |

# References

[1]	F. Bossen ," Common test conditions and software reference configurations ", Joint Collaborative Team on Video Coding, JCTVC-F900, Torino, July 2011.

[2]	CE description, " Core Experiment 9: MV Coding and Skip/Merge Operations," Joint Collaborative Team on Video Coding, JCTVC-F909, Torino, July 2011.

# Patent rights declaration(s)

TOSHIBA Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).