| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G438 |
| --- | --- |

| Title: | On complexity reduction of bi-prediction for identical motion |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Akiyuki Tanizawa Taichiro Shiodera Takeshi Chujoh Tomoo Yamakage 1, Komukai-Toshiba-cho, Saiwai-ku, Kawasaki, 212-8582, JAPAN | Tel: Email: | +81-44-549-2152 akiyuki.tanizawa@toshiba.co.jp taichiro.shiodera@toshiba.co.jp |
| Source: | TOSHIBA Corporation |  |  |

_____________________________

## Encoder description

## Change bi-prediction to uni-prediction for identical motion

If reference blocks indicated by motion information of bi-prediction for each list are identical, which means that RefPicOrderCnt( currPic, refIdxL0, L0 ) is equal to RefPicOrderCnt( currPic, refIdxL1, L1 ) and mvL0 is equal to mvL1, the inter prediction mode of current PU is changed to uni-prediction from bi-prediction.

The final prediction utilization flags predFlagL0 and predFlagL1 are derived as specified by the following ordered steps using the inputted motion vectors mvL0 and mvL1, the inputted reference indices refIdxL0 and refIdxL1, and the inputted prediction utilization flags predFlagL0 and predFlagL1;

when all of the following conditions are true,

    - predFlagL0 = = 1

    - predFlagL1 = = 1

    - RefPicOrderCnt( refIdxL0, L0 ) == RefPicOrderCnt( refIdxL1, L1 ) | | mvL0 == mvL1

the following applies.

    - predFlagL0 = = 1

    - predFlagL1 = = 0

If both predFlagL0 and predFlagL1 are equal to 1, the prediction sample of bi-prediction is derived as follows;

predSamples[ x, y ] = Clip3( 0, ( 1 << bitDepth ) – 1 ,

predSamplesL0[ x, y ] + predSamplesL1[ x, y ] + offset2 ) >> shift2 ).

Otherwise (either predFlagL0 is equal to 1or predFlagL1 is equal to 1), the prediction sample of uni-prediction is derived as follows;

predSamples[ x, y ] = ( predSamplesLX[ x, y ] + offset1 ) >> shift1,

where X is replaced by 0 for predFlagL0 is equal to 1 and by 1 for predFlagL1 is equal to 1.