| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G438 |
| --- | --- |

| Title: | On complexity reduction of bi-prediction for identical motion |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Akiyuki Tanizawa Taichiro Shiodera Takeshi Chujoh Tomoo Yamakage 1, Komukai-Toshiba-cho, Saiwai-ku, Kawasaki, 212-8582, JAPAN | Tel: Email: | +81-44-549-2152 akiyuki.tanizawa@toshiba.co.jp taichiro.shiodera@toshiba.co.jp |
| Source: | TOSHIBA Corporation |  |  |

_____________________________

# Abstract

This contribution reports results of complexity reduction of bi-prediction for identical motion described in JCTVC-F325. At the last JCT-VC Torino meeting, the implementation technique of complexity reduction by changing bi-prediction to uni-prediction in case of identical motion was proposed. This contribution provides a suitable source code and experimental results. Experimental results show that average decoding times are 99%, 99%, 95% and 95% for RA-HE, RA-LC, LB-HE and LB-LC, respectively. This scheme does not affect bitstreams, and then BD-bitrate is identical to the HM anchor.

# Introduction

At the last JCT-VC Torino meeting, the implementation technique of complexity reduction by changing bi-prediction to uni-prediction in case of identical motion was proposed in JCTVC-F325. Through the discussion, the decision to adopt this scheme to HM software was delegated to the software coordinators and its AHG. The source code including this scheme in HM3.3 and HM4.0 were provided to software coordinators. In this document, the experimental results of this scheme are reported.

# Algorithm description

## Change to uni-prediction from bi-prediction for identical motion of Merge/Skip mode

If reference blocks indicated that motion information for each list are same, the inter prediction direction mode of current PU is changed to uni-prediction from bi-prediction. The modification of source code in HM4.0 is shown in appendix.

# Experimental Results

HM software version 4.0 (r1358) with the common configuration Error: Reference source not found is used in these experiments. Table 1 and Table 2 show BD-Rate and relative average encoding/decoding time compared with HM4.0 anchor. Class F sequences are excluded in Table 1. Experimental results show that BD-bitrate is not changed. Average decoding times are 99%, 99%, 95% and 95% for RA-HE, RA-LC, LB-HE and LB-LC, respectively.

Table 1. BD-Rate[%] and relative encoding/decoding time[%] (without Class F)

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class E |  |  |  |  |  |  |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class E | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 99% |  |  | 99% |  |  |
| Dec Time[%] | 95% |  |  | 95% |  |  |

Table 2. BD-Rate[%] and relative encoding/decoding time[%] (with Class F)

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class E |  |  |  |  |  |  |
| Class F | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class E | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class F | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 99% |  |  | 99% |  |  |
| Dec Time[%] | 94% |  |  | 93% |  |  |

# Conclusion

In this contribution, the experimental results of complexity reduction of bi-prediction for identical motion described in JCTVC-F325 are reported and the modified source code including this scheme in HM4.0 is provided. This scheme can reduce the decoding time of 1%, 1%, 5% and 5% for RA-HE, RA-LC, LB-HE and LB-LC, respectively. The bitstream does not change by introducing this scheme.

It’s suggested that this scheme is adopted in the next version of HM software as an implementation technique for the encoder and the decoder.

# References

[1]	F. Bossen ," Common test conditions and software reference configurations ", Joint Collaborative Team on Video Coding, JCTVC-F900, Torino, July 2011.

# Patent rights declaration(s)

TOSHIBA Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Appendix

## HM4.0 source code modification

The modification of HM4.0 source code is just 30 lines as follow;

Void TComPrediction::motionCompensation ( TComDataCU* pcCU, TComYuv* pcYuvPred, RefPicList eRefPicList, Int iPartIdx )

{

Int         iWidth;

Int         iHeight;

UInt        uiPartAddr;

if ( iPartIdx >= 0 )

{

pcCU->getPartIndexAndSize( iPartIdx, uiPartAddr, iWidth, iHeight );

if ( eRefPicList != REF_PIC_LIST_X )

{

xPredInterUni (pcCU, uiPartAddr, iWidth, iHeight, eRefPicList, pcYuvPred, iPartIdx );

}

else

{

#if UNI_FOR_BI_IDENTICAL_MOTION

if ( xCheckIdenticalMotion( pcCU, uiPartAddr ) )

{

xPredInterUni (pcCU, uiPartAddr, iWidth, iHeight, REF_PIC_LIST_0, pcYuvPred, iPartIdx );

}

else

{

#endif

xPredInterBi  (pcCU, uiPartAddr, iWidth, iHeight, pcYuvPred, iPartIdx );

#if UNI_FOR_BI_IDENTICAL_MOTION

}

#endif

}

return;

}

for ( iPartIdx = 0; iPartIdx < pcCU->getNumPartInter(); iPartIdx++ )

{

pcCU->getPartIndexAndSize( iPartIdx, uiPartAddr, iWidth, iHeight );

if ( eRefPicList != REF_PIC_LIST_X )

{

xPredInterUni (pcCU, uiPartAddr, iWidth, iHeight, eRefPicList, pcYuvPred, iPartIdx );

}

else

{

#if UNI_FOR_BI_IDENTICAL_MOTION

if ( xCheckIdenticalMotion( pcCU, uiPartAddr ) )

{

xPredInterUni (pcCU, uiPartAddr, iWidth, iHeight, REF_PIC_LIST_0, pcYuvPred, iPartIdx );

}

else

{

#endif

xPredInterBi  (pcCU, uiPartAddr, iWidth, iHeight, pcYuvPred, iPartIdx );

#if UNI_FOR_BI_IDENTICAL_MOTION

}

#endif

}

}

return;

}

#if UNI_FOR_BI_IDENTICAL_MOTION

Bool TComPrediction::xCheckIdenticalMotion ( TComDataCU* pcCU, UInt uiPartAddr )

{

if( pcCU->getSlice()->isInterB() )

{

if( pcCU->getCUMvField(REF_PIC_LIST_0)->getRefIdx(uiPartAddr) >= 0 && pcCU->getCUMvField(REF_PIC_LIST_1)->getRefIdx(uiPartAddr) >= 0)

{

Int iRefPOCL0 = pcCU->getSlice()->getRefPic(REF_PIC_LIST_0, pcCU->getCUMvField(REF_PIC_LIST_0)->getRefIdx(uiPartAddr))->getPOC();

Int iRefPOCL1 = pcCU->getSlice()->getRefPic(REF_PIC_LIST_1, pcCU->getCUMvField(REF_PIC_LIST_1)->getRefIdx(uiPartAddr))->getPOC();

if(iRefPOCL0 == iRefPOCL1 && pcCU->getCUMvField(REF_PIC_LIST_0)->getMv(uiPartAddr) == pcCU->getCUMvField(REF_PIC_LIST_1)->getMv(uiPartAddr))

{

return true;

}

}

}

return false;

}

#endif