| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G439 |
| --- | --- |

| Title: | CE9 subtest 2.6: Results of MRG_ID01 in Merge/Skip operations for identical motion |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Akiyuki Tanizawa Taichiro Shiodera Takeshi Chujoh Tomoo Yamakage 1, Komukai-Toshiba-cho, Saiwai-ku, Kawasaki, 212-8582, JAPAN | Tel: Email: | +81-44-549-2152 akiyuki.tanizawa@toshiba.co.jp taichiro.shiodera@toshiba.co.jp |
| Source: | TOSHIBA Corporation |  |  |

_____________________________

# Abstract

This contribution reports results of MRG_ID01 described in JCTVC-F325 in CE9 subtest 2.6 (Merge/Skip operations for identical motion). MRG_ID01 modified derivation process of temporal merge/skip mode in B-slice and has two kinds of variations. The variation A means that the collocated picture for L1 motion vector is changed from RefPicList1[0] to RefPicList1[1], if the two reference frames for the collocated MV candidates are identical. The variation B means that the same derivation process as the above is performed if both reference frames and MVs of the collocated merge candidates are identical.

Experimental results show that BD-bitrate of the variation A is 0.0%, 0.0%, 0.3% and 0.3% for RA-HE, RA-LC, LB-HE and LB-LC, respectively. The positive value means loss. Average decoding times are 100%, 100%, 99% and 99% for RA-HE, RA-LC, LB-HE and LB-LC, respectively. BD-bitrate of the variation B is 0.0%, 0.0%, 0.1% and 0.1% for RA-HE, RA-LC, LB-HE and LB-LC, respectively. Average decoding times are 99%, 100%, 99% and 100% for RA-HE, RA-LC, LB-HE and LB-LC, respectively.

# Algorithm description

## Modification of MV derivation for list 1 of bi-prediction

If RefPicListX[ 1 ] is available and POC number of reference frame corresponding to refIdx for each list are same, the co-located picture (colPic) position that contains the co-located partition (colPU) for only list 1 is changed. This means that colPic for list 1 is set to RefPicListX[ 1 ] instead of RefPicListX[ 0 ].

In the variation A, only POC information of reference frames (refPOC) is used for decision on whether colPic for list 1 is set to RefPicListX[ 1 ] or RefPicListX[ 0 ]. In the variation B, refPOC and MV information are used for decision. The detailed description is shown in JCTVC-F325.

# Syntax and decoding process

The syntax and semantics do not change in this proposal. The detailed decoding process for MRG_ID01 are shown in appendix of this contribution.

# Experimental Results

HM software version 4.0 (r1358) with the common configuration Error: Reference source not found[2] is used in these experiments. Table 1 and 2 shows BD-Rate and relative average encoding/decoding time compared with HM4.0 anchor for variation A and variation B, respectively. Class F sequences are excluded in these results. The negative value of BD-Rate indicates gain. Experimental results show that BD-bitrate of the variation A is 0.0%, 0.0%, 0.3% and 0.3% for RA-HE, RA-LC, LB-HE and LB-LC, respectively. Average decoding times are 100%, 100%, 99% and 99% for RA-HE, RA-LC, LB-HE and LB-LC, respectively. BD-bitrate of the variation B is 0.0%, 0.0%, 0.1% and 0.1% for RA-HE, RA-LC, LB-HE and LB-LC, respectively. Average decoding times are 99%, 100%, 99% and 100% for RA-HE, RA-LC, LB-HE and LB-LC, respectively. Through the tests, it’s thought that both schemes don’t get gain compared with current HM anchor, because the temporal distance of the reference frame is far from one of the previous HM version.

Table 1. BD-Rate[%] and relative encoding/decoding time[%] (MRG_ID01: Variation A)

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | -0.2% | -0.1% | 0.0% | 0.1% | 0.0% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class D | 0.1% | -0.1% | 0.1% | 0.1% | 0.0% | 0.1% |
| Class E |  |  |  |  |  |  |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.3% | 0.1% | 0.3% | 0.2% | 0.2% | 0.2% |
| Class C | 0.5% | 0.4% | 0.5% | 0.5% | 0.6% | 0.4% |
| Class D | 0.2% | 0.5% | 0.2% | 0.3% | 0.1% | 0.2% |
| Class E | 0.3% | 0.2% | 0.9% | 0.3% | 0.6% | 0.1% |
| Overall | 0.3% | 0.2% | 0.4% | 0.3% | 0.3% | 0.2% |
|  | 0.3% | 0.1% | 0.4% | 0.3% | 0.2% | 0.2% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  |

Table 2. BD-Rate[%] and relative encoding/decoding time[%] (MRG_ID01: Variation B)

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.0% | -0.1% | 0.0% | 0.1% | 0.1% |
| Class B | 0.0% | 0.0% | 0.1% | 0.0% | -0.1% | 0.1% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | -0.1% | 0.1% | 0.1% | 0.0% | 0.0% |
| Class E |  |  |  |  |  |  |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 99% |  |  | 100% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.1% | -0.1% | 0.2% | 0.0% | 0.1% | 0.2% |
| Class C | 0.2% | 0.0% | 0.1% | 0.2% | 0.3% | 0.2% |
| Class D | 0.2% | 0.3% | 0.3% | 0.1% | 0.0% | 0.0% |
| Class E | 0.2% | 0.5% | 0.5% | 0.1% | 0.0% | -0.1% |
| Overall | 0.1% | 0.1% | 0.2% | 0.1% | 0.0% | 0.0% |
|  | 0.1% | 0.0% | 0.2% | 0.1% | 0.0% | -0.1% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 99% |  |  | 100% |  |  |

# Conclusion

In this contribution, the experimental results of MRG_ID01 described in JCTVC-F325 in CE9 subtest 2.6 (Merge/Skip operations for identical motion) are reported. Through the tests, it turns out that both schemes cannot get gain compared with current HM anchor, because the distance of the reference frame is far from one of the previous HM version.

# References

[1]	F. Bossen ," Common test conditions and software reference configurations ", Joint Collaborative Team on Video Coding, JCTVC-F900, Torino, July 2011.

[2]	CE description, " Core Experiment 9: MV Coding and Skip/Merge Operations," Joint Collaborative Team on Video Coding, JCTVC-F909, Torino, July 2011.

# Patent rights declaration(s)

TOSHIBA Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Appendix

## WD text for MRG_ID01

8.4.2.1.1	Derivation process for luma motion vectors for merge mode

This process is only invoked when PredMode is equal to MODE_ INTER and merge_flag [ xP ][ yP ] is equal to 1, where ( xP, yP ) specify the top-left luma sample of the current prediction unit relative to the top-left luma sample of the current picture.

Inputs of this process are

- a luma location ( xP, yP ) of the top-left luma sample of the current prediction unit relative to the top-left luma sample of the current picture,

- variables specifying the width and the height of the prediction unit for luma, nPSW and nPSH,

- a variable PartIdx specifying the index of the current prediction unit within the current coding unit.

Outputs of this process are

–	the luma motion vectors mvL0 and mvL1,

–	the reference indices refIdxL0 and refIdxL1,

–	the prediction list utilization flags predFlagL0 and predFlagL1.

The motion vectors mvL0 and mvL1, the reference indices refIdxL0 and refIdxL1, and the prediction utilization flags predFlagL0 and predFlagL1 are derived as specified by the following ordered steps:

- The derivation process for merging candidates from neighboring prediction unit partitions in subclause Error: Reference source not found is invoked with luma location ( xP, yP ), the width and the height of the prediction unit nPSW and nPSH and the partition index PartIdx as inputs and the output is assigned to the availability flags availableFlagN, the motion vectors mvL0N and mvL1N, the reference indices refIdxL0N and refIdxL1N and the prediction list utilization flags predFlagL0N and predFlagL1N with N being replaced by A, B, C or D.

- The derivation process of reference indices for temporal merging candidate in subclause Error: Reference source not found is invoked with luma location ( xP, yP ), nPSW, nPSH as the inputs and the output is directly assigned to refIdxLX.

- The derivation process for temporal luma motion vector prediction for merge mode in subclause Error: Reference source not found is invoked with luma location ( xP, yP ), refIdxLX as the inputs and with the output being the availability flag availableFlagLXCol and the temporal motion vector mvLXCol. The variables availableFlagCol and predFlagLXCol (with X being 0 or 1, respectively) are derived as specified below.

availableFlagCol = availableFlagL0Col || availableFlagL1Col	(8-xx)

predFlagLXCol = availableFlagLXCol	(8-xx)

- The merging candidate list, mergeCandList, is constructedas folloes.

(other text is same as 8.4.2.1.1 inWD3)

8.4.2.1.x	Derivation process for temporal luma motion vector prediction for merge mode

Inputs to this process are

- a luma location ( xP, yP ) specifying the top-left luma sample of the current prediction unit relative to the top-left sample of the current picture,

- variables specifying the width and the height of the prediction unit for luma, nPSW and nPSH,

- the reference index of the current prediction unit partition refIdxLX (with X being 0 or 1).

Outputs of this process are

- the motion vector prediction mvLXCol,

- the availability flag availableFlagLXCol.

The function RefPicOrderCnt( pic, refidx, LX ) is specified by the value of PicOrderCnt of the picture that is the reference picture RefPicListX[ refidx ] of pic with X being 0 or 1. PicOrderCnt of the reference picture shall be maintained until the picture is marked as “non-exisiting.”

Depending on the values of slice_type and collocated_from_l0_flag, the variable colPic, specifying the picture that contains the co-located partition, is derived as follows.

If slice_type is equal to B, the following applies.

  - If collocated_from_l0_flag is equal to 0, the following applies.

- If LX is equal to L1, RefPicList1[ 1 ] is available and RefPicOrderCnt( currPic , refIdxL0 , L0 ) is equal to RefPicOrderCnt( currPic, refIdxL1, L1 ), the variable colPic specifies the picture that contains the co-located partition as specified by RefPicList1[ 1 ].

- Otherwise (LX is equal to L0, RefPicList1[ 1 ] is not available or RefPicOrderCnt ( currPic , refIdxL0 , L0 ) is not equal to RefPicOrderCnt( currPic, refIdxL1, L1 )), the variable colPic specifies the picture that contains the co-located partition as specified by RefPicList1[ 0 ].

  - Otherwise (collocated_from_l0_flag is equal to 1), the following applies.

- If LX is equal to L1, RefPicList0[ 1 ] is available and RefPicOrderCnt( currPic , refIdxL0 , L0 ) is equal to RefPicOrderCnt( currPic, refIdxL1, L1 ), the variable colPic specifies the picture that contains the co-located partition as specified by RefPicList0[ 1 ].

- Otherwise (LX is equal to L0, RefPicList0[ 1 ] is not available or RefPicOrderCnt ( currPic , refIdxL0 , L0 ) is not equal to RefPicOrderCnt( currPic, refIdxL1, L1 )), the variable colPic specifies the picture that contains the co-located partition as specified by RefPicList0[ 0 ].

- Otherwise (slice_type is equal to P), the variable colPic specifies the picture that contains the co-located partition as specified by RefPicList0[ 0 ].

Variable colPu and its position ( xPCol, yPCol ) are derived in the following ordered steps:

- Right-bottom luma position of the current prediction unit is defined by

xPRb = xP + nPSW	(8-xx)

yPRb = yP + nPSH	(8-xx)

- The variable colPu is set as the prediction unit covering the modified position given by ( ( xPRb >> 4 ) << 4, ( yPRb >> 4 ) << 4 ) inside the colPic.

- If colPu is coded in an intra prediction mode or colPu is unavailable, the following applies.

  - Central luma position of the current prediction unit is defined by

xPCtr = ( xP + ( nPSW >> 1 ) – 1	(8-xx)

yPCtr = ( yP + ( nPSH >> 1 ) – 1	(8-xx)

  - The variable colPu is set as the prediction unit covering the modified position given by ( ( xPCtr >> 4 ) << 4, ( yPCtr >> 4 ) << 4 ) inside the colPic.

- ( xPCol, yPCol ) is set equal to the top-left luma sample of the colPu relative to the top-left luma sample of the colPic.

The variables mvLXCol and availableFlagLXCol are derived as follows.

- If colPu is coded in an intra prediction mode or colPu is unavailable, both components of mvLXCol are set equal to 0 and availableFlagLXCol is set equal to 0.

- Otherwise (colPu is not coded in an intra prediction mode and colPu is available), the variables mvCol and refIdxCol are derived as follows,

  - If PredFlagL0[ xPCol ][ yPCol ] is equal to 0, the motion vector mvCol and the reference index refIdxCol are set equal to MvL1[ xPCol ][ yPCol ] and RefIdxL1[ xPCol ][ yPCol ], respectively.

  - Otherwise (PredFlagL0[ xPCol ][ yPCol ] is equal to 1), the following applies.

  - If PredFlagL1[ xPCol ][ yPCol ] is equal to 0, the motion vector mvCol and the reference index refIdxCol are set equal to MvL0[ xPCol ][ yPCol ] and RefIdxL0[ xPCol ][ yPCol ], respectively.

  - Otherwise (PredFlagL1[ xPCol ][ yPCol ] is equal to 1), the following assignments are made.

  - If all PicOrderCnts of pictures in all reference picture lists are smaller than PicOrderCnt( currPic ), the motion vector mvCol, the reference index refIdxCol and ListCol are set equal to MvLN[ xPCol ][ yPCol ], RefIdxColLN and LN, respectively, with N being value of RefIdxLX.

  - Otherwise (one of PicOrderCnt of pictures in all reference picture lists are larger than PicOrderCnt( currPic ) ), the motion vector mvCol, the reference index refIdxCol and ListCol are set equal to MvLN[ xPCol ][ yPCol ], RefIdxColLN and LN, respectively, with N being value of collocated_from_l0_flag.

and the variable availableFlagLXCol is set equal to 1 and the following applies.

  - If PicOrderCnt( colPic ) – RefPicOrderCnt( colPic, refIdxCol, ListCol ) is equal to PicOrderCnt( currPic ) - RefPicOrderCnt( currPic, refIdxLX, LX ),

mvLXCol = mvCol	(8-xx)

  - Otherwise, mvLXCol is derived as scaled version of the motion vector mvCol as specified below

tx = ( 16384 + Abs( td / 2 ) ) / td	(8-xx)

DistScaleFactor = Clip3( -1024, 1023, ( tb * tx + 32 ) >> 6 )	(8-xx)

mvLXCol = ClipMv( ( DistScaleFactor * mvCol + 128 ) >> 8 )	(8-xx)

[Ed. (WJ): software has a clip function for scaled motion vector. Do we need this? AVC does not have it]

where td and tb are derived as

td = Clip3( -128, 127, PicOrderCnt( colPic ) – RefPicOrderCnt( colPic, refIdxCol, ListCol ) )	(8-xx)

tb = Clip3( -128, 127, PicOrderCnt( currPic ) – RefPicOrderCnt( currPic, refIdxLX, LX ) )	(8-xx)