| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G441 |
| --- | --- |

| Title: | Redundancy removal of explicit weighted prediction syntax |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Akiyuki Tanizawa Takeshi Chujoh Tomoo Yamakage 1, Komukai-Toshiba-cho, Saiwai-ku, Kawasaki, 212-8582, JAPAN | Tel: Email: | +81-44-549-2152 akiyuki.tanizawa@toshiba.co.jp |
| Source: | TOSHIBA Corporation |  |  |

_____________________________

# Abstract

This document presents a redundancy removal of explicit weighted prediction syntax. This scheme has two proposals. First one (proposal 1) is to modify redundant pred_weight_table syntax for identical reference frames. Second one (proposal 2) is to introduce the simple predictions to syntax elements in pred_weight_table syntax in order to reduce the redundant representation. The experimental results in HM software version 4.0 (HM-4.0-dev) and version 4.1 with weighted prediction under common test conditions are reported. The results show that BD-rate of proposal 1 is 0.3% and 2.5% for RA-HE and LDB-HE on average, respectively. BD-rate of proposal 2 is 0.4%, 1.0% and 0.6% for RA-HE, LDB-HE and LDP-HE on average, respectively. BD-rate of combination with prop. 1 and prop. 2 is 0.7%, 3.0% and 0.6% for RA-HE, LDB-HE and LDP-HE on average. It is reported that these schemes does not affect the encoding and decoding time. The source code is of these proposals also attached in this contribution.

# Weighted Prediction in HEVC

At the last JCT-VC Torino meeting, AVC based weighted prediction (AVCWP) scheme [1][2] was adopted in HM4 and WD4 [3]. The AVCWP can achieve large coding gain (up to 60%) for fading sequences with illumination variation. However, AVCWP has two issues on syntax representation. The first one is that the same pred_weight_table syntax might be encoded twice, when two reference frames referred for AVCWP in B-slice are identical (list 0 and list 1 are identical). The second one is that the weighting factor and offset are encoded directly. If luma_log2_denom is getting larger, weighting factor is also getting larger. In order to fix these issues, we introduce two modifications to AVC based pred_weight_table syntax, semantics and decoding process.

## Proposal 1: Introducing combined weight pred table syntax

The first one is to introduce combined_pred_weight_table syntax. In current HM4.0, combined list management was adopted [4]. The reference frames in DPB are managed in this scheme. Reference list 0 and reference list 1 are mapped to combined reference list. The combined_pred_weight_table syntax means that reference frames used for pred_weight_table syntax is managed by following in this framework. The proposed combined_pred_weight_table syntax and the related modification of slice header is shown in attached WD text. As slice header syntax shows, this proposal is switched by a value of ref_pic_list_combination_flag. By avoiding referring identical reference frames for WP, coding efficiency can be achieved.

## Proposal 2: Introducing simple prediction to weighting parameters

The second proposal is to introduce simple prediction to weighting parameters. The weighting parameters for AVCWP are encoded directly. If luma_log2_weight_denom is 7 and there is no illumination variation between target frame and reference frames, a value of default weighting factor ((1<<7) = 128) could be encoded for each reference frame. In general, since illumination variation between target frame and reference frames are not so big, the overhead of weighting factor can be reduced by subtracting the target weighting factor to the default weighting factor. In order to reduce such redundancy of weighting parameters, we introduce simple prediction to weighting factors and offsets in these following equations.

- Weighting factor prediction;

delta_luma_weight_common[ i ] = luma_weight_common[ i ] – (1<< luma_log2_weight_denom)

- Chroma offset prediction;

delta_chroma_offset_common[ i ] = (chroma_offset_common[ i ] +

( ( Med * chroma_weight_common[ i ] )>> chroma_log2_weight_denom ) – Med )

Med = (1<<(bit_depth_chroma_minus8+7))

- Denominator prediction;

delta_ chroma_log2_weight_denom = (chroma_log2_weight_denom – luma_log2_weight_denom)

# Syntax and decoding process

The detailed syntax, semantics and decoding process for proposed weighted prediction are shown in WD text of this contribution (see JCTVC-F803_d6_WD_text_WP.doc).

# Experimental Results

HM software version 4.0 (HM-4.0-dev) and version 4.1 with the common configuration Error: Reference source not found is used in these experiments. Test sequences with black/white fade are provided by WP AHG [6]. Table  shows BD-Rate and relative average encoding/decoding time for proposal 1 compared with HM4.0 AVCWP anchor (Explicit WP). The negative value of BD-Rate indicates gain. It observed that proposed 1 can achieve the gain of 0.3% and 2.5% on average for RA-HE and LDB-HE and the gain of 0.3% and 2.2% on average for RA-LC and LDB-LC, respectively.

Table 2 shows BD-Rate and relative average encoding/decoding time for proposal 2 compared with HM4.0 AVCWP anchor. It observed that proposed 1 can achieve the gain of 0.4%, 1.0% and 0.6% on average for RA-HE, LDB-HE and LDP-HE, and the gain of 0.8%, 2.1% and 1.1% on average for RA-LC, LDB-LC and LDP-LC.

Table 3 shows BD-Rate and relative average encoding/decoding time for combination with proposal 1 and 2 compared with HM4.0 AVCWP anchor. It observed that this combination can achieve the gain of 0.7%, 3.0% and 0.6% on average for RA-HE, LDB-HE and LDP-HE, and the gain of 0.9%, 3.3% and 1.1% on average for RA-LC, LDB-LC and LDP-LC.

Table 1. BD-Rate[%] and relative encoding/decoding time[%] of proposal 1 compared with AVCWP for black/white-fade sequences

|  | Black-fade |  |  |  |  |  | White-fade |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | High Efficiency |  |  | Low Complexity |  |  | High Efficiency |  |  | Low Complexity |  |  |
|  | Y | U | V | Y | U | V | Y | U | V | Y | U | V |
| RA | -0.3% | -0.3% | -0.3% | -0.3% | -0.3% | -0.3% | -0.3% | -0.3% | -0.3% | -0.3% | -0.3% | -0.3% |
| Enc Time[%] | 100% |  |  | 100% |  |  | 100% |  |  | 100% |  |  |
| Dec Time[%] | 101% |  |  | 100% |  |  | 100% |  |  | 100% |  |  |
| LDB | -2.3% | -2.1% | -2.1% | -2.1% | -1.8% | -1.9% | -2.6% | -2.4% | -2.4% | -2.4% | -2.1% | -2.1% |
| Enc Time[%] | 100% |  |  | 100% |  |  | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  | 101% |  |  | 100% |  |  |
| LDP |  |  |  |  |  |  |  |  |  |  |  |  |
| Enc Time[%] |  |  |  |  |  |  |  |  |  |  |  |  |
| Dec Time[%] |  |  |  |  |  |  |  |  |  |  |  |  |

Table 2. BD-Rate[%] and relative encoding/decoding time[%] of proposal 2 compared with AVCWP for black/white-fade sequences

|  | Black-fade |  |  |  |  |  | White-fade |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | High Efficiency |  |  | Low Complexity |  |  | High Efficiency |  |  | Low Complexity |  |  |
|  | Y | U | V | Y | U | V | Y | U | V | Y | U | V |
| RA | -0.4% | -0.4% | -0.4% | -0.7% | -0.7% | -0.7% | -0.4% | -0.4% | -0.4% | -0.8% | -0.7% | -0.7% |
| Enc Time[%] | 100% |  |  | 100% |  |  | 100% |  |  | 100% |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  | 100% |  |  | 98% |  |  |
| LDB | -1.0% | -0.9% | -0.9% | -2.0% | -1.8% | -1.8% | -1.0% | -0.9% | -0.9% | -2.1% | -1.8% | -1.9% |
| Enc Time[%] | 100% |  |  | 100% |  |  | 100% |  |  | 100% |  |  |
| Dec Time[%] | 99% |  |  | 98% |  |  | 100% |  |  | 99% |  |  |
| LDP | -0.6% | -0.5% | -0.5% | -1.1% | -0.9% | -1.0% | -0.6% | -0.5% | -0.5% | -1.1% | -1.0% | -1.0% |
| Enc Time[%] | 100% |  |  | 100% |  |  | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 98% |  |  | 100% |  |  | 99% |  |  |

Table 3. BD-Rate[%] and relative encoding/decoding time[%] of combination with prop. 1 and prop. 2 compared with AVCWP for black/white-fade sequences

|  | Black-fade |  |  |  |  |  | White-fade |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | High Efficiency |  |  | Low Complexity |  |  | High Efficiency |  |  | Low Complexity |  |  |
|  | Y | U | V | Y | U | V | Y | U | V | Y | U | V |
| RA | -0.6% | -0.6% | -0.6% | -0.9% | -0.8% | -0.8% | -0.7% | -0.6% | -0.6% | -1.0% | -0.9% | -0.9% |
| Enc Time[%] | 100% |  |  | 100% |  |  | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  | 100% |  |  | 100% |  |  |
| LDB | -2.9% | -2.6% | -2.6% | -3.1% | -2.8% | -2.8% | -3.2% | -2.9% | -2.9% | -3.5% | -3.0% | -3.1% |
| Enc Time[%] | 100% |  |  | 100% |  |  | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  | 100% |  |  | 100% |  |  |
| LDP | -0.6% | -0.5% | -0.5% | -1.1% | -0.9% | -1.0% | -0.6% | -0.5% | -0.5% | -1.1% | -1.0% | -1.0% |
| Enc Time[%] | 100% |  |  | 100% |  |  | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  | 100% |  |  | 101% |  |  |

# Conclusion

In this contribution, redundancy removal schemes of explicit weighted prediction syntax are proposed. Experimental results show that the combination with proposal 1 and proposal 2 can achieve the average BD-rate gain of 0.7%, 3.0% and 0.6% for RA-HE, LDB-HE and LDP-HE, and 0.9%, 3.3% and 1.1% on average for RA-LC, LDB-LC and LDP-LC. These schemes do not affect the encoding and decoding time.

It’s suggested that these two proposals are introduced in the HEVC test model (HM).

# References

[1]	P. Bordes, "Weighted Prediction," Joint Collaborative Team on Video Coding, JCTVC-F265, Torino, July 2011.

[2]	A. Tanizawa, " Explicit Weighted Prediction with simple WP parameter estimation," Joint Collaborative Team on Video Coding, JCTVC-F326, Torino, July 2011.

[3]	AHG report, " WD4: Working Draft 4 of High-Efficiency Video Coding," Joint Collaborative Team on Video Coding, JCTVC-F803, Torino, July 2011.

[4]	Y. Suzuki and TK Tan, " Extension of uni-prediction simplification in B slices," Joint Collaborative Team on Video Coding, JCTVC-D421, Daegu, Jan. 2011.

[5]	F. Bossen ," Common test conditions and software reference configurations ", Joint Collaborative Team on Video Coding, JCTVC-E600, Geneva, March 2011.

[6]	AHG report, " JCT-VC AHG report: Weighted prediction (AHG 18)," Joint Collaborative Team on Video Coding, JCTVC-F018, Torino, July 2011.

# Patent rights declaration(s)

TOSHIBA Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

-

-

-

-

-

-

-

-

-

-