| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G442 |
| --- | --- |

| Title: | Improvement to chroma TU specification |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Akira Minezawa Kazuo Sugimoto Shun-ichi Sekiguchi Information Technology R&D Center Mitsubishi Electric Corporation 5-1-1, Ofuna, Kamakura, JAPAN Atsuro Ichigaya Shinichi Sakaida NHK Science & Technology Research Laboratories 1-10-11, Kinuta, Setagaya-ku, Tokyo, JAPAN | Tel: Email: | +81-467-41-2463 Minezawa.Akira@ds.MitsubishiElectric.co.jp +81-3-5494-3354 ichigaya.a-go@nhk.or.jp |
| Source: | Mitsubishi Electric Corporation, NHK(Japan Broadcasting Corporation) |  |  |

_____________________________

# Abstract

This contribution proposes an improvement to chroma TU structure of the current HM. The proposed method introduces up to 32x32 TU size for chroma when LCU size is 64x64. In addition, it is proposed to apply fixed depth for chroma TU instead of using the same TU depth with luma component. The proposed method applies TU depth=0 for CU with 2Nx2N PU, and applies TU depth=1 for CU with rectangular PU and NxN PU while TU split for Luma is unchanged from HM-4. The proposed method achieves 2.0%, 2.1% and 1.0% chroma BD-rate gain for AI-HE, RA-HE and LD-HE, and 2.9%, 2.0% and 0.9% chroma BD-rate gain for AI-LC, RA-LC and LD-LC settings with 2-3% encoding time reduction. Luma BD-rate gain is on average 0.1% and 0.2% for HE and LC settings.

# Introduction

Chroma TU split is aligned with TU split for luma in HM-4 so that no overhead for chroma TU structure needs be signaled. In general, because of the low frequency characteristics of chroma signal in 4:2:0 format, larger TU size for chroma could represent chroma signals more efficiently. In this contribution, an improvement to chroma TU structure is proposed which enables applying larger TU size for chroma component compared to the current HM.

# Proposed technique

## Method 1: RQT extension of a 64x64 CU for chroma component

The proposed method is to extend RQT partitioning for TU depth=0 of a 64x64 CU. Figure 1 shows the new partitioning.

Fig 1. Proposed TU depth extension for a 64x64 CU

This method extended TU depth=0 for a 64x64 CU. Current HM can not choose TU depth=0 in the case of a 64x64 CU since 64x64 transform is not adopted. Therefore a luma TU depth is forced to be 1 and TU depth=0 is applied to chroma partitioning. This strategy is same as the combination of a 4x4 luma TU and a 4x4 chroma TU.

## Method 2: Fixed depth for chroma TU

The proposed technique applies fixed TU depth for chroma depending on partition mode as depicted in Fig. 2.

Fig. 2 Fixed TU depth for chroma

Chroma TU depth is set to 0 for CU with 2Nx2N partition mode, and chroma TU depth is set to 1 for other cases. By using this method, up to 32x32 transform, which performs better coding efficiency for smoother region, can be used for chroma component instead of 16x16 transform. As for luma TU tree, it can be constructed optimally by evaluating only luma component, as chroma TU tree construction is independent from luma TU tree construction, which brings not only luma coding performance but also encoder complexity reduction.

# Experimental results

The proposed techniques are implemented on HM-4.0 and evaluated using the common test conditions described in JCTVC-F900 [1].

## Result 1: The coding result of the method 1

Table 1: Performance of method 1 against HM-4.0

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | -1.9% | -2.0% | 0.0% | -1.4% | -1.5% |
| Class B | 0.1% | -0.7% | -0.8% | 0.1% | -0.1% | -0.1% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class E | 0.1% | -1.6% | -1.2% | 0.0% | -0.6% | -0.5% |
| Overall | 0.0% | -0.8% | -0.8% | 0.0% | -0.4% | -0.4% |
|  | 0.0% | -0.8% | -0.8% | 0.0% | -0.4% | -0.5% |
| Enc Time[%] | 101% |  |  | 100% |  |  |
| Dec Time[%] | 99% |  |  | 100% |  |  |
|  | Random Access HE |  |  | Random Access LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0.1% | -2.5% | -2.4% | 0.1% | -1.8% | -1.8% |
| Class B | 0.1% | -1.0% | -1.0% | 0.1% | -0.3% | -0.1% |
| Class C | 0.0% | -0.4% | -0.2% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | -0.1% | -0.2% | 0.0% | 0.0% | -0.1% |
| Class E |  |  |  |  |  |  |
| Overall | 0.0% | -1.0% | -1.0% | 0.0% | -0.5% | -0.5% |
|  | 0.0% | -1.0% | -0.9% | 0.0% | -0.5% | -0.5% |
| Enc Time[%] | 103% |  |  | 102% |  |  |
| Dec Time[%] | 99% |  |  | 101% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.1% | -1.4% | -1.9% | 0.1% | -0.5% | -0.3% |
| Class C | 0.0% | -0.5% | -0.3% | 0.0% | 0.0% | -0.2% |
| Class D | 0.1% | 0.5% | -0.2% | 0.0% | 0.3% | -0.1% |
| Class E | 0.1% | -4.0% | -1.5% | 0.0% | -1.2% | -1.6% |
| Overall | 0.1% | -1.2% | -1.0% | 0.0% | -0.3% | -0.5% |
|  | 0.1% | -1.2% | -1.1% | 0.0% | -0.3% | -0.5% |
| Enc Time[%] | 102% |  |  | 101% |  |  |
| Dec Time[%] | 100% |  |  | 98% |  |  |

## Result 2: The coding result of the method 2

Table 2: Performance of method 2 against HM-4.0

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.1% | -4.3% | -4.3% | -0.2% | -3.8% | -3.8% |
| Class B | -0.2% | -1.8% | -1.9% | -0.3% | -2.7% | -3.2% |
| Class C | -0.2% | -0.2% | -0.2% | -0.2% | -1.6% | -1.9% |
| Class D | -0.1% | -0.3% | -0.2% | -0.1% | -1.1% | -1.2% |
| Class E | -0.1% | -4.3% | -3.2% | -0.3% | -6.3% | -5.2% |
| Overall | -0.1% | -2.1% | -1.9% | -0.2% | -2.9% | -3.0% |
|  | -0.1% | -2.1% | -1.9% | -0.2% | -2.9% | -3.1% |
| Enc Time[%] | 101% |  |  | 99% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |
|  | Random Access HE |  |  | Random Access LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | -6.3% | -6.0% | -0.2% | -5.2% | -4.8% |
| Class B | -0.1% | -2.0% | -2.0% | -0.3% | -2.1% | -2.1% |
| Class C | -0.1% | -0.4% | -0.4% | -0.2% | -0.6% | -0.8% |
| Class D | 0.0% | 0.1% | -0.1% | -0.1% | -0.5% | -0.3% |
| Class E |  |  |  |  |  |  |
| Overall | -0.1% | -2.1% | -2.1% | -0.2% | -2.1% | -2.0% |
|  | -0.1% | -2.1% | -2.1% | -0.2% | -2.0% | -2.0% |
| Enc Time[%] | 97% |  |  | 97% |  |  |
| Dec Time[%] | 100% |  |  | 101% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.1% | -1.3% | -1.5% | -0.3% | -0.4% | -0.6% |
| Class C | -0.1% | -0.2% | -0.1% | -0.2% | 0.4% | -0.2% |
| Class D | -0.1% | 1.3% | 0.8% | -0.1% | 0.5% | -0.1% |
| Class E | 0.1% | -4.9% | -3.8% | -0.2% | -5.0% | -4.3% |
| Overall | 0.0% | -1.0% | -1.0% | -0.2% | -0.8% | -1.1% |
|  | 0.0% | -1.1% | -1.1% | -0.2% | -0.8% | -1.0% |
| Enc Time[%] | 98% |  |  | 97% |  |  |
| Dec Time[%] | 101% |  |  | 101% |  |  |

# Discussions

In method 1, TU depth=0 of a 64x64 CU is extended for chroma TU by using the same strategy of minimal TU size partitioning. There is not any change in other partitioning process. This method improves chroma coding performance of a 64x64 CU only, which means that the coding gain of this method comes directly from the introduction of 32x32 TU for chroma component. On the other hand, the encoder need to choose from 16x16 and 32x32 for the size of chroma TU when 32x32 TU is used for luma, which brings additional computational cost for the encoder decision.

In addition to method 1, method 2 uses fixed depth for chroma TU which applies TU depth=0 for CU with 2Nx2N PU, and applies TU depth=1 for CU with rectangular PU and NxN PU. Thus additional gain coming from applying fixed TU depth for chroma up to 1 can be estimated from the difference between the simulation results of method 1 and method 2. The encoder does not need additional computational cost for decision of TU size for chroma, as it is fixed depending on the CU size and PU mode. In addition, chroma TU cost calculation does not needed for Luma TU split decision, therefore encoding could be even faster than the original HM.

The proposed method 2 achieves 2.0%, 2.1% and 1.0% chroma BD-rate gain for AI-HE, RA-HE and LD-HE, and 2.9%, 2.0% and 0.9% chroma BD-rate gain for AI-LC, RA-LC and LD-LC settings with 2-3% encoding time reduction. 0.0%-0.2% luma BD-rate gain is also achieved.

# Conclusion

In this contribution two methods to modify chroma TU structure are proposed. It is recommended to adopt method 2 proposed in this contribution to HM5 and WD5.

# Reference

- Frank Bossen, “Common test conditions and software reference configurations”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-F900, 6th Meeting: Torino, IT, July, 2011.

# Patent rights declaration(s)

Mitsubishi Electric Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

NHK may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Appendix: Normative descriptions to be reflected to WD text

A description of the proposal is shown as follows.

### 7.3.6  Coding unit syntax

| coding_unit( x0, y0, log2CUSize ) { | Descriptor |
| --- | --- |
| if( entropy_coding_mode_flag && slice_type != I ) |  |
| skip_flag[ x0 ][ y0 ] | u(1) ¦ ae(v) |
| if( skip_flag[ x0 ][ y0 ] ) |  |
| prediction_unit( x0, y0 , log2CUSize ) |  |
| else { |  |
| if( !entropy_coding_mode_flag ) { |  |
| if( slice_type == I && log2CUSize == Log2MinCUSize ) |  |
| intra_part_mode | u(1) |
| } else if( slice_type != I ¦ ¦ log2CUSize = = Log2MinCUSize ) |  |
| pred_type | u(v) ¦ ae(v) |
| x1 = x0 + ( ( 1 << log2CUSize ) >> 1 ) |  |
| y1 = y0 + ( ( 1 << log2CUSize ) >> 1 ) |  |
| x2 = x1 − ( ( 1 << log2CUSize ) >> 2 ) |  |
| y2 = y1 − ( ( 1 << log2CUSize ) >> 2 ) |  |
| x3 = x1 + ( ( 1 << log2CUSize ) >> 2 ) |  |
| y3 = y1 + ( ( 1 << log2CUSize ) >> 2 ) |  |
| if( PartMode == PART_2Nx2N ) { |  |
| prediction_unit( x0, y0 , log2CUSize ) |  |
| } else if( PartMode == PART_2NxN ) { |  |
| prediction_unit( x0, y0 , log2CUSize ) |  |
| prediction_unit( x0, y1 , log2CUSize ) |  |
| } else if( PartMode == PART_Nx2N ) { |  |
| prediction_unit( x0, y0 , log2CUSize ) |  |
| prediction_unit( x1, y0 , log2CUSize ) |  |
| } else if( PartMode == PART_2NxnU ) { |  |
| prediction_unit( x0, y0 , log2CUSize ) |  |
| prediction_unit( x0, y2 , log2CUSize ) |  |
| } else if( PartMode == PART_2NxnD ) { |  |
| prediction_unit( x0, y0 , log2CUSize ) |  |
| prediction_unit( x0, y3 , log2CUSize ) |  |
| } else if( PartMode == PART_nLx2N ) { |  |
| prediction_unit( x0, y0 , log2CUSize ) |  |
| prediction_unit( x2, y0 , log2CUSize ) |  |
| } else if( PartMode == PART_nRx2N ) { |  |
| prediction_unit( x0, y0 , log2CUSize ) |  |
| prediction_unit( x3, y0 , log2CUSize ) |  |
| } else { /* PART_NxN */ |  |
| prediction_unit( x0, y0 , log2CUSize ) |  |
| prediction_unit( x1, y0 , log2CUSize ) |  |
| prediction_unit( x0, y1 , log2CUSize ) |  |
| prediction_unit( x1, y1 , log2CUSize ) |  |
| } |  |
| if( !pcm_flag ) { |  |
| if( entropy_coding_mode_flag && IntraSplitFlag = = 0) { |  |
| if( PredMode != MODE_INTRA ) { |  |
| if( !(PartMode == PART_2Nx2N && merge_flag[ x0 ][ y0 ]) ) |  |
| no_residual_data_flag | ae(v) |
| } |  |
| residualDataPresentFlag = !no_residual_data_flag |  |
| } else { |  |
| residualDataPresentFlag = TRUE |  |
| } |  |
| if( residualDataPresentFlag) { |  |
| chromaResidualDataPresentFlag = 0 |  |
| transform_tree_first( x0, y0, log2CUSize, log2CUSize, 0, 0 ) |  |
| transform_tree_second( x0, y0, log2CUSize, log2CUSize, 0, 0) |  |
| transform_coeff( x0, y0, log2CUSize, log2CUSize, 0, 0 ) |  |
| transform_coeff( x0, y0, log2CUSize, log2CUSize, 0, 1 ) |  |
| transform_coeff( x0, y0, log2CUSize, log2CUSize, 0, 2 ) |  |
| } |  |
| } |  |
| } |  |
| } |  |

### 7.3.8  Transform tree syntax

### 7.3.8.1  First transform tree syntax

| transform_tree_first( x0, y0, log2TrafoWidth, log2TrafoHeight, trafoDepth, blkIdx ) { | Descriptor |
| --- | --- |
| log2TrafoSize = ( log2TrafoWidth + log2TrafoHeight ) >> 2 |  |
| intraSplitFlag = ( IntraSplitFlag && trafoDepth = = 0 ? 1 : 0 ) |  |
| maxDepth = ( PredMode = = MODE_INTRA ? max_transform_hierarchy_depth_intra + IntraSplitFlag : max_transform_hierarchy_depth_inter ) |  |
| xBase = x0 − ( x0 & ( 1 << log2TrafoWidth ) ) |  |
| yBase = y0 − ( y0 & ( 1 << log2TrafoHeight ) ) |  |
| inferred_split_transform_flag[ x0 ][ y0 ][ trafoDepth ] = ( ((PartMode != PART_2Nx2N) && (trafoDepth = = 0)) ¦¦ (log2TrafoSize > Log2MaxTrafoSize) ) |  |
| if( !entropy_coding_mode_flag ) { |  |
| if( trafoDepth != 0 ) { |  |
| if( cbf_cb[ xBase ][ yBase ][ trafoDepth − 1 ] && log2TrafoSize > Log2MinTrafoSize ) |  |
| cbf_cb[ x0 ][ y0 ] [ trafoDepth ] | u(1) |
| if( cbf_cr[ xBase ][ yBase ][ trafoDepth − 1 ] && log2TrafoSize > Log2MinTrafoSize ) |  |
| cbf_cr[ x0 ][ y0 ] [ trafoDepth ] | u(1) |
| } else |  |
| cbp_and_split_transform | vlc(n,v) |
| } |  |
| if( log2TrafoSize <= Log2MaxTrafoSize && entropy_coding_mode_flag ) { |  |
| firstChromaCbf = ( log2TrafoSize = = Log2MaxTrafoSize ¦ ¦ trafoDepth = = 0 ? 1 : 0 ) |  |
| if( firstChromaCbf ¦ ¦ log2TrafoSize > Log2MinTrafoSize ) { |  |
| if( firstChromaCbf ¦ ¦ cbf_cb[ xBase ][ yBase ][ trafoDepth − 1 ] ) { |  |
| readCbf = true |  |
| if( blkIdx = = 3 && log2TrafoSize < Log2MaxTrafoSize ) |  |
| readCbf = cbf_cb[ xBase ][ yBase ][ trafoDepth ] ¦ ¦ cbf_cb[ xBase + ( 1 << log2TrafoWidth ) ][ yBase ][ trafoDepth ] ¦ ¦ cbf_cb[ xBase ][ yBase + ( 1 << log2TrafoHeight ) ][ trafoDepth ] |  |
| if ( !readCbf ) |  |
| cbf_cb[ x0 ][ y0 ][ trafoDepth ] = 1 |  |
| else |  |
| cbf_cb[ x0 ][ y0 ][ trafoDepth ] | ae(v) |
| if (cbf_cb[ x0 ][ y0 ][ trafoDepth ] = 1) |  |
| chromaResidualDataPresentFlag = 1 |  |
| } |  |
| if( firstChromaCbf ¦ ¦ cbf_cr[ xBase ][ yBase ][ trafoDepth − 1 ] ) { |  |
| readCbf = true |  |
| if( blkIdx = = 3 && log2TrafoSize < Log2MaxTrafoSize ) |  |
| readCbf = cbf_cr[ xBase ][ yBase ][ trafoDepth ] ¦ ¦ cbf_cr[ xBase + ( 1 << log2TrafoWidth ) ][ yBase ][ trafoDepth ] ¦ ¦ cbf_cr[ xBase ][ yBase + ( 1 << log2TrafoHeight ) ][ trafoDepth ] |  |
| if ( !readCbf ) |  |
| cbf_cr[ x0 ][ y0 ][ trafoDepth ] = 1 |  |
| else |  |
| cbf_cr[ x0 ][ y0 ][ trafoDepth ] | ae(v) |
| if (cbf_cr[ x0 ][ y0 ][ trafoDepth ] = 1) |  |
| chromaResidualDataPresentFlag = 1 |  |
| } |  |
| } |  |
| } |  |
| if( inferred_split_transform_flag[ x0 ][ y0 ][ trafoDepth ] ) { |  |
| if( InterTUSplitDirection = = 2 ) { |  |
| x1 = x0 + ( ( 1 << log2TrafoWidth ) >> 1 ) |  |
| y1 = y0 |  |
| x2 = x0 |  |
| y2 = y0 + ( ( 1 << log2TrafoHeight ) >> 1 ) |  |
| x3 = x1 |  |
| y3 = y2 |  |
| } else { |  |
| x1 = x0 + ( ( 1 << log2TrafoWidth ) >> 2 ) * InterTUSplitDirection |  |
| y1 = y0 + ( ( 1 << log2TrafoHeight) >> 2 ) * ( 1 − InterTUSplitDirection ) |  |
| x2 = x1 + ( ( 1 << log2TrafoWidth ) >> 2 ) * InterTUSplitDirection |  |
| y2 = y1 + ( ( 1 << log2TrafoHeight) >> 2 ) * ( 1 − InterTUSplitDirection ) |  |
| x3 = x2 + ( ( 1 << log2TrafoWidth ) >> 2 ) * InterTUSplitDirection |  |
| y3 = y2 + ( ( 1 << log2TrafoHeight) >> 2 ) * ( 1 − InterTUSplitDirection ) |  |
| log2TrafoHeight = log2TrafoHeight + 2 * InterTUSplitDirection − 1 |  |
| log2TrafoWidth = log2TrafoWidth − 2 * InterTUSplitDirection + 1 |  |
| } |  |
| transform_tree_first ( x0, y0, log2TrafoWidth − 1, log2TrafoHeight − 1, trafoDepth + 1, 0 ) |  |
| transform_tree_first ( x1, y1, log2TrafoWidth − 1, log2TrafoHeight − 1, trafoDepth + 1, 1 ) |  |
| transform_tree_first ( x2, y2, log2TrafoWidth − 1, log2TrafoHeight − 1, trafoDepth + 1, 2 ) |  |
| transform_tree_first ( x3, y3, log2TrafoWidth − 1, log2TrafoHeight − 1, trafoDepth + 1, 3 ) |  |
| } |  |
| } |  |

### 7.3.8.2  Second transform tree syntax

| transform_tree_second( x0, y0, log2TrafoWidth, log2TrafoHeight, trafoDepth, blkIdx ) { | Descriptor |
| --- | --- |
| log2TrafoSize = ( log2TrafoWidth + log2TrafoHeight ) >> 1 |  |
| intraSplitFlag = ( IntraSplitFlag && trafoDepth = = 0 ? 1 : 0 ) |  |
| maxDepth = ( PredMode = = MODE_INTRA ? max_transform_hierarchy_depth_intra + IntraSplitFlag : max_transform_hierarchy_depth_inter ) |  |
| xBase = x0 − ( x0 & ( 1 << log2TrafoWidth ) ) |  |
| yBase = y0 − ( y0 & ( 1 << log2TrafoHeight ) ) |  |
| if( !entropy_coding_mode_flag && trafoDepth != 0 ) { |  |
| if( !cbf_luma[ xBase ][ yBase ][ trafoDepth − 1 ] ) { |  |
| if( cbf_cb[ xBase ][ yBase ][ trafoDepth − 1 ] && log2TrafoSize > Log2MinTrafoSize ) |  |
| cbf_cb[ x0 ][ y0 ] [ trafoDepth ] | u(1) |
| if( cbf_cr[ xBase ][ yBase ][ trafoDepth − 1 ] && log2TrafoSize > Log2MinTrafoSize ) |  |
| cbf_cr[ x0 ][ y0 ] [ trafoDepth ] | u(1) |
| if(log2TrafoSize <= Log2MaxTrafoSize && !intraSplitFlag && log2TrafoSize > Log2MinTrafoSize && trafoDepth < maxDepth ) |  |
| split_transform_flag[ x0 ][ y0 ] [ trafoDepth ] | u(1) |
| } else |  |
| cbp_and_split_transform_luma | vlc(n,v) |
| } |  |
| if( log2TrafoSize <= Log2MaxTrafoSize && log2TrafoSize > Log2MinTrafoSize && trafoDepth < maxDepth && !intraSplitFlag && entropy_coding_mode_flag ) |  |
| split_transform_flag[ x0 ][ y0 ][ trafoDepth ] | ae(v) |
| if( split_transform_flag[ x0 ][ y0 ][ trafoDepth ] ) { |  |
| if( InterTUSplitDirection = = 2 ) { |  |
| x1 = x0 + ( ( 1 << log2TrafoWidth ) >> 1 ) |  |
| y1 = y0 |  |
| x2 = x0 |  |
| y2 = y0 + ( ( 1 << log2TrafoHeight ) >> 1 ) |  |
| x3 = x1 |  |
| y3 = y2 |  |
| } else { |  |
| x1 = x0 + ( ( 1 << log2TrafoWidth ) >> 2 ) * InterTUSplitDirection |  |
| y1 = y0 + ( ( 1 << log2TrafoHeight) >> 2 ) * ( 1 − InterTUSplitDirection ) |  |
| x2 = x1 + ( ( 1 << log2TrafoWidth ) >> 2 ) * InterTUSplitDirection |  |
| y2 = y1 + ( ( 1 << log2TrafoHeight) >> 2 ) * ( 1 − InterTUSplitDirection ) |  |
| x3 = x2 + ( ( 1 << log2TrafoWidth ) >> 2 ) * InterTUSplitDirection |  |
| y3 = y2 + ( ( 1 << log2TrafoHeight) >> 2 ) * ( 1 − InterTUSplitDirection ) |  |
| log2TrafoHeight = log2TrafoHeight + 2 * InterTUSplitDirection − 1 |  |
| log2TrafoWidth = log2TrafoWidth − 2 * InterTUSplitDirection + 1 |  |
| } |  |
| transform_tree_second ( x0, y0, log2TrafoWidth − 1, log2TrafoHeight − 1, trafoDepth + 1, 0 ) |  |
| transform_tree_second ( x1, y1, log2TrafoWidth − 1, log2TrafoHeight − 1, trafoDepth + 1, 1 ) |  |
| transform_tree_second ( x2, y2, log2TrafoWidth − 1, log2TrafoHeight − 1, trafoDepth + 1, 2 ) |  |
| transform_tree_second ( x3, y3, log2TrafoWidth − 1, log2TrafoHeight − 1, trafoDepth + 1, 3 ) |  |
| } else if( entropy_coding_mode_flag ) { |  |
| if( PredMode = = MODE_INTRA ¦ ¦ trafoDepth != 0 ¦ ¦ chromaResidualDataPresentFlag ) { |  |
| readCbf = true |  |
| if( blkIdx = = 3 && ((log2TrafoSize < Log2MaxTrafoSize) ¦¦ (chromaResidualDataPresentFlag && (log2CUSize <= Log2MaxTrafoSize+1) ) ) && PredMode != MODE_INTRA ) |  |
| readCbf = cbf_luma[ xBase ][ yBase ][ trafoDepth ] ¦ ¦ cbf_luma[ xBase + ( 1 << log2TrafoWidth ) ][ yBase ][ trafoDepth ] ¦ ¦ cbf_luma[ xBase ][ yBase + ( 1 << log2TrafoHeight ) ][ trafoDepth ] ¦ ¦ cbf_cb[ xBase ][ yBase ][ trafoDepth – 1] ¦ ¦ cbf_cr[ xBase ][ yBase ][ trafoDepth – 1] |  |
| if ( !readCbf ) |  |
| cbf_luma[ x0 ][ y0 ][ trafoDepth ] = 1 |  |
| else |  |
| cbf_luma[ x0 ][ y0 ][ trafoDepth ] | ae(v) |
| } |  |
| } |  |
| } |  |

### 7.3.9  Transform coefficient syntax

| transform_coeff( x0, y0, log2TrafoWidth, log2TrafoHeight, trafoDepth, cIdx ) { | Descriptor |
| --- | --- |
| if( ( ( cIdx = = 0 && cbf_luma[ x0 ][ y0 ][ trafoDepth ] ) ¦ ¦ ( cIdx = = 1 && cbf_cb[ x0 ][ y0 ][ trafoDepth ] ) ¦ ¦ ( cIdx = = 2 && cbf_cr[ x0 ][ y0 ][ trafoDepth ] ) ) ) { |  |
| if( cu_qp_delta_enabled_flag && !IsCuQpDeltaCoded ) { |  |
| cu_qp_delta | se(v) ¦ ae(v) |
| IsCuQpDeltaCoded = 1 |  |
| } |  |
| split_flag = (cIdx = = 0) ? split_transform_flag[ x0 ][ y0 ][ trafoDepth ] : inferred_split_transform_flag[ x0 ][ y0 ][ trafoDepth ] |  |
| if( split_flag && ( cIdx = = 0 ¦ ¦ log2TrafoSize > Log2MinTrafoSize + 1 ) ) { |  |
| if( InterTUSplitDirection = = 2 ) { |  |
| x1 = x0 + ( ( 1 << log2TrafoWidth ) >> 1 ) |  |
| y1 = y0 |  |
| x2 = x0 |  |
| y2 = y0 + ( ( 1 << log2TrafoHeight ) >> 1 ) |  |
| x3 = x1 |  |
| y3 = y2 |  |
| } else { |  |
| x1 = x0 + ( ( 1 << log2TrafoWidth ) >> 2 ) * InterTUSplitDirection |  |
| y1 = y0 + ( ( 1 << log2TrafoHeight) >> 2 ) * ( 1 − InterTUSplitDirection ) |  |
| x2 = x1 + ( ( 1 << log2TrafoWidth ) >> 2 ) * InterTUSplitDirection |  |
| y2 = y1 + ( ( 1 << log2TrafoHeight) >> 2 ) * ( 1 − InterTUSplitDirection ) |  |
| x3 = x2 + ( ( 1 << log2TrafoWidth ) >> 2 ) * InterTUSplitDirection |  |
| y3 = y2 + ( ( 1 << log2TrafoHeight) >> 2 ) * ( 1 − InterTUSplitDirection ) |  |
| log2TrafoHeight = log2TrafoHeight + 2 * InterTUSplitDirection − 1 |  |
| log2TrafoWidth = log2TrafoWidth − 2 * InterTUSplitDirection + 1 |  |
| } |  |
| transform_coeff( x0, y0, log2TrafoWidth − 1, log2TrafoHeight − 1, trafoDepth + 1, cIdx ) |  |
| transform_coeff( x1, y1, log2TrafoWidth − 1, log2TrafoHeight − 1, trafoDepth + 1, cIdx ) |  |
| transform_coeff( x2, y2, log2TrafoWidth − 1, log2TrafoHeight − 1, trafoDepth + 1, cIdx ) |  |
| transform_coeff( x3, y3, log2TrafoWidth − 1, log2TrafoHeight − 1, trafoDepth + 1, cIdx ) |  |
| } else { |  |
| log2TrafoSize = ( ( log2TrafoWidth + log2TrafoHeight ) >> 1 ) − ( cIdx > 0 ? 1 : 2 ) |  |
| if ( PredMode = = MODE_INTRA ) { |  |
| if (cIdx = = 0) |  |
| scanIdx = ScanType[ log2TrafoSize – 2 ][ IntraPredMode ] |  |
| else |  |
| scanIdx = ScanType[ log2TrafoSize – 2 ][ IntraPredModeC ] |  |
| } else { |  |
| scanIdx = 0 |  |
| } |  |
| if ( entropy_coding_mode_flag ) |  |
| if ( scanIdx = = 0 ) |  |
| scanIdx = 3 |  |
| residual_coding_cabac( x0, y0, log2TrafoSize, trafoDepth, scanIdx, cIdx ) |  |
| else if ( !entropy_coding_mode_flag ) |  |
| residual_coding_cavlc( x0, y0, log2TrafoSize, trafoDepth, cIdx ) |  |
| } |  |
| } |  |
| } |  |

[Ed. (BB): the condition ( cIdx  = =  0  | |  log2TrafoSize > Log2MinTrafoSize + 1 ) assumes 4:2:0 color sampling and assures that the luma log2TrafoSize is greater than Log2MinTrafoSize for chroma. ]

### 7.4.8  Transform tree semantics

…

…

…

split_transform_flag[ x0 ][ y0 ][ trafoDepth ] specifies whether a luma block is split into four blocks with smaller horizontal or vertical size for the purpose of transform coding. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered block relative to the top-left luma sample of the picture. The array index trafoDepth specifies the current subdivision level of a coding unit into blocks for the purpose of transform coding. trafoDepth is equal to 0 for blocks that correspond to coding units.

When split_transform_flag[ x0 ][ y0 ][ trafoDepth ] is not present, it is inferred as follows:

- If log2TrafoSize is greater than Log2MaxTrafoSize or intraSplitFlag is equal to 1, the value of split_transform_flag[ x0 ][ y0 ][ trafoDepth ] is inferred to be equal to 1.

- Otherwise (log2TrafoSize is less than or equal to Log2MaxTrafoSize and intraSplitFlag is equal to 0), the value of split_transform_flag[ x0 ][ y0 ][ trafoDepth ] is inferred to be equal to 0.

inferred_split_transform_flag [ x0 ][ y0 ][ trafoDepth ] specifies whether a chroma block is split into four blocks with smaller horizontal or vertical size for the purpose of transform coding. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered block relative to the top-left luma sample of the picture. The array index trafoDepth specifies the current subdivision level of a coding unit into blocks for the purpose of transform coding. trafoDepth is equal to 0 for blocks that correspond to coding units.

…

…

…

cbp_and_split_transform contains a number of coded block flags and/or a split_transform_flag. Depending on a 4-bit value flagPattern defined as follows, the value of cbp_and_split_transform is defined according to Table 7-15.

codeLuma = trafoDepth  = =  0  | |  cbf_luma[ xB ][ yB ][ trafoDepth − 1 ]
codeCb = 		trafoDepth  = =  0  | | (cbf_cb[ xB ][ yB ][ trafoDepth − 1 ]  &&  
				log2TrafoSize  >  Log2MinTrafoSize )
codeCr = 		trafoDepth  = =  0  | | (cbf_cr[ xB ][ yB ][ trafoDepth − 1 ]  &&  
				log2TrafoSize  >  Log2MinTrafoSize ) 	(7-1)
codeSplitTrans  =  log2TrafoSize  <=  Log2MaxTrafoSize  &&  
					log2TrafoSize  >  Log2MinTrafoSize  &&  
					trafoDepth  <  maxDepth  &&  
					!intraSplitFlag
flagPattern = (codeLuma << 3) + (codeCb << 2) + (codeCr << 1) + codeSplitTrans

The values of codeLuma, codeCb, codeCr and codeSplitTrans specify respectivley if an associated flag of cbf_luma, cbf_cb, cbf_cr, and split_transform_flag is included in the syntax element cbp_and_split_transform.

In Table 7-15, Luma, Cb, Cr and Split denote the values of cbf_luma[ x0 ][ y0 ][ trafoDepth ], cbf_cb[ x0 ][ y0 ][ trafoDepth ], cbf_cr[ x0 ][ y0 ][ trafoDepth ] and split_transform_flag[ x0 ][ y0 ][ trafoDepth ] respectively. Luma1, Luma2 and Luma3 denote respectively the values of cbf_luma[ x1 ][ y1 ][ trafoDepth ], cbf_luma[ x2 ][ y2 ][ trafoDepth ] and cbf_luma[ x3 ][ y3 ][ trafoDepth ]. The values of x1, y1, x2, y2, x3 and y3 are derived as follows.

If InterTUSplitDirection is equal to 2,

x1 = x0 + ( ( 1 << log2TrafoWidth ) >> 1 )		(7-2)

y1 = y0			(7-3)

x2 = x0			(7-4)

y2 = y0 + ( ( 1 << log2TrafoHeight ) >> 1 )		(7-5)

x3 = x1			(7-6)

y3 = y2			(7-7)

Otherwise (InterTUSplitDirection is equal to 0 or 1),

x1 = x0 + ( ( 1 << log2TrafoWidth ) >> 2 ) * InterTUSplitDirection	(7-8)

y1 = y0 + ( ( 1 << log2TrafoHeight) >> 2 ) * ( 1 − InterTUSplitDirection ) 	(7-9)

x2 = x1 + ( ( 1 << log2TrafoWidth ) >> 2 ) * InterTUSplitDirection	(7-10)

y2 = y1 + ( ( 1 << log2TrafoHeight) >> 2 ) * ( 1 − InterTUSplitDirection ) 	(7-11)

x3 = x2 + ( ( 1 << log2TrafoWidth ) >> 2 ) * InterTUSplitDirection	(7-12)

y3 = y2 + ( ( 1 << log2TrafoHeight) >> 2 ) * ( 1 − InterTUSplitDirection ) 	(7-13)

Table 7-15 - Specification of cbp_and_split_transform

| - flagPattern | - cbp_and_split_transform |
| --- | --- |
| - 8 | - (Luma<<3) + (Luma1<<2) + (Luma2<<1) + Luma3 - 1 |
| - 9 | - (Luma<<1) + Split |
| - 10 | - (Luma<<1) + Cr |
| - 11 | - (Luma<<2) + (Cr<<1) + Split |
| - 12 | - (Luma<<1) + Cb |
| - 13 | - (Luma<<2) + (Cb<<1) + Split |
| - 14 | - (Luma<<2) + (Cb<<1) + Cr |
| - 15 | - (Luma<<2) + ((Cb¦Cr)<<1) + Split |

cbp_and_split_transform_luma contains a number of coded block flags and/or a split_transform_flag. Depending on a 4-bit value flagPattern defined as follows, the value of cbp_and_split_transform is defined according to Table 7-15.

codeLuma = trafoDepth  = =  0  | |  cbf_luma[ xB ][ yB ][ trafoDepth − 1 ]	(7-14)
codeSplitTrans  =  log2TrafoSize  <=  Log2MaxTrafoSize  &&  
					log2TrafoSize  >  Log2MinTrafoSize  &&  
					trafoDepth  <  maxDepth  &&  
					!intraSplitFlag
flagPattern = (codeLuma << 3) + codeSplitTrans

The values of codeLuma and codeSplitTrans specify respectivley if an associated flag of cbf_luma and split_transform_flag is included in the syntax element cbp_and_split_transform.

### 8.4.3  Decoding process for the residual signal of coding units coded in inter prediction mode

…

…

…

### 8.4.3.2  Decoding process for chroma residual blocks

Inputs to this process are:

–	a luma location ( xC, yC ) specifying the top-left luma sample of the current coding unit relative to the top-left luma sample of the current picture,

–	a luma location ( xB, yB ) specifying the top-left luma sample of the current block relative to the top-left luma sample of the current coding unit,

–	a variable log2TrafoWidth specifying the width of the current block,

–	a variable log2TrafoHeight specifying the height of the current block,

–	a variable trafoDepth specifying the hierarchy depth of the current block relative to the coding unit,

–	a variable cIdx specifying the chroma component of the current block,

–	a variable nCS specifying the size, in chroma samples, of the current coding unit,

–	a (nCS)x(nCS) array resSamples of chroma residual samples.

Output of this process is:

–	a modified version of the (nCS)x(nCS) array of chroma residual samples.

The variable splitChromaFlag is derived as follows:

–	If inferred_split_transform_flag[ xB ][ yB ][ trafoDepth ] is equal to 1 and log2TrafoSize is greater than Log2MinTrafoSize + 1, splitChromaFlag is set equal to 1.

–	Otherwise (inferred_split_transform_flag[ xB ][ yB ][ trafoDepth ] is equal to 0 or log2TrafoSize is equal to Log2MinTrafoSize + 1), splitChromaFlag is set equal to 0.

Depending inferred_splitChromaFlag, the following applies:

–	If inferred_splitChromaFlag is equal to 1, the following ordered steps apply:

- The variables xB1, yB1, xB2, yB2, xB3 and yB3 are derived as follows.

  - If InterTUSplitDirection is equal to 2, the following applies.

  - The variable xB1 is set equal to xB + ( ( 1 << log2TrafoWidth ) >> 1 ).

  - The variable yB1 is set equal to yB.

  - The variable xB2 is set equal to xB.

  - The variable yB2 is set equal to yB + ( ( 1 << log2TrafoHeight ) >> 1 ).

  - The variable xB3 is set equal to xB1.

  - The variable yB3 is set equal to yB2.

  - The variable log2TrafoWidth1 is set equal to log2TrafoWidth − 1.

  - The variable log2TrafoHeight1 is set equal to log2TrafoHeight − 1.

  - Otherwise (InterTUSplitDirection is equal to 0 or 1), the following applies.

  - The variable xB1 is set equal to xB + ((1 << (log2TrafoWidth)) >> 2) * InterTUSplitDirection.

  - The variable yB1 is set equal to yB + ((1 << (log2TrafoHeight)) >> 2) * (1 − InterTUSplitDirection).

  - The variable xB2 is set equal to xB1 + ((1 << (log2TrafoWidth)) >> 2) * InterTUSplitDirection.

  - The variable yB2 is set equal to yB1 + ((1 << (log2TrafoHeight)) >> 2) * (1 − InterTUSplitDirection).

  - The variable xB3 is set equal to xB2 + ((1 << (log2TrafoWidth)) >> 2) * InterTUSplitDirection.

  - The variable yB3 is set equal to yB2 + ((1 << (log2TrafoHeight)) >> 2) * (1 − InterTUSplitDirection).

  - The variable log2TrafoWidth1 is set equal to (log2TrafoWidth − 2) * InterTUSplitDirection.

  - The variable log2TrafoHeight1 is set equal to (log2TrafoHeight − 2) * (1 − InterTUSplitDirection).

- The decoding process for residual chroma blocks as specified in this subclause is invoked with the luma location ( xC, yC ), the luma location ( xB, yB ), the variable log2TrafoWidth set equal to log2TrafoWidth1, the variable log2TrafoHeight set equal to log2TrafoHeight1, the variable trafoDepth set equal to trafoDepth + 1, the variable cIdx, the variable nCS, and the (nCS)x(nCS) array resSamples as the inputs and the output is a modified version of the (nCS)x(nCS) array resSamples.

- The decoding process for residual chroma blocks as specified in this subclause is invoked with the luma location ( xC, yC ), the luma location ( xB1, yB1 ), the variable log2TrafoWidth set equal to log2TrafoWidth1, the variable log2TrafoHeight set equal to log2TrafoHeight1, the variable trafoDepth set equal to trafoDepth + 1, the variable cIdx, the variable nCS, and the (nCS)x(nCS) array resSamples as the inputs and the output is a modified version of the (nCS)x(nCS) array resSamples.

- The decoding process for residual chroma blocks as specified in this subclause is invoked with the luma location ( xC, yC ), the luma location ( xB2, yB2 ), the variable log2TrafoWidth set equal to log2TrafoWidth1, the variable log2TrafoHeight set equal to log2TrafoHeight1, the variable trafoDepth set equal to trafoDepth + 1, the variable cIdx, the variable nCS, and the (nCS)x(nCS) array resSamples as the inputs and the output is a modified version of the (nCS)x(nCS) array resSamples.

- The decoding process for residual chroma blocks as specified in this subclause is invoked with the luma location ( xC, yC ), the luma location ( xB3, yB3 ), the variable log2TrafoWidth set equal to log2TrafoWidth1, the variable log2TrafoHeight set equal to log2TrafoHeight1, the variable trafoDepth set equal to trafoDepth + 1, the variable cIdx, the variable nCS, and the (nCS)x(nCS) array resSamples as the inputs and the output is a modified version of the (nCS)x(nCS) array resSamples.

–	Otherwise (inferred_splitChromaFlag is equal to 0), the following ordered steps apply:

- The variable nW is set equal to ( 1 << log2TrafoWidth ) >> 1.

- The variable nH is set equal to (1 << log2TrafoHeight ) >> 1.

- The scaling and transformation process as specified in subclause Error: Reference source not found is invoked with the luma location ( xC + xB, yC +yB ), the variable trafoDepth, the variable cIdx, the transform width trafoWidth set equal to nW, and the transform height trafoHeight set equal to nH as the inputs and the output is a (nW)x(nH) array resSamplesBlock.

The array construction process as specified in subclause Error: Reference source not found is invoked with the luma location ( xB, yB ), the variable cIdx, the variable inputArrayWidth set equal to nW, the variable inputArrayHeight set equal to nH, the variable outputArraySize set equal to nCS, the (nW)x(nH) array resSamplesBlock, and the (nCS)x(nCS) array resSamples as the inputs and the output is a modified version of the (nCS)x(nCS) array resSamples.

### 8.6.1  Deblocking filter process

…

…

…

### 8.6.1.1  Derivation process of luma transform unit boundary

…

…

…

### 8.6.1.2  Derivation process of chroma transform unit boundary

Inputs of this process are:

–	a luma location ( xB, yB ) specifying the top-left luma sample of the current block relative to the top-left luma sample of the current coding unit,

–	a variable log2TrafoWidth specifying the width of the current block,

–	a variable log2TrafoHeight specifying the height of the current block,

–	a variable trafoDepth.

Outputs of this process are:

–	two-dimensional arrays of (nS)x(nS), horEdgeFlagsChroma and verEdgeFlagsChroma.

Depending on inferred_split_transform_flag[ xB ][ yB ][ trafoDepth ], the following applies:

–	If inferred_split_transform_flag[ xB ][ yB ][ trafoDepth ] is equal to 1, the following ordered steps apply:

- The variables xB1, yB1, xB2, yB2, xB3 and yB3 are derived as follows.

  - If InterTUSplitDirection is equal to 2, the following applies.

  - The variable xB1 is set equal to xB + ( ( 1 << log2TrafoWidth ) >> 1 ).

  - The variable yB1 is set equal to yB.

  - The variable xB2 is set equal to xB.

  - The variable yB1 is set equal to yB + ( ( 1 << log2TrafoHeight ) >> 1 ).

  - The variable xB3 is set equal to xB1.

  - The variable yB3 is set equal to yB2.

  - The variable log2TrafoWidth1 is set equal to log2TrafoWidth − 1.

  - The variable log2TrafoHeight1 is set equal to log2TrafoHeight − 1.

  - Otherwise (InterTUSplitDirection is equal to 0 or 1), the following applies.

  - The variable xB1 is set equal to xB + ((1 << (log2TrafoWidth)) >> 2) * InterTUSplitDirection.

  - The variable yB1 is set equal to yB + ((1 << (log2TrafoHeight)) >> 2) * (1 − InterTUSplitDirection).

  - The variable xB2 is set equal to xB1 + ((1 << (log2TrafoWidth)) >> 2) * InterTUSplitDirection.

  - The variable yB2 is set equal to yB1 + ((1 << (log2TrafoHeight)) >> 2) * (1 − InterTUSplitDirection).

  - The variable xB3 is set equal to xB2 + ((1 << (log2TrafoWidth)) >> 2) * InterTUSplitDirection.

  - The variable yB3 is set equal to yB2 + ((1 << (log2TrafoHeight)) >> 2) * (1 − InterTUSplitDirection).

  - The variable log2TrafoWidth1 is set equal to (log2TrafoWidth − 2) * InterTUSplitDirection.

  - The variable log2TrafoHeight1 is set equal to (log2TrafoHeight − 2) * (1 − InterTUSplitDirection).

- The deriviation process of transform unit boundary as specified in this subclause is invoked with the luma location ( xB, yB ), the variable log2TrafoWidth set equal to log2TrafoWidth1, the variable log2TrafoHeight set equal to log2TrafoHeight1 and the variable trafoDepth1 set equal to trafoDepth + 1 as inputs and the outputs are the modified versions of two arrays, horEdgeFlags and verEdgeFlags.

- The deriviation process of transform unit boundary as specified in this subclause is invoked with the luma location ( xB1, yB1 ), the variable log2TrafoWidth set equal to log2TrafoSizeWidth1, the variable log2TrafoHeight set equal to log2TrafoHeight1 and the variable trafoDepth1 set equal to trafoDepth + 1 as inputs and the outputs are the modified versions of two arrays, horEdgeFlags and verEdgeFlags.

- The deriviation process of transform unit boundary as specified in this subclause is invoked with the luma location ( xB2, yB2 ), the variable log2TrafoWidth set equal to log2TrafoSizeWidth1, the variable log2TrafoHeight set equal to log2TrafoHeight1 and the variable trafoDepth1 set equal to trafoDepth + 1 as inputs and the outputs are the modified versions of two arrays, horEdgeFlags and verEdgeFlags.

- The deriviation process of transform unit boundary as specified in this subclause is invoked with the luma location ( xB3, yB3 ), the variable log2TrafoWidth1 set equal to log2TrafoSizeWidth1, the variable log2TrafoHeight set equal to log2TrafoHeight1 and the variable trafoDepth1 set equal to trafoDepth + 1 as inputs and the outputs are the modified versions of two arrays, horEdgeFlags and verEdgeFlags.

–	Otherwise (inferred_split_transform_flag[ xB ][ yB ][ trafoDepth ] is equal to 0), the following applies:

- If yB is equal to zero, horEdgeFlagsChroma[ xB + k ][ yB ] is set equal to FilterTopCuEdgeFlag, otherwise horEdgeFlagsChroma[ xB + k ][ yB ] is set equal to FilterInternalEdgesFlag for k = 0.. ( 1 << log2TrafoWidth ) – 1.

- If xB is equal to zero, verEdgeFlagsChroma[ xB ][ yB + k ] is set equal to FilterLeftCuEdgeFlag, otherwise verEdgeFlagsChroma[ xB ][ yB + k ] is set equal to FilterInternalEdgesFlag for k = 0.. ( 1 << log2TrafoHeight ) – 1.

### 8.6.1.3  Derivation process of prediction unit boundary

…

…

…

#### 8.6.1.4  Derivation process of luma boundary filtering strength

Inputs of this process are:

–	a luma location ( xC, yC ) specifying the top-left luma sample of the current coding unit relative to the top-left luma sample of the current picture,

–	a variable log2CUSize specifying the size of the current coding unit,

–	a two-dimensional arrays of size (nS)x(nS), horEdgeFlags and verEdgeFlags.

Output of this process is an array of size (2)x(nS)x(nS), bS specifying the luma boundary filtering strength.

…

…

…

#### 8.6.1.5  Derivation process of chroma boundary filtering strength

Inputs of this process are:

–	a luma location ( xC, yC ) specifying the top-left luma sample of the current coding unit relative to the top-left luma sample of the current picture,

–	a variable log2CUSize specifying the size of the current coding unit,

–	a two-dimensional arrays of size (nS)x(nS), horEdgeFlagsChroma and verEdgeFlagsChroma.

Output of this process is an array of size (2)x(nS)x(nS), bSChroma specifying the chroma boundary filtering strength.

Let ( xEk, yEj ) with k = 0..nE-1 and j = 0..nE-1 specify a set of edge sample locations where nE is set equal to ( ( 1 << log2CUSize ) >> 2 ), xE0 = 0, yE0 = 0, xEk+1 = xEk + 4 and yEj+1 = yEj + 4.

For ( xEk, yEj ) with k = 0..nE-1 and j = 0..nE-1, the following applies.

- If horEdgeFlagsChroma[ xEk ][ yEj ] is equal to 1,

- Set sample p0 = recPicture[ xC + xEk ][ yC + yEj – 1 ] and q0 = recPicture[ xC + xEk ][ yC + yEj ].

- The variable filterDir is set equal to 1.

- Otherwise, if verEdgeFlagsChroma[ xEk ][ yEj ] is equal to 1,

- Set sample p0 = recPicture[ xC + xEk – 1 ][ yC + yEj ] and q0 = recPicture[ xC + xEk ][ yC + yEj ].

- The variable filterDir is set equal to 0.

- Depending on the value of filterDir, the variable bSChroma[ filterDir ][ xEk ][ yEj ] is derived as follows.

- If the block edge is also a coding unit edge and the following condition is true, the variable bSChroma[ filterDir ][ xEk ][ yEj ] is set equal to 4.

- The sample p0 or q0 is in a coding unit coded with intra prediction mode

- Otherwise, if the following condition is true, the variable bSChroma[ filterDir ][ xEk ][ yEj ] is set equal to 3.

- The sample p0 or q0 is in a coding unit coded with intra prediction mode

- Otherwise, if the block edge is also a transform unit edge and the following condition is true, the variable bSChroma[ filterDir ][ xEk ][ yEj ] is set equal to 2.

- The sample p0 or q0 is in a transform unit which contains non-zero transform coefficient level.

- Otherwise, if any of the following conditions are true, the variable bSChroma[ filterDir ][ xEk ][ yEj ] is set equal to 1.

- The prediction unit containing sample p0 has different reference pictures or a different number of motion vectors with the prediction unit containing the sample q0.

NOTE – The determination of whether the reference pictures used for the two prediction are the same or different is based on which pictures are referenced, without regard to whether a prediction is formed using an index into list 0 or an index into list 1, and also without regard to whether or not the index position within a reference picture list is different or not.

- One motion vector is used to predict the prediction unit containing sample p0, one motion vector is used to predict the prediction unit containing sample q0, and the absolute difference between the horizontal or vertical component of the motion vector used is greater than or equal to 4 in units of quarter luma samples.

- [Ed.: (WJ) needs to be checked again whether this condition covers all 2-motion cases] Two motion vectors are used to predict the prediction unit containing sample p0, two motion vectors are used to predict the prediction unit containing sample q0, and at least one of the motion vector pairs corresponding the same reference pictures and the different boundary samples p0 and q0 satisfies the following condition:

- The absolute difference between the horizontal or vertical component of a motion vector used in the prediction of the two prediction units is greater than or equal to 4 in units of quarter luma samples.

- Otherwise, the variable bSChroma[ filterDir ][ xEk ][ yEj ] is set equal to 0.

#### 8.6.1.6  Filtering process for coding unit

Inputs of this process are:

–	a luma location ( xC, yC ) specifying the top-left luma sample of the current coding unit relative to the top left luma sample of the current picture,

–	a variable log2CUSize specifying the coding unit size,

–	an array bS specifying the boundary filtering strength.

Output of this process is:

–	modified reconstruction of the picture.

The filtering process for luma edges in the current coding unit consists of the following ordered steps:

…

…

…

The filtering process for chroma edges in the current coding unit consists of the following ordered steps:

- The variable nD is set equal to 1 << ( Max( log2CUSize, 4 ) – 4 ).

- For xDk set equal to ( xC / 2 )+( k << 3 ), k=0..nD – 1, the following applies:

- For yDm set equal to ( yC / 2)+( m << 2 ), m=0..nD*2 – 1, the following ordered steps apply:

- Boundary filtering strength bSVer is derived as follows:

bSVer = bSChroma[ 0 ][ xDk*2 ][ yDm*2 ] 	(8-15)

- Filtering process for chroma block edge in subclause Error: Reference source not found is invoked with the chroma location of the coding unit ( xC/2, yC/2 ), the chroma location of the block ( xDk, yDm ), a variable verticalEdgeFlag set equal to 1, a chroma component index cIdx set equal to 1 and the boundary filtering strength bSVer as inputs and the modified chroma picture buffer as output.

- Filtering process for chroma block edge in subclause Error: Reference source not found is invoked with the chroma location of the coding unit ( xC/2, yC/2 ), the chroma location of the block ( xDk, yDm ), a variable verticalEdgeFlag set equal to 1, a chroma component index cIdx set equal to 2 and the boundary filtering strength bSVer as inputs and the modified chroma picture buffer as output.

- For yDm set equal to ( yC / 2 )+( m << 3 ), m=0..nD – 1, the following applies:

- For xDk set equal to ( xC / 2 )+( k << 2 ), k=0..nD*2 – 1, the following ordered steps apply:

- If xDk is equal to 0, the parameter xPOS is set equal to 1. If xDk is equal to xB+( ( nD*2 – 1) << 2 ) xPOS is set equal to 2. Otherwise xPOS is set to 0.

- Boundary filtering strength bSHor is derived as follows:

bSHor = bSChroma[ 1 ][ xDk*2 ][ yDm*2 ] 	(8-16)

- Filtering process for chroma block edge in subclause Error: Reference source not found is invoked with the chroma location of the coding unit ( xC/2, yC/2 ), the chroma location of the block ( xDk, yDm ), a variable verticalEdgeFlag set equal to 0, a chroma component index cIdx set equal to 1 and the boundary filtering strength bSHor, xPOS, bSL[m] and tCL[m] as inputs and the modified chroma picture buffer as output.

- Filtering process for chroma block edge in subclause Error: Reference source not found is invoked with the chroma location of the coding unit ( xC/2, yC/2 ), the chroma location of the block ( xDk, yDm ), a variable verticalEdgeFlag set equal to 0, a chroma component index cIdx set equal to 2 and the boundary filtering strength bSHor, xPOS, bSL[m] and tCL[m] as inputs and the modified chroma picture buffer as output.