| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G443 |
| --- | --- |

| Title: | Improved directional intra prediction smoothing |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Akira Minezawa Kazuo Sugimoto Shun-ichi Sekiguchi Information Technology R&D Center Mitsubishi Electric Corporation 5-1-1, Ofuna, Kamakura, JAPAN | Tel: Email: | +81-467-41-2463 Minezawa.Akira@ds.MitsubishiElectric.co.jp |
| Source: | Mitsubishi Electric Corporation |  |  |

_____________________________

# Abstract

In this contribution, a modification on intra angular prediction smoothing is proposed. It is reported that the proposed scheme improves the harmonized scheme of intra prediction smoothing with secondary boundary based on CE6.d results without any additional calculations. It is reported that the proposed scheme improves the 0.02% and 0.01% in BD-rate performance on average with the same complexity for high efficiency and low complexity intra configuration respectively. It is also reported that the simplified version of the proposed scheme achieves the same coding performance compared to the anchor with complexity reduction.

# Introduction

In CE6.d, a harmonized scheme of intra prediction smoothing with secondary boundary shown in Figure 1 in terms of tradeoff between coding performance and computational complexity[1].

In this contribution, we introduce minor modifications to improve the harmonized scheme in terms of tradeoff between coding performance and computational complexity.

Figure 1.  The harmonized scheme of intra prediction smoothing with secondary boundary

# Proposed scheme

We introduce to modify the position of the reference samples to be used for filtering in some modes as shown in Figure 2. In the proposed method, the reference samples to be used for intra prediction modes with steep angle are modified so that the reference samples are aligned to the direction of the intra prediction. We also propose a simplified version of the proposed scheme as shown in Figure 3 to reduce the complexity. In the simplified method, the intra prediction filtering is disabled for the intra prediction modes which have down-right directions.

Figure 2.  harmonized scheme with the proposed modification

Figure 3.  Simplified version of the harmonized scheme with the proposed modification

# Simulations & Results

The proposed schemes have been implemented on top of the HM-4.0 based CE6.d software, and the simulations have been conducted using the common test configurations [2]. Table 1 shows the results of the original harmonization scheme compared with original HM-4.0 anchor.

Table 1. BD bitrate savings of the original harmonized scheme

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.93% | -0.82% | -0.86% | -0.83% | -0.98% | -1.12% |
| Class B | -0.63% | -0.60% | -0.59% | -0.57% | -0.78% | -0.78% |
| Class C | -0.64% | -0.65% | -0.66% | -0.68% | -0.77% | -0.73% |
| Class D | -0.66% | -0.64% | -0.65% | -0.70% | -0.69% | -0.73% |
| Class E | -0.80% | -0.71% | -0.87% | -0.75% | -0.88% | -0.92% |
| Overall | -0.72% | -0.68% | -0.71% | -0.70% | -0.82% | -0.85% |
|  | -0.73% | -0.69% | -0.71% | -0.70% | -0.82% | -0.85% |
| Enc Time[%] | 100% |  |  | 101% |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  |

The performance of the proposed scheme in Figure 2 compared with original HM-4.0 anchor is shown in Table 2.

Table 2. BD bitrate savings of the proposed scheme shown in Figure 2

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.91% | -0.78% | -0.87% | -0.81% | -0.93% | -1.08% |
| Class B | -0.66% | -0.62% | -0.60% | -0.60% | -0.78% | -0.78% |
| Class C | -0.66% | -0.65% | -0.64% | -0.71% | -0.79% | -0.72% |
| Class D | -0.68% | -0.62% | -0.62% | -0.71% | -0.64% | -0.73% |
| Class E | -0.83% | -0.75% | -0.83% | -0.76% | -0.82% | -0.91% |
| Overall | -0.74% | -0.68% | -0.70% | -0.71% | -0.79% | -0.84% |
|  | -0.74% | -0.68% | -0.70% | -0.71% | -0.79% | -0.84% |
| Enc Time[%] | 100% |  |  | 101% |  |  |
| Dec Time[%] | 98% |  |  | 99% |  |  |

Table 3 shows the performance of the proposed simplified scheme shown in Figure 3 compared with the original HM-4.0 anchor.

Table 3. BD bitrate savings of the simplified version of the proposed scheme shown in Figure 3

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.92% | -0.78% | -0.81% | -0.82% | -0.96% | -1.06% |
| Class B | -0.65% | -0.59% | -0.60% | -0.59% | -0.77% | -0.79% |
| Class C | -0.64% | -0.61% | -0.64% | -0.68% | -0.75% | -0.70% |
| Class D | -0.64% | -0.61% | -0.63% | -0.68% | -0.69% | -0.71% |
| Class E | -0.82% | -0.69% | -0.83% | -0.75% | -0.90% | -0.89% |
| Overall | -0.72% | -0.65% | -0.69% | -0.70% | -0.80% | -0.82% |
|  | -0.73% | -0.66% | -0.70% | -0.70% | -0.81% | -0.82% |
| Enc Time[%] | 100% |  |  | 101% |  |  |
| Dec Time[%] | 98% |  |  | 100% |  |  |

# Additional investigation

We have also investigated some intra prediction smoothing schemes with restricting the filtering to only 1-line. 1-line filter version of the CE6’s harmonized scheme shown in Figure 1 is depicted in Figure 4. 1-line filter version of the proposed scheme and its simplified version are shown in Figure5 and 6, respectively.

Figure 4.  1-line filtering version of harmonized scheme of CE6

Figure 5.  1-line filtering version of the proposed scheme

Figure 6.  1-line filtering version of the simplified proposed scheme

Three schemes shown in Figures 4 to 6 have been implemented on top of the HM-4.0 based CE6.d software, respectively. Simulations have been conducted using the common test configurations [2]. Table 4 shows the results of 1-line filter version of the original harmonized scheme compared with the original HM-4.0 anchor.

Table 4. BD bitrate savings of 1-line filtering version of the original harmonized scheme

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.75% | -0.64% | -0.69% | -0.64% | -0.76% | -0.87% |
| Class B | -0.53% | -0.49% | -0.52% | -0.44% | -0.65% | -0.67% |
| Class C | -0.55% | -0.56% | -0.54% | -0.57% | -0.67% | -0.60% |
| Class D | -0.59% | -0.55% | -0.63% | -0.61% | -0.59% | -0.66% |
| Class E | -0.66% | -0.58% | -0.67% | -0.60% | -0.72% | -0.78% |
| Overall | -0.61% | -0.56% | -0.60% | -0.56% | -0.67% | -0.71% |
|  | -0.61% | -0.56% | -0.61% | -0.57% | -0.68% | -0.71% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 97% |  |  | 98% |  |  |

The performance of the proposed scheme in Figure 5 compared with the original HM-4.0 anchor is shown in Table 5.

Table 5. BD bitrate savings of 1-line filtering version of the proposed scheme

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.73% | -0.62% | -0.62% | -0.61% | -0.74% | -0.85% |
| Class B | -0.55% | -0.49% | -0.52% | -0.46% | -0.64% | -0.67% |
| Class C | -0.58% | -0.54% | -0.52% | -0.60% | -0.63% | -0.60% |
| Class D | -0.60% | -0.57% | -0.59% | -0.63% | -0.59% | -0.64% |
| Class E | -0.70% | -0.53% | -0.62% | -0.61% | -0.65% | -0.77% |
| Overall | -0.62% | -0.55% | -0.57% | -0.58% | -0.65% | -0.70% |
|  | -0.63% | -0.56% | -0.58% | -0.58% | -0.65% | -0.70% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 98% |  |  | 98% |  |  |

Table 6 shows the performance of the simplified version of the proposed scheme shown in Figure 6 compared with the original HM-4.0 anchor.

Table 6. BD bitrate savings of 1-line filtering version of the simplified version of the proposed scheme

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.74% | -0.63% | -0.61% | -0.62% | -0.72% | -0.86% |
| Class B | -0.54% | -0.48% | -0.50% | -0.45% | -0.66% | -0.63% |
| Class C | -0.54% | -0.51% | -0.49% | -0.56% | -0.64% | -0.60% |
| Class D | -0.56% | -0.51% | -0.55% | -0.60% | -0.56% | -0.61% |
| Class E | -0.68% | -0.52% | -0.63% | -0.61% | -0.72% | -0.75% |
| Overall | -0.61% | -0.53% | -0.55% | -0.56% | -0.66% | -0.68% |
|  | -0.61% | -0.54% | -0.56% | -0.56% | -0.65% | -0.69% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 98% |  |  | 100% |  |  |

# Conclusion

In this contribution, a modification on intra angular prediction smoothing is proposed. It has been confirmed that the proposed scheme achieves the same BD-rate performance on average with reduction of computational complexity for both high efficiency and low complexity intra configurations. We propose the scheme shown in Figure 3 to be adopted to HM-5.

# Reference

- Xun Guo “CE6.d: Intra Prediction with Secondary Boundary”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-G280, 7th Meeting: Geneva, CH, November 2011.

- Frank Bossen, “Common test conditions and software reference configurations”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-E900, 6th Meeting: Torino, IT, July 2011.

# Patent rights declaration(s)

Mitsubishi Electric Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).