| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G0444_r1 |
| --- | --- |

| Title: | Proposed fix on cbf flag signaling |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Akira Minezawa † Bin Li * Kazuo Sugimoto † Shun-ichi Sekiguchi † Jizheng Xu § † Information Technology R&D Center Mitsubishi Electric Corporation 5-1-1, Ofuna, Kamakura, JAPAN | Tel: Email: | +81-467-41-2463 Minezawa.Akira@ds.MitsubishiElectric.co.jp jzxu@microsoft.com |
| Source: | † Mitsubishi Electric Corporation * University of Science and Technology of China § Microsoft Corp. |  |  |

_____________________________

# Abstract

In this contribution, a fix on condition for cbf flag to be inferred is proposed. In the proposed fix, a  condition for inferred cbf flag decision is refined to increase the chance to infer cbf flag. This modification is implemented onto HM-4.0 software, and simulations are conducted using common test configurations to evaluate the performance of the fix. It is reportedly confirmed that the proposed fix shows almost the same BD-rate performance on average compared to the original HM-4.0.

# Introduction

In HM-4, in some cases cbf flag is inferred instead of signaling explicitly. In this contribution, we propose to remove an unnecessary condition of inferred cbf flag decision for cleaning-up HM software and WD [1].

# Proposed fix

In WD, transform tree syntax is described as follows.

| transform_tree( x0, y0, log2TrafoWidth, log2TrafoHeight, trafoDepth, blkIdx ) { | Descriptor |
| --- | --- |
| if( entropy_coding_mode_flag && trafoDepth = = 0 && IntraSplitFlag = = 0) { |  |
| if( PredMode != MODE_INTRA ) |  |
| no_residual_data_flag | ae(v) |
| residualDataPresentFlag = !no_residual_data_flag |  |
| } else { |  |
| residualDataPresentFlag = TRUE |  |
| } |  |
| if( residualDataPresentFlag) { |  |
| . . . |  |
| if( split_transform_flag[ x0 ][ y0 ][ trafoDepth ] ) { |  |
| . . . |  |
| } else if( entropy_coding_mode_flag ) { |  |
| if( PredMode = = MODE_INTRA ¦ ¦ trafoDepth != 0 ¦ ¦ cbf_cb[ x0 ][ y0 ][ trafoDepth ] ¦ ¦ cbf_cr[ x0 ][ y0 ][ trafoDepth ] ) { |  |
| readCbf = true |  |
| if( blkIdx = = 3 && log2TrafoSize < Log2MaxTrafoSize && PredMode != MODE_INTRA ) |  |
| readCbf = cbf_luma[ xBase ][ yBase ][ trafoDepth ] ¦ ¦ cbf_luma[ xBase + ( 1 << log2TrafoWidth ) ][ yBase ][ trafoDepth ] ¦ ¦ cbf_luma[ xBase ][ yBase + ( 1 << log2TrafoHeight ) ][ trafoDepth ] ¦ ¦ cbf_cb[ xBase ][ yBase ][ trafoDepth – 1] ¦ ¦ cbf_cr[ xBase ][ yBase ][ trafoDepth – 1] |  |
| if ( !readCbf ) |  |
| cbf_luma[ x0 ][ y0 ][ trafoDepth ] = 1 |  |
| else |  |
| cbf_luma[ x0 ][ y0 ][ trafoDepth ] | ae(v) |
| } |  |
| if( PredMode = = MODE_INTRA ) |  |
| if( log2TrafoSize > Log2MinTrafoSize ) { |  |
| cbf_cb[ x0 ][ y0 ][ trafoDepth ] | ae(v) |
| cbf_cr[ x0 ][ y0 ][ trafoDepth ] | ae(v) |
| } else if( blkIdx = = 0 ) { |  |
| cbf_cb[ x0 ][ y0 ][ trafoDepth − 1 ] | ae(v) |
| cbf_cr[ x0 ][ y0 ][ trafoDepth − 1 ] | ae(v) |
| } |  |
| } |  |
| } |  |
| } |  |

We propose to change the syntax highlighted by yellow line to the following syntax.

| transform_tree( x0, y0, log2TrafoWidth, log2TrafoHeight, trafoDepth, blkIdx ) { | Descriptor |
| --- | --- |
| if( entropy_coding_mode_flag && trafoDepth = = 0 && IntraSplitFlag = = 0) { |  |
| if( PredMode != MODE_INTRA ) |  |
| no_residual_data_flag | ae(v) |
| residualDataPresentFlag = !no_residual_data_flag |  |
| } else { |  |
| residualDataPresentFlag = TRUE |  |
| } |  |
| if( residualDataPresentFlag) { |  |
| . . . |  |
| if( split_transform_flag[ x0 ][ y0 ][ trafoDepth ] ) { |  |
| . . . |  |
| } else if( entropy_coding_mode_flag ) { |  |
| if( PredMode = = MODE_INTRA ¦ ¦ trafoDepth != 0 ¦ ¦ cbf_cb[ x0 ][ y0 ][ trafoDepth ] ¦ ¦ cbf_cr[ x0 ][ y0 ][ trafoDepth ] ) { |  |
| readCbf = true |  |
| if( blkIdx = = 3 && ((log2CUSize <= Log2MaxTrafoSize+1) ¦¦ ( log2TrafoSize < Log2MaxTrafoSize )) && PredMode != MODE_INTRA ) |  |
| readCbf = cbf_luma[ xBase ][ yBase ][ trafoDepth ] ¦ ¦ cbf_luma[ xBase + ( 1 << log2TrafoWidth ) ][ yBase ][ trafoDepth ] ¦ ¦ cbf_luma[ xBase ][ yBase + ( 1 << log2TrafoHeight ) ][ trafoDepth ] ¦ ¦ cbf_cb[ xBase ][ yBase ][ trafoDepth – 1] ¦ ¦ cbf_cr[ xBase ][ yBase ][ trafoDepth – 1] |  |
| if ( !readCbf ) |  |
| cbf_luma[ x0 ][ y0 ][ trafoDepth ] = 1 |  |
| Else |  |
| cbf_luma[ x0 ][ y0 ][ trafoDepth ] | ae(v) |
| } |  |
| if( PredMode = = MODE_INTRA ) |  |
| if( log2TrafoSize > Log2MinTrafoSize ) { |  |
| cbf_cb[ x0 ][ y0 ][ trafoDepth ] | ae(v) |
| cbf_cr[ x0 ][ y0 ][ trafoDepth ] | ae(v) |
| } else if( blkIdx = = 0 ) { |  |
| cbf_cb[ x0 ][ y0 ][ trafoDepth − 1 ] | ae(v) |
| cbf_cr[ x0 ][ y0 ][ trafoDepth − 1 ] | ae(v) |
| } |  |
| } |  |
| } |  |
| } |  |

In the case when the current CU size is smaller or equal to the max TU size multiplied by 2, and when the first 3 cbf_luma and cbf_chroma are all 0, cbf_luma of the fourth TU shall be 1 even if log2TrafoSize is identical to Log2MaxTrafoSize, therefore the condition of “log2TrafoSize  <  Log2MaxTrafoSize” is unnecessary to decide the cbf to be inferred or not. By modifying in this way, there is more chance to infer cbf_luma instead of signaling it explicitly, as cbf_luma can be inferred even if log2TrafoSize is equal to Log2MaxTrafoSize.

For instance, when the max CU size is 64 and the max TU size is 32 as in the common test condition, (log2CUSize <= Log2MaxTrafoSize+1) is always true. Thus in this case, the if statement becomes as follows.

if (blkIdx==3 && PredMode != MODE_INTRA)

# Simulations & Results

The proposed fix has been implemented on top of the HM-4.0 software, and the simulations have been conducted using the common test configurations[2]. Table 1 shows the results of the proposed fix compared with original HM-4.0 anchor.

Table 1. BD bitrate savings of the proposed fix

|  | All Intra HE |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | 0.0% | 0.0% | 0.0% |
| Class B | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% |
| Class E | 0.0% | 0.0% | 0.0% |
| Overall | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 100% |  |  |
|  | Random Access HE |  |  |
|  | Y | U | V |
| Class A | 0.0% | -0.1% | -0.2% |
| Class B | 0.0% | 0.1% | 0.0% |
| Class C | 0.0% | 0.1% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% |
| Class E |  |  |  |
| Overall | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 98% |  |  |
|  | Low delay B HE |  |  |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | -0.1% | -0.1% |
| Class D | 0.0% | 0.1% | 0.1% |
| Class E | 0.1% | 0.0% | 0.8% |
| Overall | 0.0% | 0.0% | 0.1% |
|  | 0.0% | 0.0% | 0.1% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 100% |  |  |

According to the simulations, the proposed fix shows almost the same BD-rate performance on average compared to the original HM-4.0, and local decoded images at the encoder side perfectly matched with decoded images at the decoder.

# Conclusion

In this contribution, WD refinement on inferred cbf process has been proposed. It has been confirmed that the proposed fix shows almost the same coding performance with the original HM-4.0 anchor. We propose this to be adopted to HM-5.

# Reference

- “WD4: Working Draft 4 of High-Efficiency Video Coding”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-E803, 6th Meeting: Torino, IT, July 2011.

- Frank Bossen, “Common test conditions and software reference configurations”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-E900, 6th Meeting: Torino, IT, July 2011.

# Patent rights declaration(s)

Mitsubishi Electric Corporation, University of Science and Technology of China, and Microsoft Corp. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).