| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G 446 |
| --- | --- |

| Title: | Reduction of the number of pixels used in Adaptive Loop Filter |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kazuyuki Miyazawa Kazuo Sugimoto Akira Minezawa Shun-ichi Sekiguchi Tokumichi Murakami Information Technology R&D Center Mitsubishi Electric Corporation 5-1-1, Ofuna, Kamakura, JAPAN | Tel: Email: | +81-467-41-2466 Miyazawa.Kazuyuki@cw.MitsubishiElectric.co.jp |
| Source: | Mitsubishi Electric Corporation |  |  |

_____________________________

# Abstract

This contribution proposes a method for reducing the number of pixels used in Adaptive Loop Filter (ALF). Prior to applying ALF, this proposed technique calculates a score predicting the effectiveness of ALF for each pixel, then, skips all ALF processes (e.g. pixel classification, filter design, filter apply) at the pixels whose scores are less than a threshold. This threshold is adaptively determined for each frame in the encoding process, and is transmitted to a decoder. The proposed method reports 2% ~ 9% encoding time reductions and 1% ~ 2% decoding time reductions, with 0.1% ~ 0.2% BD-rate loss for AI-HE, RA-HE, and LD-HE structures.

# Introduction

In the current WD4 [1], Block-based Adaptation (BA) mode of ALF classifies individual pixels into five categories based on Laplacian activity metric (and subdivides each of them into three more categories based on directional feature). Figure 1 shows how the coding efficiency is affected when filters are not applied to one of five categories. From the figure, it can be said that the effectiveness of ALF is not so significant in lower activity categories such as categories 0 and 1. That is, we can reduce the computational cost of ALF without noticeable performance loss by not applying ALF to such categories. However, such method has the following problems:

- Not applicable to Region-based Adaptation (RA) mode because the activity-based classification is not performed in a decoder.

- As shown in Figure 2, the distribution of the number of pixels in each category differs depending on the sequence and frame. So, for example, if we simply skip ALF processes for categories 0 and 1, almost all pixels are not applied ALF in some sequences such as Kimono (the categories 0 and 1 account for 95% of whole pixels).

In order to address the above problem 1), the proposed technique calculates a score for every pixel prior to all ALF processes, and skips ALF at pixels whose scores are less than a threshold. These score calculation and thresholding are performed in both the encoding and decoding process. The score reflects the activity of texture around the pixel, and can be calculated with low computational cost compared to the Laplacian activity calculation adopted in HM-4.0. Also, the problem 2) can be solved by adaptively changing the threshold for every frame. The threshold is determined in the encoding process based on the percentage of the number of ALF-skipped pixels, and transmitted to a decoder.

Figure 1: The effectiveness of ALF for each category.

Figure 2: The distributions of the number of pixels in each category (Class B, All intra).

# Proposed technique

The proposed technique consists of score calculation and threshold decision. The following explains these two steps.

## Score calculation

First, the proposed method calculates a score for every 4x4 block using 4 pixels as shown in Figure 3. Let a top-left pixel of the 4x4 block be at (i, j) and let the luma value of the pixel be X(i, j). The score S(i, j) is given by

Si,j=absXi,j−m+absXi+2,j−m+absXi,j+2−m+absXi+2,j+2−m,

where m is an average of X(i, j), X(i+2, j), X(i, j+2) and X(i+2, j+2). All 16 pixels in the 4x4 block have the same score S(i, j). This variance-like score reflects the texture smoothness of the block, so the lower score means flat texture, and also means the effectiveness of ALF is insignificant at the block.

Figure 3: Score calculation in 4x4 block.

## Threshold decision (encoder-only)

The threshold which determines ALF-skipped pixels is decided for each frame in the encoding process. The threshold is calculated based on the percentage of the number of ALF-skipped pixels in the current frame. In the proposed method, the percentage can be set in the encoder configuration, so if we want to apply ALF only to a very limited area in order to reduce the encoding/decoding time significantly, we can set large percentage. The threshold is calculated as follows:

- Generate a cumulative histogram of the score S(i, j) for the current frame.

- Find a bin whose value is approximately equal to the specified percentage.

- The index of the bin is employed as a threshold.

- If the threshold exceeds a predetermined maximum threshold, the latter is selected as the threshold.

Step 4 avoids skipping ALF at pixels which have absolute high score (that is, the pixels have not smooth texture). The threshold is signaled to a decoder in PPS (Picture Parameter Set). The proposed method skips all ALF processes at pixels whose scores are less than the threshold. The same score and threshold are used in both BA and RA modes, and are also used in both luma and chroma ALF. A decoder performs only the score calculation, and skips ALF at pixels whose scores are less than the threshold parsed from PPS.

# Experimental results

The proposed technique is implemented on HM-4.0 and evaluated using the High Efficiency (HE) test configurations in JCTVC-F900 [2]. In this experiment, the percentage of the number of ALF-skipped pixels is set to 40%, and the maximum threshold is set to 16. The relative performance of the proposed method against the original HM-4.0 is listed in Table 1. The proposed method can reduce encoding time 2% ~ 9%, and can reduce decoding time 1% ~ 2%, with 0.1% ~ 0.2% BD-rate loss.

Table 1: Relative performance of the proposed method against HM-4.0.

|  | All Intra HE |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | 0.1% | 0.0% | 0.1% |
| Class B | 0.1% | 0.0% | 0.1% |
| Class C | 0.1% | 0.1% | 0.1% |
| Class D | 0.0% | 0.1% | 0.0% |
| Class E | 0.1% | 0.2% | 0.2% |
| Overall | 0.1% | 0.1% | 0.1% |
| Enc Time[%] | 91% |  |  |
| Dec Time[%] | 99% |  |  |

|  | Random Access HE |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | 0.2% | -0.3% | -0.2% |
| Class B | 0.2% | -0.2% | -0.2% |
| Class C | 0.1% | -0.2% | -0.2% |
| Class D | 0.1% | 0.0% | -0.2% |
| Class E |  |  |  |
| Overall | 0.2% | -0.2% | -0.2% |
| Enc Time[%] | 97% |  |  |
| Dec Time[%] | 98% |  |  |

|  | Low delay B HE |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | 0.3% | -0.1% | 0.1% |
| Class C | 0.2% | 0.2% | 0.1% |
| Class D | 0.1% | -0.4% | -0.3% |
| Class E | 0.2% | -0.3% | 1.0% |
| Overall | 0.2% | -0.1% | 0.2% |
| Enc Time[%] | 98% |  |  |
| Dec Time[%] | 98% |  |  |

Table 2 compares computation time measured only for ALF between the proposed method and the original HM-4.0. In this experimental setting (the ratio of ALF-skipped pixels is 40%), the proposed method can reduce the computation time of ALF 24% ~ 27% in the encoding process, and 5% ~ 11% in the decoding process. These time reduction ratios can be controlled by changing the percentage of ALF-skipped pixels as one of the encoder setting parameters. That is, the proposed method can provide a tool for adjusting the trade-off between the computation time and the coding efficiency.

Table 2: Relative computation time of the proposed method against HM-4.0 (ALF-only).

|  | All Intra HE | Random Access HE | Low delay B HE |
| --- | --- | --- | --- |
| ALF Time in Enc[%] | 76% | 73% | 75% |
| ALF Time in Dec[%] | 95% | 90% | 89% |

# Conclusion

In this contribution, a technique for reducing the number of pixels used in Adaptive Loop Filter is proposed. The proposed method selectively applies ALF to pixels whose qualities are considered to be significantly improved by ALF, and skips ALF at other pixels. The ratio of the number of ALF-skipped pixels can be set as one of the encoder parameters, and this enables us to control the trade-off between the encoding/decoding time and the coding efficiency. In comparison with the JCTVC-F900 anchor, the proposed method can reduce encoding time 2% ~ 9% (24% ~ 27% reduction only in ALF), and can reduce decoding time 1% ~ 2% (5% ~ 11% reduction only in ALF), with 0.1% ~ 0.2% BD-rate loss. With the significant computation time reduction and the minor performance loss, it is recommended to adopt the proposed method.

# Reference

- B. Bross, W.-J. Han, J.-R. Ohm, G. J. Sullivan, and T. Wiegand, “WD4: Working Draft 4 of High-Efficiency Video Coding,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-F803, 6th Meeting: Torino, IT, July 2011.

- Frank Bossen, “Common test conditions and software reference configurations”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-F900, 6th Meeting: Torino, IT, July, 2011.

# Patent rights declaration(s)

Mitsubishi Electric Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).