| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G 446 |
| --- | --- |

| Title: | Reduction of the number of pixels used in Adaptive Loop Filter |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kazuyuki Miyazawa Kazuo Sugimoto Akira Minezawa Shun-ichi Sekiguchi Tokumichi Murakami Information Technology R&D Center Mitsubishi Electric Corporation 5-1-1, Ofuna, Kamakura, JAPAN | Tel: Email: | +81-467-41-2466 Miyazawa.Kazuyuki@cw.MitsubishiElectric.co.jp |
| Source: | Mitsubishi Electric Corporation |  |  |

_____________________________

# Abstract

This contribution proposes a method for reducing the number of pixels used in Adaptive Loop Filter (ALF). Prior to applying ALF, the proposed technique calculates a score predicting the effectiveness of ALF for each pixel, then, skips all ALF processes (e.g. pixel classification, filter design, filter apply) for the pixels whose scores are less than a threshold. This threshold is adaptively determined for each frame in the encoding process, and is transmitted to a decoder. The simulation results report that the proposed method achieves 2% ~ 9% encoding time reductions and 1% ~ 2% decoding time reductions, with 0.1% ~ 0.2% BD-rate loss for AI-HE, RA-HE, and LD-HE structures.

# Introduction

In the current WD4 [1], Block-based Adaptation (BA) mode of ALF classifies individual pixels into five categories based on Laplacian activity metric (and subdivides each of them into three more categories based on directional feature). Figure 1 shows how the coding efficiency is affected when the filters are not applied to one of the five categories. From the figure, it can be said that the effectiveness of ALF is not so significant in the lower activity categories such as categories 0 and 1. That is, we can reduce the computational cost of ALF without noticeable performance loss by not applying ALF to such categories. However, this method has the following problems:

- Not applicable to Region-based Adaptation (RA) mode because the activity-based classification is not performed in a decoder.

- As shown in Figure 2, the distribution of the number of pixels in each category differs depending on the sequence and frame. So, for example, if we simply skip ALF for categories 0 and 1, ALF is not applied to the most part of the picture in some sequences such as Kimono (the categories 0 and 1 account for 95% of whole pixels).

In order to address the above problem 1), the proposed technique calculates a score for each pixel prior to all ALF processes, and skips ALF for the pixels whose scores are less than a threshold. These score calculation and thresholding are performed in both the encoding and decoding process. The score reflects the activity of local texture around the pixel, and can be calculated with low computational cost compared to the Laplacian activity calculation adopted in HM-4.0. Also, the problem 2) can be solved by adaptively changing the threshold for each frame. The threshold is determined in the encoding process based on the percentage of the number of ALF-skipped pixels, and is transmitted to a decoder.

Figure 1: Performance degradation when the filters are not applied to one of the five activity-based categories (BA-only).

Figure 2: Distributions of the percentage of the number of pixels in each category (Class B, All intra).

# Proposed technique

The proposed technique consists of score calculation and threshold decision. The following explains these two steps.

## Score calculation

First, the proposed method calculates a score for each 4x4 block using 4 pixels as shown in Figure 3. Let a top-left pixel of the 4x4 block be at (i, j) and let the luma value of the pixel be X(i, j). The score S(i, j) is given by

Si,j=absXi,j−m+absXi+2,j−m+absXi,j+2−m+absXi+2,j+2−m,

where m is an average of X(i, j), X(i+2, j), X(i, j+2) and X(i+2, j+2). All 16 pixels in the 4x4 block have the same score S(i, j). This variance-like score reflects the texture smoothness of the block, so the lower score means flat texture, and also means the effectiveness of ALF is insignificant at the block.

Figure 3: Score calculation in 4x4 block.

## Threshold decision (encoder-only)

The threshold which decides the ALF-skipped pixels is determined for each frame in the encoding process. The threshold is calculated based on the percentage of the number of ALF-skipped pixels in the current frame. In the proposed method, the percentage can be set in the encoder configuration, so if we want to apply ALF only to a very limited area in order to reduce the encoding/decoding time significantly, we can set large percentage. The threshold is calculated as follows:

- Generate a cumulative histogram of the score S(i, j) for the current frame.

- Find a bin whose value is approximately equal to the specified percentage.

- The index of the bin is employed as a threshold.

- If the threshold exceeds a predetermined maximum threshold, the latter is selected as the threshold.

Step 4 avoids skipping ALF for the pixels which have absolute high score (that is, the pixels have not smooth texture). The threshold is signaled to a decoder in PPS (Picture Parameter Set). The proposed method skips all ALF processes for the pixels whose scores are less than the threshold. The same scores and threshold are used in both BA and RA modes, and are also used in both luma and chroma ALF. A decoder performs only the score calculation, and skips ALF for the pixels whose scores are less than the threshold parsed from PPS.

# Experimental results

The proposed technique is implemented on HM-4.0 and evaluated using the High Efficiency (HE) test configurations in JCTVC-F900 [2]. In this experiment, the percentage of the number of ALF-skipped pixels is set to 40%, and the maximum threshold is set to 16. The relative performance of the proposed method against the original HM-4.0 is listed in Table 1. The proposed method can reduce encoding time 2% ~ 9%, and can reduce decoding time 1% ~ 2%, with 0.1% ~ 0.2% BD-rate loss.

Table 1: Relative performance of the proposed method against HM-4.0.

|  | All Intra HE |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | 0.1% | 0.0% | 0.1% |
| Class B | 0.1% | 0.0% | 0.1% |
| Class C | 0.1% | 0.1% | 0.1% |
| Class D | 0.0% | 0.1% | 0.0% |
| Class E | 0.1% | 0.2% | 0.2% |
| Overall | 0.1% | 0.1% | 0.1% |
| Enc Time[%] | 91% |  |  |
| Dec Time[%] | 99% |  |  |

|  | Random Access HE |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | 0.2% | -0.3% | -0.2% |
| Class B | 0.2% | -0.2% | -0.2% |
| Class C | 0.1% | -0.2% | -0.2% |
| Class D | 0.1% | 0.0% | -0.2% |
| Class E |  |  |  |
| Overall | 0.2% | -0.2% | -0.2% |
| Enc Time[%] | 97% |  |  |
| Dec Time[%] | 98% |  |  |

|  | Low delay B HE |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | 0.3% | -0.1% | 0.1% |
| Class C | 0.2% | 0.2% | 0.1% |
| Class D | 0.1% | -0.4% | -0.3% |
| Class E | 0.2% | -0.3% | 1.0% |
| Overall | 0.2% | -0.1% | 0.2% |
| Enc Time[%] | 98% |  |  |
| Dec Time[%] | 98% |  |  |

Table 2 compares computation time measured only for ALF between the proposed method and the original HM-4.0. In this experimental setting (the ratio of ALF-skipped pixels is set to 40%), the proposed method can reduce the computation time of ALF 24% ~ 27% in the encoding process, and 5% ~ 11% in the decoding process. These time reduction ratios can be controlled by changing the percentage of ALF-skipped pixels as one of the encoder setting parameters. That is, the proposed method can provide a tool for adjusting the trade-off between the computation time and the coding efficiency.

Table 2: Relative computation time of the proposed method against HM-4.0 (ALF-only).

|  | All Intra HE | Random Access HE | Low delay B HE |
| --- | --- | --- | --- |
| ALF Time in Enc[%] | 76% | 73% | 75% |
| ALF Time in Dec[%] | 95% | 90% | 89% |

# Conclusion

In this contribution, a technique for reducing the number of pixels used in Adaptive Loop Filter is proposed. The proposed method selectively applies ALF to the regions whose qualities are considered to be significantly improved by ALF, and skips ALF for other regions. The ratio of the number of ALF-skipped pixels can be set as one of the encoder parameters, and this enables us to control the trade-off between the complexity and coding efficiency. In comparison with the JCTVC-F900 anchor, the proposed method can reduce encoding time 2% ~ 9% (24% ~ 27% reduction only in ALF), and can reduce decoding time 1% ~ 2% (5% ~ 11% reduction only in ALF), with 0.1% ~ 0.2% BD-rate loss. With the significant computation time reduction and the minor performance loss, it is recommended to adopt the proposed method.

# Reference

- B. Bross, W.-J. Han, J.-R. Ohm, G. J. Sullivan, and T. Wiegand, “WD4: Working Draft 4 of High-Efficiency Video Coding,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-F803, 6th Meeting: Torino, IT, July 2011.

- Frank Bossen, “Common test conditions and software reference configurations”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-F900, 6th Meeting: Torino, IT, July, 2011.

# Patent rights declaration(s)

Mitsubishi Electric Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Appendix: Normative descriptions to be reflected to WD text

A description of the proposal is shown as follows.

- 7.3.3.5	Adaptive loop filter parameter syntax

| alf_param() { | Descriptor |
| --- | --- |
| adaptive_loop_filter_flag | u(1) |
| if ( adaptive_loop_filter_flag ) { |  |
| alf_region_adaptation_flag | u(1) |
| alf_length_luma_minus_5_div2 | ue(v) |
| alf_no_filters_minus1 | ue(v) |
| if (alf_no_filters_minus1 == 1) |  |
| alf_start_second_filter | ue(v) |
| else if (alf_no_filters_minus1 > 1) { |  |
| for (i=1; i< 16; i++) |  |
| alf_filter_pattern[i] | u(1) |
| } |  |
| if (AlfNumFilters > 1) |  |
| alf_pred_method | u(1) |
| alf_min_kstart_minus1 | ue(v) |
| for (i=0; i < AlfMaxDepth; i++) |  |
| alf_golomb_index_bit[i] | u(1) |
| for (i=0; i< AlfNumFilters; i++) |  |
| for (j=0; j< AlfCodedLengthLuma; j++) |  |
| alf_coeff_luma[i][j] | ge(v) |
| alf_skip_threshold | ue(v) |
| alf_chroma_idc | ue(v) |
| if ( alf_chroma_idc ) { |  |
| alf_length_chroma_minus_5_div2 | ue(v) |
| for( i = 0; i< AlfCodedLengthChroma; i++ ) |  |
| alf_coeff_chroma[i] | se(v) |
| } |  |
| } |  |
| } |  |

- 7.4.3.5	Adaptive loop filter parameter semantics

adaptive_loop_filter_flag specifies whether adaptive loop filter applies or not for the current slice.

alf_region_adaptation_flag specifies the filter adaptation method shall be applied for the current slice. If alf_region_adaptation_flag is equal to 1, the region-based filter adaptation shall be applied, otherwise, the block-based directional-activity filter adaptation shall be applied.

alf_length_luma_minus5_div2 specifies the filter length in horizontal direction for the luma component alfTap used in the adaptive loop filter process as specified by

alfTap = ( alf_length_luma_minus5_div2 << 1 ) + 5		(7-1)

The number of coded luma filter coefficients AlfCodedLengthLuma and the number of luma filter coefficients AlfLengthLuma are derived as follows:

- If alfTap is equal to 9,  [Ed.: (WJ) truncated diamond shape]

AlfCodedLengthLuma = ( ( alfTap * alfTap ) >> 2 ) + 1	(7-2)

AlfLengthLuma = ( ( alfTap * alfTap ) >> 1 )		(7-3)

- Otherwise (alfTap is less than 9), [Ed.: (WJ) diamond shape]

AlfCodedLengthLuma = ( ( alfTap * alfTap ) >> 2 ) + 2	(7-4)

AlfLengthLuma = ( ( alfTap * alfTap ) >> 1 ) + 2		(7-5)

alf_no_filters_minus1 plus 1 specifies the number of filter sets for the current slice.

alf_start_second_filter specifies the variance index of luma samples where the second filter is applied, when alf_no_filters_minus1 is equal to 1.

alf_filter_pattern[ i ] specifies the filter index array corresponding to i-th variance index of luma samples, when alf_no_filters_minus1 is greater than 1. The number of filter sets AlfNumFilters is derived as follows:

- If alf_no_filters_minus1 is less than 2,

AlfNumFilters = alf_no_filters_minus1 + 1		(7-6)

- Otherwise (alf_no_filters_minus1 is greater than 2)

AlfNumFilters = i alf_filter_pattern[ i ] with i = 0..15	(7-7)

alf_pred_method specifies whether the filter coefficients are coded in a predictive way.

alf_min_kstart_minus1 plus 1 specifies the minimum order k of k-th order exponential golomb code for the luma filter coefficients for the adaptive loop filter.

alf_golomb_index_bit specifies the difference in order k of k-th order exponential golomb code for the different groups of the luma filter coefficients. Note that there are several groups of the luma filter coefficients where each group may have different order k.

alf_coeff_luma[ i ][ j ] specifies the j-th filter coefficient of i-th filter for the adaptive loop filtering process for luma samples.

alf_skip_threshold specifies the threshold for deciding whether ALF is skipped or not for each sample.

alf_chroma_idc specifies which chroma components are to be filtered.

Table 7-1 – Specification of alf_chroma_idc

| - alf_chroma_idc | - chroma component to be filtered |
| --- | --- |
| - 0 | - None |
| - 1 | - Cr |
| - 2 | - Cb |
| - 3 | - Cb and Cr |

alf_length_chroma_minus5_div2 specifies the filter length in horizontal direction for the luma component alfTapC used in the adaptive loop filter process as specified by

alfTapC = ( alf_length_chroma_minus5_div2 << 1 ) + 5	(7-8)

The number of coded chroma filter coefficients AlfCodedLengthChroma and the number of chroma filter coefficients AlfLengthChroma are derived as follows:

AlfCodedLengthChroma = ( ( alfTapC * alfTapC – 1 ) >> 1 ) + 2	(7-9)

AlfLengthChroma = ( alfTapC * alfTapC ) + 1		(7-10)

alf_coeff_chroma[ i ] specifies the i-th filter coefficient for the adaptive loop filtering process for chroma samples.

8.6.3	Adaptive loop filter process

8.6.3.1	Boundary padding process

8.6.3.2	Derivation process for filter coefficients

8.6.3.3	Derivation process for score array

8.6.3.3	Derivation process for filter index array for luma samples

8.6.3.4	Derivation process for filter index array for luma samples

8.6.3.4	Filtering process for luma samples

8.6.3.5	Filtering process for luma samples

8.6.3.5	Filtering process for chroma samples

8.6.3.6	Filtering process for chroma samples

#### 8.6.3.3 Derivation process for score array

Inputs of this process are:

–	a luma location ( xC, yC ) specifying the top-left luma sample of the current coding unit relative to the top left luma sample of the current picture,

–	a variable log2CUSize specifying the size of the current coding unit.

Output of this process is the two-dimensional score array of (nS >> 2)x(nS >> 2), scArr.

A variable nS is set equal to ( 1 << log2CUSize ).

The boundary padding process specified in subclause Error: Reference source not found is invoked with the luma location ( xC, yC ), the size of coding unit log2CUSize and the chroma component index cIdx set equal to 0, and the output is assigned to the luma sample array s’’. [Ed. (WJ): s’’ is now a picture-size array, but actually CU size + appropriate margin is enough]

The score array scArr is specified in the follows:

- The variable mean[ x, y ] with x, y = 0..((nS)-1) >> 2 is derived as

mean[ x ][ y ] = ij s’’[ xC+(x << 2)+i , xC+(y << 2)+j ] with i, j = 0, 2	(8-11)

- The score array scArr[ x, y ] with x, y = 0..((nS)-1) >> 2 is derived as

scArr[ x ][ y ] = ij | s’’[ xC+(x << 2)+i , xC+(y << 2)+j ] – mean[ x ][ y ] | with i, j = 0, 2	(8-12)

#### 8.6.3.4 Derivation process for filter index array for luma samples

Inputs of this process are:

–	a luma location ( xC, yC ) specifying the top-left luma sample of the current coding unit relative to the top left luma sample of the current picture,

–	a variable log2CUSize specifying the size of the current coding unit.

–	a score array of (nS >> 2)x(nS >> 2), scArr.

Output of this process is the two-dimensional filter index array of (nS)x(nS), fIdx.

A variable nS is set equal to ( 1 << log2CUSize ).

The boundary padding process specified in subclause Error: Reference source not found is invoked with the luma location ( xC, yC ), the size of coding unit log2CUSize and the chroma component index cIdx set equal to 0, and the output is assigned to the luma sample array s’’. [Ed. (WJ): s’’ is now a picture-size array, but actually CU size + appropriate margin is enough]

The filter index array fIdx is specified in the follows:

- When alf_region_adaptation_flag is equal to 1, the following ordered steps apply.

- The variables xIdx and yIdx are derived as

regionTab[16] = { 0, 1, 4, 5, 15, 2, 3, 6, 14, 11, 10, 7, 13, 12, 9, 8 }	(8-13)
	offset = 1 << (Log2MaxCUSize – 1) 		(8-14)
	xInterval = ( ( ( PicWidthInSamplesL + offset ) >> Log2MaxCUSize ) + 1 ) >> 2	(8-15)
	yInterval = ( ( ( PicHeightInSamplesL + offset ) >> Log2MaxCUSize ) + 1 ) >> 2	(8-16)
	xIdx = Min( 3, Floor( ( xC + x ) / ( xInterval << Log2MaxCUSize ) ) )	(8-17)
	yIdx = Min( 3, Floor( ( yC + y ) / ( yInterval << Log2MaxCUSize ) ) )	(8-18)

- The filter index fIdx[ x, y ] with x, y = 0..(nS)-1 is derived as

fIdx[ x ][ y ] = regionTab[ ( yIdx << 2 ) + xIdx ]		(8-19)

- Otherwise (alf_region_flag is equal to 0), the following ordered steps apply.

- The variables varTempH[ x ][ y ], varTempV[ x ][ y ] and varTemp1[ x ][ y ] with x, y = 0, 2, 4,...(nS) is derived as

- If scArr[ x >> 2 ][ y >> 2 ] is greater than alf_skip_threshold,

varTempH[ x ][ y ] = | ( s’’[ xC+x, yC+y ] << 1 ) – s’’[ xC+x-1, yC+y ] – s’’[ xC+x+1, yC+y ] |	(8-20)
		varTempV[ x ][ y ] = | ( s’’[ xC+x, yC+y ] << 1 ) – s’’[ xC+x, yC+y-1 ] – s’’[ xC+x, yC+y+1 ] |	(8-21)

- Otherwise, varTempH[ x ][ y ], varTempV[ x ][ y ] and varTemp1[ x ][ y ] remain the default values.

- The variables varTempH1[ x, y ], varTempV1[ x, y ], and varTemp3[ x, y ] with x, y = 0..( (nS) – 1 )>>2 are derived as

- If scArr[ x ][ y ] is greater than alf_skip_threshold,

varTempH1[ x ][ y ] = ij varTempH[ (x << 2 ) + i ][ (y << 2) + j ] with i, j = 0, 2	(8-22)
		varTempV1[ x ][ y ] = ij varTempV[ (x << 2 ) + i ][ (y << 2) + j ] with i, j = 0, 2	(8-23)
		varTemp3[ x ][ y ] = ( varTempH1[ x ][ y ] + varTempV1[ x ][ y ] ) >> 2	(8-24)

- Otherwise, varTempH1[ x, y ], varTempV1[ x, y ], and varTemp3[ x, y ] remain the default values.

- The variable direction is derived as

- If scArr[ x >> 2 ][ y >> 2 ] is greater than alf_skip_threshold,

      - If varTempV1[ x >> 2 ][ y >> 2 ] is greater than varTempH1[ x >> 2 ][ y >> 2 ] << 1,

direction = 1

      - Otherwise, if varTempH1[ x >> 2 ][ y >> 2 ] is greater than varTempV1[ x >> 2 ][ y >> 2 ] << 1,

direction = 2

      - Otherwise,

direction = 0

- Otherwise, direction remains the default value.

- The variable avgvar is derived as

- If scArr[ x >> 2 ][ y >> 2 ] is greater than alf_skip_threshold,

varTab[16] = { 0, 1, 2, 2, 2, 3, 3, 3, 3, 3, 4, 4, 4, 4, 4, 4 }	(8-25)
		avgVar = Clip3( 0, 15, (varTemp3[ x >> 2 ][ y >> 2 ] * 1024 ) >> (3 + BitDepthY) )	(8-26)

- Otherwise, avgVar remains the default value.

- The filter index fIdx[ x, y ] with x, y = 0..(nS)-1 is derived as

- If scArr[ x >> 2 ][ y >> 2 ] is greater than alf_skip_threshold,

fIdx[ x ][ y ] = Clip3( 0, 4, varTab[avgVar] ) + 5 * direction	(8-27)

- Otherwise, fIdx[ x ][ y ] remains the default value.

#### 8.6.3.5 Filtering process for luma samples

Inputs of this process are:

–	a luma location ( xC, yC ) specifying the top-left luma sample of the current coding unit relative to the top left luma sample of the current picture,

–	a variable log2CUSize specifying the size of the current coding unit,

–	a filter index array of (nS)x(nS), fIdx.

–	a score array of (nS >> 2)x(nS >> 2), scArr.

Output of this process is the filtered reconstruction of luma picture.

The boundary padding process specified in subclause Error: Reference source not found is invoked with the luma location ( xC, yC ), the size of coding unit log2CUSize and the chroma component index cIdx set equal to 0, and the output is assigned to the luma sample array s’’. [Ed. (WJ): s’’ is now a picture-size array, but actually CU size + appropriate margin is enough]

A variable nS is set equal to ( 1 << log2CUSize ) and a variable alfTap is set equal to ( alf_length_luma_minus_5_div2 << 1 ) + 5.

Each sample of luma picture recFiltPictureL[ xC + x ][ yC + y ] with x, y = 0..(nS)-1, is derived as follows.

- If scArr[ (xC + x) >> 2 ][ (yC + y) >> 2 ] is greater than alf_skip_threshold,

recFiltPictureL[xC+x][yC+y]=             cL[fIdx[x][y]][N]+i=0N−1s''[xC+x+horPos[i],yC+y+verPos[i]]∗cL[fIdx[x][y]][i]	(8-28)
where N is set equal to AlfNumCoeffLuma-1 and horPos[i] and verPos[i] are specified in Error: Reference source not found and Error: Reference source not found, respectively.

- Otherwise,

recFiltPictureL[xC+x][yC+y]=s''[xC+x,yC+y]					(8-29)

- 8.6.3.6 Filtering process for chroma samples

Inputs of this process are:

–	a chroma location ( xC, yC ) specifying the top-left luma sample of the current coding unit relative to the top left chroma sample of the current picture,

–	a variable log2CUSize specifying the size of the current coding unit.

–	a variable cIdx specifying the chroma component index.

–	a score array of (nS >> 1)x(nS >> 1), scArr.

Output of this process is the filtered reconstruction of chroma picture.

The boundary padding process specified in subclause Error: Reference source not found is invoked with the chroma location ( xC, yC ), the size of coding unit log2CUSize and the chroma component index cIdx, and the output is assigned to the luma sample array s’’. [Ed. (WJ): s’’ is now a picture-size array, but actually CU size + appropriate margin is enough]

A variable nS is set equal to ( 1 << log2CUSize ) and a variable alfTapChroma is set equal to ( alf_length_chroma_minus_5_div2 << 1 ) + 5.

Filtered samples of chroma picture recFiltPicture[ xC + x ][ yC + y ] with x, y = 0..(nS)-1, are derived as follows.

- If scArr[ (xC + x) >> 1 ][ (yC + y) >> 1 ] is greater than alf_skip_threshold,

(8-30)
where

N = AlfNumCoeffChroma – 1,		(8-31)

horPos[ i ] = ( i %  alfTapChroma ) – ( alfTapChroma >> 1 ), and	(8-32)

verPos[ i ] = ( i /  alfTapChroma ) – ( alfTapChroma >> 1 )	(8-33)

- Otherwise,

recFiltPicture[xC+x][yC+y]=s''[xC+x,yC+y]	(8-34)