| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G447 |
| --- | --- |

| Title: | Reduced number of intra 64x64 prediction mode |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kazuo Sugimoto Akira Minezawa Shun-ichi Sekiguchi Information Technology R&D Center Mitsubishi Electric Corporation 5-1-1, Ofuna, Kamakura, JAPAN | Tel: Email: | +81-467-41-2463 Sugimoto.Kazuo@ak.MitsubishiElectric.co.jp |
| Source: | Mitsubishi Electric Corporation |  |  |

_____________________________

# Abstract

This contribution proposes reducing the number of intra 64x64 prediction mode to 1. It is reported that using only DC prediction mode for intra 64x64 gives average BD-rate loss of 0.0% and 0.0% for AI-HE and AI-LC conditions with around 2% and 3% encoding run-time reduction.

# Introduction

There are four modes for intra 64x64 prediction in HM-4, which are vertical, horizontal, DC and Planar prediction modes. As 64x64 block is large, intra prediction usually fails with complex textured images, therefore only one mode for intra 64x64 could be enough. By reducing the number of modes for intra 64x64, encoder does not need to decide the mode which results encoding complexity reduction.

# Proposed method

Proposal 1: Only DC mode for intra 64x64

Proposal 2: Only Planar mode for intra 64x64

We modified HM-4.0 by limiting the number of intra prediction modes to one for 64x64 block. Intra prediction mode is not signaled for 64x64 block as there is only one prediction mode.

# Simulations

We evaluated the performance of limiting the number of intra 64x64 modes to one using common test conditions. Our tests are conducted on Linux OS with the same CPU/Memory equipped PCs.

The test results are shown in Table 1 with DC mode only, and Table 2 with Planar mode only.

Table 1. DC mode only

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.3% | 0.3% | 0.0% | 0.1% | 0.0% |
| Class B | 0.1% | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class E | 0.1% | 0.5% | 0.2% | 0.1% | -0.1% | 0.1% |
| Overall | 0.0% | 0.2% | 0.1% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.2% | 0.1% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 98% |  |  | 97% |  |  |
| Dec Time[%] | 99% |  |  | 100% |  |  |

Table 2. Planar mode only

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.1% | 0.2% | 0.2% | 0.1% | 0.4% | 0.2% |
| Class B | 0.0% | 0.1% | 0.0% | 0.0% | 0.1% | 0.1% |
| Class C | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class E | 0.0% | 0.4% | 0.2% | 0.1% | -0.1% | 0.1% |
| Overall | 0.0% | 0.1% | 0.1% | 0.0% | 0.1% | 0.1% |
|  | 0.0% | 0.1% | 0.1% | 0.0% | 0.1% | 0.1% |
| Enc Time[%] | 98% |  |  | 97% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

# Conclusion

Using only DC prediction mode for intra 64x64 brings almost no affect on coding performance but gives encoder run-time reduction. We propose that only DC prediction mode instead of 4 modes for intra 64x64 shall be supported in the next HEVC test model.

# Patent rights declaration(s)

Mitsubishi Electric Corporation may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

APPENDIX: proposed WD text

prediction_unit( x0, y0, log2PUWidth, log2PUHeight, PartIdx ) {

if( skip_flag[ x0 ][ y0 ] ) {

merge_idx[ x0 ][ y0 ]

} else if( PredMode = = MODE_INTRA ) {

if( PartMode == PART_2Nx2N && log2PUWidth >= Log2IPCMCUSize )

pcm_flag

if( pcm_flag ) {

while ( !byte_aligned( ) )

pcm_alignment_zero_bit

for( i = 0; i < 1 << ( log2CUSize << 1 ); i++ )

pcm_sample_luma[ i ]

for( i = 0; i < ( 1 << ( log2CUSize << 1 ) ) >> 1; i++ )

pcm_sample_chroma[ i ]

} else {

if(log2PUWidth==6){

IntraPredMode[ x0 ][ y0 ] = 2

planar_flag_luma[ x0 ][ y0 ] = 0

} else {

prev_intra_luma_pred_flag[ x0 ][ y0 ]

if( prev_intra_luma_pred_flag[ x0 ][ y0 ] )

if( NumMPMCand > 1 )

mpm_idx[ x0 ][ y0 ]

else

rem_intra_luma_pred_mode[ x0 ][ y0 ]

if( IntraPredMode[ x0 ][ y0 ] == 2 )

planar_flag_luma[ x0 ][ y0 ]

}

intra_chroma_pred_mode[ x0 ][ y0 ]

SignaledAsChromaDC = ( chroma_pred_from_luma_enabled_flag ? intra_chroma_pred_mode[ x0 ][ y0 ] == 3 : intra_chroma_pred_mode[ x0 ][ y0 ] == 2 )

if( IntraPredMode[ x0 ][ y0 ] != 2 && IntraPredMode[ x0 ][ y0 ]!=34 && SignaledAsChromaDC )

planar_flag_chroma[ x0 ][ y0 ]

}

Table 7-1 – Number of bins and number of mode for rem_intra_luma_pred_mode associated to PuSize

| PuSize | IntraLumaModeBins | Number of Modes |
| --- | --- | --- |
| PU_4x4 | 4 | 17 |
| PU_8x8 | 5 | 34 |
| PU_16x16 | 5 | 34 |
| PU_32x32 | 5 | 34 |
| PU_64x64 | 1 | 3 |

Table 8-2 – Specification of intraPredModeNum

| log2TrafoSize | intraPredModeNum |
| --- | --- |
| 2 | 18 |
| 3 | 35 |
| 4 | 35 |
| 5 | 35 |
| 6 | 4 1 |

Table 8-3 specifies the mapping table used for converting the number of intra prediction modes.

Table 8-3 – Specification of mapIntraPredMode3 and mapIntraPredMode9

| value | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| mapIntraPredMode3[ value ] | 0 | 1 | 2 | 2 | 2 | 0 | 2 | 1 | 1 | 2 | 2 | 0 | 0 | 2 | 2 | 1 | 1 |
| mapIntraPredMode9[ value ] | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 |
| value | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 | 32 | 33 |
| mapIntraPredMode3[ value ] | 2 | 2 | 2 | 2 | 0 | 0 | 0 | 2 | 2 | 2 | 2 | 1 | 1 | 1 | 1 | 2 | 2 |
| mapIntraPredMode9[ value ] | 2 | 3 | 4 | 4 | 0 | 0 | 5 | 5 | 6 | 3 | 7 | 7 | 1 | 1 | 8 | 8 | 2 |

Table 8-4 – Specification of intraFilterType[ nS ][ IntraPredMode ] for various prediction unit sizes

| IntraPredMode | intraFilterType for nS = 4 | intraFilterType for nS = 8 | intraFilterType for nS = 16 | intraFilterType for nS = 32 | intraFilterType for nS = 64 |
| --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 | 0 |
| 1 | 0 | 0 | 0 | 0 | 0 |
| 2 | 0 | 0 | 0 | 0 | 0 |
| 3 | 1 | 1 | 1 | 1 | 1 0 |
| 4 | 0 | 1 | 1 | 1 | 1 0 |
| 5 | 0 | 1 | 1 | 1 | 0 |
| 6 | 1 | 1 | 1 | 1 | 0 |
| 7 | 0 | 1 | 1 | 1 | 0 |
| 8 | 0 | 1 | 1 | 1 | 0 |
| 9 | 1 | 1 | 1 | 1 | 0 |
| 10 | 0 | 0 | 1 | 1 | 0 |
| 11 | 0 | 0 | 1 | 1 | 0 |
| 12 | 0 | 0 | 1 | 1 | 0 |
| 13 | 0 | 0 | 1 | 1 | 0 |
| 14 | 0 | 0 | 1 | 1 | 0 |
| 15 | 0 | 0 | 1 | 1 | 0 |
| 16 | 0 | 0 | 1 | 1 | 0 |
| 17 | 0 | 0 | 1 | 1 | 0 |
| 18-33 | 0 | 0 | 0 | 1 | 0 |
| 34 | 0 | 0 | 0 | 0 | 0 |
| 35 | n/a | n/a | n/a | n/a | n/a |