| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G448 |
| --- | --- |

| Title: | Non-CE11: Context reduction for coding transform coefficients |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Shih-Ta Hsiang and Shawmin Lei MediaTek No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 | Tel: Email: | Shawmin Lei +886-3-5670766 ext. 25555 {shih-ta.hsiang, shawmin.lei}@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

This contribution proposes a modified context modeling method to further reduce the number of the contexts used for coding significant_coeff_flag in 4x4 and 8x8 transform blocks. It is achieved by having the coefficients from the highest frequency region share the same context. The two proposed sets of context configurations can reduce the numbers of the contexts used for coding significant_coeff_flag by 18 and 10 or 29.0 % and 16.1 %, respectively, in the 4x4 and 8x8 blocks for each slice type. Experimental results reportedly show Y BD-rate increases 0.03%, 0.04% and 0.06% for HE-AI, HE-RA and HE-LDB, respectively, by using the first set of the context configuration maps, and show Y BD-rate increases 0.01%, 0.01%, and 0.02% for HE-AI, HE-RA and HE-LDB, respectively, by using the 2nd set of the context configuration maps

# Introduction

There had been some efforts [1] in the previous meetings to further reduce the number of the contexts used for coding significant_coeff_flag for 16x16 and 32 x32 transform blocks. However, the current context selection method for coding significant_coeff_flag in 4x4 and 8x8 transform blocks is dependent on the current coefficient position. Figure 1 illustrates the context configuration map for 4x4 and 8x8 transform blocks in HEVC WD 4 0. As we can see, this method needs to employ a large number of contexts, 31 contexts are currently used for 4x4 and 8x8 transform blocks. This number accounts for more than 70 %of total contexts used for coding significant_coeff_flag in HEVC WD 4 0.

Figure 1: The context configuration maps for 4x4 and 8x8 transform blocks in HEVC WD 4.

Figure 2: The proposed set of context configuration maps BR6.

Figure 3: The proposed set of context configuration maps BR4

.

# Proposal

This contribution attempts to reduce the number of the contexts used for coding significant_coeff_flag for 4x4 and 8x8 transform blocks. It is achieved by having the coefficients from the highest frequency region share the same context. Figures 2 and 3 show the two proposed sets of context configuration maps. The red dot is used to indicate the context that is shared by all high-frequency coefficients from the yellow region. Set BR6 can save 18 contexts (9 for Luma) and Set BR 6 can save 10 contexts.

# Experimental Results

The proposed context modeling method has been implemented using HM-4.0 as software base and experimentally evaluated according to the common test conditions defined in JCTVC-F900 [2]. In addition, we further provide the simulation results over a high bitrate range using QP = 12, 17, 22, and 27, the test condition specified by CE11.a [3] on context reduction. The experimental results are provided in the included spread sheet and summarized in Tables 1-4.

Table 1: Summary of the BD-rate results according to the common test conditions for BR6.

|  | All Intra HE |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | 0.00% | -0.07% | -0.02% |
| Class B | 0.04% | 0.04% | 0.02% |
| Class C | 0.04% | 0.05% | 0.06% |
| Class D | 0.03% | 0.01% | 0.08% |
| Class E | 0.01% | -0.03% | 0.06% |
| Overall | 0.03% | 0.01% | 0.04% |
|  | 0.03% | 0.01% | 0.03% |
|  | Random Access HE |  |  |
|  | Y | U | V |
| Class A | 0.02% | -0.09% | -0.14% |
| Class B | 0.02% | 0.20% | -0.03% |
| Class C | 0.05% | 0.09% | 0.04% |
| Class D | 0.07% | 0.13% | -0.07% |
| Class E |  |  |  |
| Overall | 0.04% | 0.09% | -0.05% |
|  | 0.04% | 0.08% | -0.03% |
|  | Low delay B HE |  |  |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | 0.02% | -0.15% | -0.34% |
| Class C | 0.06% | -0.19% | -0.09% |
| Class D | 0.09% | 0.33% | 0.23% |
| Class E | 0.10% | -0.01% | 0.05% |
| Overall | 0.06% | -0.01% | -0.06% |
|  | 0.06% | -0.01% | -0.14% |

Table 2: Summary of the BD-rate results according to the common test conditions, BR4.

|  | All Intra HE |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | 0.00% | -0.03% | 0.01% |
| Class B | 0.02% | 0.01% | -0.01% |
| Class C | 0.01% | 0.05% | -0.03% |
| Class D | 0.01% | 0.00% | -0.01% |
| Class E | 0.00% | 0.01% | 0.00% |
| Overall | 0.01% | 0.01% | -0.01% |
|  | 0.01% | 0.01% | -0.01% |
|  | Random Access HE |  |  |
|  | Y | U | V |
| Class A | 0.01% | -0.17% | 0.02% |
| Class B | -0.03% | 0.10% | -0.04% |
| Class C | 0.02% | 0.09% | -0.06% |
| Class D | 0.04% | 0.07% | 0.01% |
| Class E |  |  |  |
| Overall | 0.01% | 0.03% | -0.02% |
|  | 0.01% | 0.00% | -0.03% |
|  | Low delay B HE |  |  |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | 0.01% | -0.10% | -0.21% |
| Class C | 0.04% | 0.08% | -0.02% |
| Class D | 0.01% | 0.12% | -0.15% |
| Class E | 0.04% | 0.16% | 1.06% |
| Overall | 0.02% | 0.05% | 0.09% |
|  | 0.02% | 0.02% | 0.06% |

Table 3: Summary of the BD-rate results for QP = 12, 17, 22, 27, BR6

|  | All Intra HE |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | 0.02% | 0.02% | 0.01% |
| Class B | 0.05% | 0.06% | 0.07% |
| Class C | 0.06% | 0.11% | 0.12% |
| Class D | 0.07% | 0.15% | 0.11% |
| Class E | 0.01% | -0.04% | -0.02% |
| Overall | 0.05% | 0.06% | 0.06% |
|  | 0.05% | 0.06% | 0.06% |
|  | Random Access HE |  |  |
|  | Y | U | V |
| Class A | 0.01% | -0.02% | -0.11% |
| Class B | 0.02% | 0.06% | 0.05% |
| Class C | 0.07% | 0.14% | 0.12% |
| Class D | 0.14% | 0.24% | 0.02% |
| Class E |  |  |  |
| Overall | 0.06% | 0.10% | 0.02% |
|  | 0.06% | 0.09% | 0.01% |
|  | Low delay B HE |  |  |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | 0.03% | 0.05% | 0.11% |
| Class C | 0.08% | 0.05% | 0.09% |
| Class D | 0.10% | 0.04% | 0.18% |
| Class E | 0.03% | -0.24% | 0.30% |
| Overall | 0.06% | -0.01% | 0.16% |
|  | 0.06% | 0.00% | 0.13% |

Table 4: Summary of the BD-rate results for QP = 12, 17, 22, 27, BR4

|  | All Intra HE |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | 0.00% | 0.00% | 0.01% |
| Class B | 0.05% | 0.03% | 0.01% |
| Class C | 0.01% | 0.02% | 0.03% |
| Class D | 0.00% | -0.01% | -0.02% |
| Class E | 0.01% | 0.00% | 0.01% |
| Overall | 0.02% | 0.01% | 0.01% |
|  | 0.02% | 0.01% | 0.00% |
|  | Random Access HE |  |  |
|  | Y | U | V |
| Class A | 0.01% | 0.05% | 0.04% |
| Class B | 0.03% | 0.00% | -0.06% |
| Class C | 0.03% | 0.03% | 0.04% |
| Class D | 0.02% | 0.15% | -0.07% |
| Class E |  |  |  |
| Overall | 0.02% | 0.05% | -0.02% |
|  | 0.02% | 0.05% | -0.01% |
|  | Low delay B HE |  |  |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | 0.03% | 0.00% | -0.03% |
| Class C | 0.03% | -0.01% | -0.04% |
| Class D | 0.01% | -0.03% | 0.01% |
| Class E | 0.01% | -0.56% | 0.13% |
| Overall | 0.02% | -0.12% | 0.01% |
|  | 0.02% | -0.14% | -0.03% |

# Conclusions

This contribution proposes a modified method for reducing the number of the contexts used for coding significant_coeff_flag in 4x4 and 8x8 transform blocks. The first set of the proposed context configuration can save 18 contexts per slice, or 29.0 % for 4x4 & 8x8 blocks and leads to overall average Y BD-rate increases 0.03 - 0.06% under common test condition and 0.05 – 0.06 % with low QP values. The second set of the proposed context configuration can save 10 contexts per slice, or 16.1 % for 4x4 & 8x8 blocks and leads to overall average Y BD-rate increases 0.01 - 0.02% under common test condition and 0.02 % with low QP values.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- V. Sze, “Reduction in contexts used for significant_coeff_flag and coefficient level,” Document of Joint Collaborative Team on Video Coding, JCTVC-F132, Torino, IT, 14-22 July, 2011.

- F. Bossen, “Common test conditions and software reference configurations,” Document of Joint Collaborative Team on Video Coding, JCTVC-F900, Torino, IT, 14-22 July, 2011.

- “Description of Core Experiment (CE11): Coefficient scanning and coding,” Document of Joint Collaborative Team on Video Coding, JCTVC-F911, Torino, IT, 14-22 July, 2011.