These WD modifications are based on WD4 (JCTVC-F803) for context configuration BR6.

#### 9.3.3.1.1.4 Derivation process of ctxIdxInc for the syntax element significant_coeff_flag

Inputs to this process are the color component index cIdx, the current coefficient scan position ( xC , yC ) and the transform block size log2TrafoSize.

Output of this process is ctxIdxInc.

The variable sigCtx depends on the current position ( xC, yC ), the transform block size and previsously decoded bins of the syntax element significant_coeff_flag. For the derivation of sigCtx, the following applies.

- If log2TrafoSize is less than or equal to 3, sigCtx is derived as follows.

shift  	=  log2TrafoSize  = =  3 ? 1 : 0

xxC 	=  xC  >>  shift;

yyC 	=  yC  >>  shift;

sigCtx  =  ( shift  *  15 )  +  (  xxC + yyC  <=  3 ?  (yyC  <<  2)  +  xxC  :  10)	(9-1)

- Otherwise if xC + yC is less than 2, sigCtx is derived as follows.

sigCtx  =  31  +  ( yC  <<  1 )  +  xC	(9-2)

- Otherwise if xC + yC is less than 5, sigCtx is derived as follows.

temp  =  significant_coeff_flag[ xC + 1 ][ yC ]  +  significant_coeff_flag[ xC + 2 ][ yC ]  +  
		significant_coeff_flag[ xC ][ yC + 1 ]  +  significant_coeff_flag[ xC + 1 ][ yC + 1 ]  +  
		significant_coeff_flag[ xC ][ yC + 2 ]	(9-3)
sigCtx  =  34  +  Min( 4, temp )

- Otherwise (xC  +  yC is greater than 4), sigCtx is derived using previously decoded bins of the syntax element significant_coeff_flag as follows.

- The variable sigCtx is initialized as follows.

sigCtx  =  39		(9-4)

- When xC is less than ( 1 << log2TrafoSize ) − 1, the following applies.

sigCtx  =  sigCtx  +  significant_coeff_flag[ xC + 1 ][ yC ]	(9-5)

- When xC and yC are less than ( 1 << log2TrafoSize ) − 1, the following applies.

sigCtx  =  sigCtx  +  significant_coeff_flag[ xC + 1 ][ yC + 1 ]	(9-6)

- When xC is less than ( 1 << log2TrafoSize ) − 2, the following applies.

sigCtx  =  sigCtx  +  significant_coeff_flag[ xC + 2 ][ yC ]	(9-7)

- When yC is less than ( 1 << log2TrafoSize ) − 1, the following applies.

sigCtx  =  sigCtx  +  significant_coeff_flag[ xC ][ yC + 1 ]	(9-8)

- When yC is less than ( 1 << log2TrafoSize ) − 2 and sigCtx is less than 43, the following applies.

sigCtx  =  sigCtx  +  significant_coeff_flag[ xC ][ yC + 2 ]	(9-9)

The context index increment ctxIdxInc is derived using the color component index cIdx and sigCtx as follows.

- If cIdx is equal to 0, ctxIdxInc is derived as follows.

ctxIdxInc  =  sigCtx		(9-10)

- Otherwise (cIdx is greater than 0), ctxIdxInc is derived as follows.

ctxIdxInc  =  44  +  sigCtx		(9-11)

[Ed. (BB): The context derivation assumes maximum transform sizes less than or equal to 32x32 for luma and 16x16 for chroma and minimum transform sizes greater than or equal to 4x4.]