| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G449 |
| --- | --- |

| Title: | Reduced number of intra chroma prediction mode |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kazuo Sugimoto Akira Minezawa Shun-ichi Sekiguchi Information Technology R&D Center Mitsubishi Electric Corporation 5-1-1, Ofuna, Kamakura, JAPAN | Tel: Email: | +81-467-41-2463 Sugimoto.Kazuo@ak.MitsubishiElectric.co.jp |
| Source: | Mitsubishi Electric Corporation |  |  |

_____________________________

# Abstract

This contribution proposes reducing the number of intra chroma prediction mode from 6 to 5. It is reported that removing DC prediction mode for intra chroma gives average chroma BD-rate loss of 0.1% and 0.1% for AI-HE and AI-LC conditions with around 2% encoding run-time reduction. It is also reported that removing Planar prediction mode for intra chroma gives average BD-rate loss of 0.0% and 0.01% for AI-HE and AI-LC conditions with around 2% encoding run-time reduction.

# Introduction

There are six modes for intra chroma prediction in HM-4, which are listed in the table as follows:

| intra_chroma_pred_mode | IntraPredMode[ xB ][ yB ] |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | X ( 4 <= X < 35 ) |
| 0(Planar) | 7 | 0 | 0 | 0 | 0 |
| 1(VER) | 1 | 7 | 1 | 1 | 1 |
| 2(HOR) | 2 | 2 | 7 | 2 | 2 |
| 3(DC) | 3 | 3 | 3 | 7 | 3 |
| 4(LM) | LM | LM | LM | LM | LM |
| 5(DM) | 0 | 1 | 2 | 3 | X |

When the associated intra luma prediction mode is one of intra chroma prediction modes, the intra chroma prediction mode which is the same mode with luma is replaced by diagonal VER+8 prediction mode. In this contribution, reducing the number of intra chroma prediction to five is proposed.

# Proposed method

Two methods are proposed as follows:

[Method 1]

DC mode is removed from intra chroma prediction, and when the associated intra luma prediction mode is one of intra chroma prediction modes, the intra chroma prediction mode which is the same mode with luma is replaced by DC prediction mode.

| intra_chroma_pred_mode | IntraPredMode[ xB ][ yB ] |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | X ( 4 <= X < 35 ) |
| 0(Planar) | 3 | 0 | 0 | 0 | 0 |
| 1(VER) | 1 | 3 | 1 | 1 | 1 |
| 2(HOR) | 2 | 2 | 3 | 2 | 2 |
| 3(LM) | LM | LM | LM | LM | LM |
| 4(DM) | 0 | 1 | 2 | 3 | X |

[Method 2]

Planar mode is removed from intra chroma prediction, and when the associated intra luma prediction mode is one of intra chroma prediction modes, the intra chroma prediction mode which is the same mode with luma is replaced by Planar prediction mode.

| intra_chroma_pred_mode | IntraPredMode[ xB ][ yB ] |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | X ( 4 <= X < 35 ) |
| 0(DC) | 3 | 3 | 3 | 0 | 3 |
| 1(VER) | 1 | 0 | 1 | 1 | 1 |
| 2(HOR) | 2 | 2 | 0 | 2 | 2 |
| 3(LM) | LM | LM | LM | LM | LM |
| 4(DM) | 0 | 1 | 2 | 3 | X |

# Simulations

We evaluated the performance of the proposed two methods using common test conditions. Our tests are conducted on Linux OS with the same CPU/Memory equipped PCs.

The test results of method 1 and 2 are shown in Table 1 and 2 respectively.

Table 1. method 1 (removing DC prediction mode)

Table 2. method 2 (removing Planar prediction mode)

# Conclusion

Reducing intra prediction mode for chroma to 5 brings almost no affect on coding performance but gives encoder and decoder run-time reduction. We propose reducing intra prediction mode for chroma to 5 by removing DC prediction mode to be adopted in the next HEVC test model.

# Patent rights declaration(s)

Mitsubishi Electric Corporation may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).