| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G451 |
| --- | --- |

| Title: | Qp offset for intra block |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kazuo Sugimoto Akira Minezawa Shun-ichi Sekiguchi Information Technology R&D Center Mitsubishi Electric Corporation 5-1-1, Ofuna, Kamakura, JAPAN | Tel: Email: | +81-467-41-2463 Sugimoto.Kazuo@ak.MitsubishiElectric.co.jp |
| Source: | Mitsubishi Electric Corporation |  |  |

_____________________________

# Abstract

This contribution proposes to add a syntax signaling a qp offfset value for intra block at the slice header. Syntax and semantics of the proposed offset is provided.

# Introduction

In the current WD, quantization parameter is coded using cu_qp_delta signaled in CU level. One of commonly used techniques to improve visual quality in commercial encoder products is encoding intra blocks by with smaller qp value compared to inter blocks. Insertion of cyclic vertical intra block line is also commonly used in commercial encoder to realize a very low delay codec as depicted in figure 1. In these cases, switching intra and inter occurs frequently, and as a result qp value change occurs frequently during encoding which causes signaling of large overhead of cu_qp_delta. To solve this problem, signaling a qp offset value for intra block is proposed.

Figure 1. qp changes across intra block boundaries

# Proposed method

We propose to signal a qp offset, intra_qp_offset, at the slice header as follows. It is obvious that by signaling intra_qp_offset, cu_qp_delta overhead caused by assigning smaller qp for intra is significantly reduced.

| slice_header( ) { | Descriptor |
| --- | --- |
| lightweight_slice_flag | u(1) |
| if( !lightweight_slice_flag ) { |  |
| slice_type | ue(v) |
| pic_parameter_set_id | ue(v) |
| frame_num | u(v) |
| if( IdrPicFlag ) |  |
| idr_pic_id | ue(v) |
| if( pic_order_cnt_type = = 0 ) |  |
| pic_order_cnt_lsb /* | u(v) |
| if( slice_type = = P ¦ ¦ slice_type = = B ) { |  |
| num_ref_idx_active_override_flag | u(1) |
| if( num_ref_idx_active_override_flag ) { |  |
| num_ref_idx_l0_active_minus1 | ue(v) |
| if( slice_type = = B ) |  |
| num_ref_idx_l1_active_minus1 | ue(v) |
| } |  |
| } |  |
| ref_pic_list_modification( ) |  |
| ref_pic_list_combination( ) |  |
| if( nal_ref_idc != 0 ) |  |
| dec_ref_pic_marking( ) |  |
| } |  |
| if( entropy_coding_mode_flag && slice_type != I) |  |
| cabac_init_idc | ue(v) |
| first_slice_in_pic_flag | u(1) |
| if( first_slice_in_pic_flag == 0 ) |  |
| slice_address | u(v) |
| if( !lightweight_slice_flag ) { |  |
| slice_qp_delta | se(v) |
| intra_qp_offset | se(v) |
| if( sample_adaptive_offset_enabled_flag ) |  |
| sao_param() |  |
| if( deblocking_filter_control_present_flag ) { |  |
| disable_deblocking_filter_idc |  |
| if( disable_deblocking_filter_idc != 1 ) { |  |
| slice_alpha_c0_offset_div2 |  |
| slice_beta_offset_div2 |  |
| } |  |
| } |  |
| if( slice_type = = B ) |  |
| collocated_from_l0_flag | u(1) |
| if( adaptive_loop_filter_enabled_flag ) { |  |
| if( !shared_pps_info_enabled_flag ) |  |
| alf_param( ) |  |
| alf_cu_control_param( ) |  |
| } |  |
| } |  |
| } |  |

intra_qp_offset can change add an offset value to QPY for intra blocks. The decoded value of intra_qp_offset shall be in the range of –( 26+ QpBdOffsetY / 2 ) to +( 25+ QpBdOffsetY / 2 ), inclusive.

cu_qp_delta can change the value of QPY for a quantization group of coding units where the quantization group of coding units is specified as follows:

- If a coding unit with the split_coding_unit_flag[ x0 ][ y0 ] equal to 0 and the log2CUSize is greater than or equal to log2MinCUDQPSize, the quantization group includes this coding unit only.

- Otherwise, if a coding unit with the split_coding_unit_flag[ x0 ][ y0 ] equal to 1 and the log2CUSize is equal to log2MinCUDQPSize, the quantization group includes all coding units split from this coding unit.

The decoded value of cu_qp_delta shall be in the range of –( 26+ QpBdOffsetY / 2 ) to +( 25+ QpBdOffsetY / 2 ), inclusive. cu_qp_delta shall be inferred to be equal to 0 when it is not present for any quantization group of coding units.

The value of QPY is derived as

if (pred_type[ x0 ][ y0 ] == MODE_INTRA)

QPY = ( ( ( QPY,PREV + cu_qp_delta +52+intra_qp_offset+ 2*QpBdOffsetY )%( 52 + QpBdOffsetY ) ) - QpBdOffsetY

otherwise

QPY = ( ( ( QPY,PREV + cu_qp_delta +52+ 2*QpBdOffsetY )%( 52 + QpBdOffsetY ) ) - QpBdOffsetY	(7-1)

where QPY,PREV is the luma quantization parameter, QPY, of the left neighbor quantization group of coding units in the current slice. If the left neighbor quantization group in the current slice is not available, QPY,PREV is the luma quantization parameter, of the previous quantization group in decoding order in the current slice. For the first quantization group of coding units in the slice QPY,PREV is initially set equal to SliceQPY at the start of each slice.

The value of QP’Y is derived as

QP’Y = QPY + QpBdOffsetY

# Conclusion

We propose intra qp offset syntax to be supported in the next HEVC test model.

# Patent rights declaration(s)

Mitsubishi Electric Corporation may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form)