| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G452 |
| --- | --- |

| Title: | SABAC - Scalable-complexity Adaptive Binary Arithmetic Coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kazuo Sugimoto Akira Minezawa Shun-ichi Sekiguchi Information Technology R&D Center Mitsubishi Electric Corporation 5-1-1, Ofuna, Kamakura, JAPAN | Tel: Email: | +81-467-41-2463 Sugimoto.Kazuo@ak.MitsubishiElectric.co.jp |
| Source: | Mitsubishi Electric Corporation |  |  |

_____________________________

# Abstract

This contribution proposes a mechanism to realize scalable complexity entropy coding architecture by signaling flags at slice header to disable some parts of functionalities of CABAC process.

# Introduction

There are two different entropy coding architectures in HM4, which are CABAC and CAVLC. Implementing both entropy coders increases not only program size or area of silicon but also significant amount of verification test points, which finally results cost increase of a product. On the other hand, more than single operating point for entropy coding algorithm would help to support wide range of applications including mobile devices, TV sets, cam coders and professional video codec.

Multiple operating points with small overhead of program size for entropy coding could be achieved through switchable or scalable architecture. In this contribution, we propose a mechanism for entropy coding realizing multiple operating points using a scalable architecture on CABAC/PIPE.

# Proposed method

Error: Reference source not found shows a high-level block diagram of CABAC encoding process, which consists of four processing stages: binarization stage, context modeling stage, binary arithmetic coding stage and probability updating stage. For the context modeling stage, context derivation process typically involves deriving neighboring syntax elements to switch context model for specific bins. Probability updating stage has a feedback loop to update the probability of a bin every time when a bin is coded at the binary arithmetic coding stage.

By removing the aspects of context derivation process and feedback loop of probability update from the CABAC operation brings ability of higher throughput, however, it also brings degradation of coding performance.

Figure 1. Block diagram of CABAC encoding process

Our proposed method prohibits context derivation process and probability updating process when a flag specified as SABAC_fast_mode_flag is signaled. For example, when encoding time becomes critical within a picture or a tile, encoder can prohibit context derivation and probability updates so that encoder can complete the encoding process in time. By signaling flags at the slice header only when encoding time becomes critical, the coding performance loss can be minimized. The normal encoding mode with SABAC_fast_mode_flag disabled does all CABAC stages to realize high coding performance, while SABAC fast mode skips both context modeling stage and probability updating stage to realize low complexity processing.

We propose to add a flag for picture parameter set, and a flag for slice header as follows:

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| pic_parameter_set_id | ue(v) |
| seq_parameter_set_id | ue(v) |
| entropy_coding_mode_flag | u(1) |
| if ( entropy_coding_mode_flag ) |  |
| SABAC_enable_flag | u(1) |
| num_temporal_layer_switching_point_flags | ue(v) |
| for( i = 0; i < num_temporal_layer_switching_point_flags; i++ ) |  |
| temporal_layer_switching_point_flag[ i ] | u(1) |
| num_ref_idx_l0_default_active_minus1 | ue(v) |
| num_ref_idx_l1_default_active_minus1 | ue(v) |
| pic_init_qp_minus26 /* relative to 26 */ | se(v) |
| constrained_intra_pred_flag | u(1) |
| slice_granularity | u(2) |
| shared_pps_info_enabled_flag | u(1) |
| if( shared_pps_info_enabled_flag ) |  |
| if( adaptive_loop_filter_enabled_flag ) |  |
| alf_param( ) |  |
| if( cu_qp_delta_enabled_flag ) |  |
| max_cu_qp_delta_depth | u(4) |
| rbsp_trailing_bits( ) |  |
| } |  |

| slice_header( ) { | Descriptor |
| --- | --- |
| lightweight_slice_flag | u(1) |
| if( !lightweight_slice_flag ) { |  |
| slice_type | ue(v) |
| pic_parameter_set_id | ue(v) |
| frame_num | u(v) |
| if( IdrPicFlag ) |  |
| idr_pic_id | ue(v) |
| if( pic_order_cnt_type = = 0 ) |  |
| pic_order_cnt_lsb /* | u(v) |
| if( slice_type = = P ¦ ¦ slice_type = = B ) { |  |
| num_ref_idx_active_override_flag | u(1) |
| if( num_ref_idx_active_override_flag ) { |  |
| num_ref_idx_l0_active_minus1 | ue(v) |
| if( slice_type = = B ) |  |
| num_ref_idx_l1_active_minus1 | ue(v) |
| } |  |
| } |  |
| ref_pic_list_modification( ) |  |
| ref_pic_list_combination( ) |  |
| if( nal_ref_idc != 0 ) |  |
| dec_ref_pic_marking( ) |  |
| } |  |
| if( entropy_coding_mode_flag && slice_type != I) |  |
| cabac_init_idc | ue(v) |
| if( entropy_coding_mode_flag && SABAC_enable_flag ){ |  |
| SABAC_fast_mode_flag | u(1) |
| } |  |
| first_slice_in_pic_flag | u(1) |
| if( first_slice_in_pic_flag == 0 ) |  |
| slice_address | u(v) |
| if( !lightweight_slice_flag ) { |  |
| slice_qp_delta | se(v) |
| if( sample_adaptive_offset_enabled_flag ) |  |
| sao_param() |  |
| if( deblocking_filter_control_present_flag ) { |  |
| disable_deblocking_filter_idc |  |
| if( disable_deblocking_filter_idc != 1 ) { |  |
| slice_alpha_c0_offset_div2 |  |
| slice_beta_offset_div2 |  |
| } |  |
| } |  |
| if( slice_type = = B ) |  |
| collocated_from_l0_flag | u(1) |
| if( adaptive_loop_filter_enabled_flag ) { |  |
| if( !shared_pps_info_enabled_flag ) |  |
| alf_param( ) |  |
| alf_cu_control_param( ) |  |
| } |  |
| } |  |
| } |  |

- SABAC_enable_flag

  - enabling scalable complexity mechanism for CABAC within a sequence

- SABAC_fast_mode_flag

  - skip context selection stage and probability updating stage when enabled

By using these flags, multiple operating points for entropy coder without additional program size or area of silicon is flexibly realized using slice level switching of CABAC stages.

# Conclusion

Scalable complexity mechanism for CABAC is proposed. Slice level complexity switching realizes flexible multiple operating point for entropy coder by using the proposed method. We propose to adopt this mechanism into HM5/WD5.

# Patent rights declaration(s)

Mitsubishi Electric Corporation may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).