| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G453 |
| --- | --- |

| Title: | New definition for Tile/slice boundary |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kazuo Sugimoto Akira Minezawa Shun-ichi Sekiguchi Information Technology R&D Center Mitsubishi Electric Corporation 5-1-1, Ofuna, Kamakura, JAPAN | Tel: Email: | +81-467-41-2463 Sugimoto.Kazuo@ak.MitsubishiElectric.co.jp |
| Source: | Mitsubishi Electric Corporation |  |  |

_____________________________

# Abstract

This document proposes to change the definition of tile boundary to support parallel processing of non-LCU aligned regions. It is proposed to define that left-top corner of tile shall be left-top corner of an LCU which is not necessarily be adjacent to another LCU boundary. It is described that by defining as proposed, multiple of non-LCU aligned regions in a picture can be processed in parallel, such as sixteen 1920x1080 regions in SHV format.

# Introduction

Tiles mechanism realizes high level parallel processing by partitioning a picture into rectangular regions whose boundaries break prediction mechanism in the same way as slice boundaries[1]. However, in the current working draft, tiles should contain an integer number of LCUs, therefore picture sizes used in the market in common such as WVGA, 720p, 1080p, etc., can not be partitioned equally using tiles. It is a commonly used way to reuse a video coder design for smaller format to realize a video coder for larger format such as reusing HDTV video coder design to realize 4Kx2K or 8Kx4K video coder. In this contribution, we propose to change the definition of tile and/or slice boundary to support parallel processing of non-LCU aligned regions.

# Changing the definition of tile boundary

Figure 1 shows the current tile boundary definition which aligns LCU boundary, where a tile should have integer number of LCUs. In this case, a picture can not be partitioned equally using tiles as depicted.  Figure 2 shows the proposed tile boundary definition where left-top corner of tiles is aligned with left-top corner of LCU while left-top corner of a tile does not need to be at the position of multiple of LCU size position, therefore a picture can be partitioned equally using tiles structure. The right and bottom boundaries of a tile are treated in a manner same as picture boundary.

When a new flag, pixelunit_tile_flag, in the sequence parameter set is enabled, column_width and column_height syntax represent the size of tile in pixel unit.

Figure 1. current tiles boundary definition

Figure 2. Proposed definition of tiles boundary

The proposed syntax and semantics changes are shown below.

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| profile_idc | u(8) |
| reserved_zero_8bits /* equal to 0 */ | u(8) |
| level_idc | u(8) |
| seq_parameter_set_id | ue(v) |
| max_temporal_layers_minus1 | u(3) |
| pic_width_in_luma_samples | u(16) |
| pic_height_in_luma_samples | u(16) |
| bit_depth_luma_minus8 | ue(v) |
| bit_depth_chroma_minus8 | ue(v) |
| pcm_bit_depth_luma_minus1 | u(4) |
| pcm_bit_depth_chroma_minus1 | u(4) |
| log2_max_frame_num_minus4 | ue(v) |
| pic_order_cnt_type | ue(v) |
| if( pic_order_cnt_type = = 0 ) |  |
| log2_max_pic_order_cnt_lsb_minus4 | ue(v) |
| else if( pic_order_cnt_type = = 1 ) { |  |
| delta_pic_order_always_zero_flag | u(1) |
| offset_for_non_ref_pic | se(v) |
| num_ref_frames_in_pic_order_cnt_cycle | ue(v) |
| for( i = 0; i < num_ref_frames_in_pic_order_cnt_cycle; i++ ) |  |
| offset_for_ref_frame[ i ] | se(v) |
| } |  |
| max_num_ref_frames | ue(v) |
| gaps_in_frame_num_value_allowed_flag | u(1) |
| log2_min_coding_block_size_minus3 | ue(v) |
| log2_diff_max_min_coding_block_size | ue(v) |
| log2_min_transform_block_size_minus2 | ue(v) |
| log2_diff_max_min_transform_block_size | ue(v) |
| log2_min_pcm_coding_block_size_minus3 | ue(v) |
| max_transform_hierarchy_depth_inter | ue(v) |
| max_transform_hierarchy_depth_intra | ue(v) |
| chroma_pred_from_luma_enabled_flag | u(1) |
| loop_filter_across_slice_flag | u(1) |
| sample_adaptive_offset_enabled_flag | u(1) |
| adaptive_loop_filter_enabled_flag | u(1) |
| pcm_loop_filter_disable_flag | u(1) |
| cu_qp_delta_enabled_flag | u(1) |
| temporal_id_nesting_flag | u(1) |
| inter_4x4_enabled_flag | u(1) |
| pixelunit_tile_flag |  |
| num_tile_columns_minus1 |  |
| num_tile_rows_minus1 |  |
| if (num_tile_columns_minus1 != 0 ¦¦ num_tile_rows_minus1 != 0 ¦¦ pixelunit_tile_flag) { |  |
| tile_boundary_independence_idc |  |
| uniform_spacing_idc |  |
| if (uniform_spacing_idc != 1) { |  |
| for (i=0; i<num_tile_columns_minus1 ; i++) |  |
| column_width[i] |  |
| for (i=0; i <num_tile_rows_minus1; i++) |  |
| row_height[i] |  |
| } |  |
| } |  |
| rbsp_trailing_bits( ) |  |
| } |  |

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| pic_parameter_set_id | ue(v) |
| seq_parameter_set_id | ue(v) |
| entropy_coding_mode_flag | u(1) |
| num_temporal_layer_switching_point_flags | ue(v) |
| for( i = 0; i < num_temporal_layer_switching_point_flags; i++ ) |  |
| temporal_layer_switching_point_flag[ i ] | u(1) |
| num_ref_idx_l0_default_active_minus1 | ue(v) |
| num_ref_idx_l1_default_active_minus1 | ue(v) |
| pic_init_qp_minus26 /* relative to 26 */ | se(v) |
| constrained_intra_pred_flag | u(1) |
| slice_granularity | u(2) |
| shared_pps_info_enabled_flag | u(1) |
| tile_info_present_flag |  |
| if (tile_info_present_flag == 1) { |  |
| pixelunit_tile_flag |  |
| num_tile_columns_minus1 |  |
| num_tile_rows_minus1 |  |
| if (num_tile_columns_minus1 != 0 ¦¦ num_tile_rows_minus1 != 0 ¦¦ pixelunit_tile_flag) { |  |
| tile_boundary_independence_idc |  |
| uniform_spacing_idc |  |
| if (uniform_spacing_idc != 1) { |  |
| for (i=0; i<num_tile_columns_minus1 ; i++) |  |
| column_width[i] |  |
| for (i=0; i <num_tile_rows_minus1; i++) |  |
| row_height[i] |  |
| } |  |
| } |  |
| } |  |
| if( shared_pps_info_enabled_flag ) |  |
| if( adaptive_loop_filter_enabled_flag ) |  |
| alf_param( ) |  |
| if( cu_qp_delta_enabled_flag ) |  |
| max_cu_qp_delta_depth | u(4) |
| rbsp_trailing_bits( ) |  |
| } |  |

pixelunit_tile_flag  specifies the unit of tile size. When pixelunit_tile_flag is equal to 0, the unit of tile size is one treeblock. When pixelunit_tile_flag is equal to 1, the unit of tile size is one pixel.

column_width[i] specifies the width of the i-th column. When pixelunit_tile_flag is equal to 0, column_width[i] specifies the width of the i-th column in units of treeblocks. The value of column_width[i] shall be limited such that columnBoundary[i+1] is less than or equal to FrameWidthInCU for i in the range of 0 to num_tile_columns_minus1 – 1, inclusive, and num_tile_columns_minus1 greater than 0. When pixelunit_tile_flag is equal to 1, column_width[i] specifies the width of the i-th column in units of pixels. The value of column_width[i] shall be limited such that columnBoundary[i+1] is less than or equal to FrameWidth for i in the range of 0 to num_tile_columns_minus1 – 1, inclusive, and num_tile_columns_minus1 greater than 0.

row_height[i] specifies the height of the i-th row. When pixelunit_tile_flag is equal to 0, row_height[i] specifies the height of the i-th row in units of treeblocks. The value of row_height[i] shall be limited such that rowBoundary [i+1] is less than or equal to FrameHeightInCU for i in the range of 0 to num_tile_rows_minus1 – 1, inclusive, and num_tile_rows_minus1 greater than 0. When pixelunit_tile_flag is equal to 1, row_height[i] specifies the height of the i-th row in units of pixels. The value of row_height[i] shall be limited such that rowBoundary [i+1] is less than or equal to FrameHeight for i in the range of 0 to num_tile_rows_minus1 – 1, inclusive, and num_tile_rows_minus1 greater than 0.

# Conclusion

To support parallel processing of tiles, we propose to change the definition of tile boundary in the proposed way in the next version of HEVC test model.

References:

- A. Fuldseth, M. Horowitz, S. Xu, A. Segall and M. Zhou, “Tiles”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-F335, 6th Meeting: Torino, IT, July 2011.

# Patent rights declaration(s)

Mitsubishi Electric Corporation may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).