| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G454 |
| --- | --- |

| Title: | Paralell processing of ALF and SAO for tiles |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kazuo Sugimoto Akira Minezawa Shun-ichi Sekiguchi Information Technology R&D Center Mitsubishi Electric Corporation 5-1-1, Ofuna, Kamakura, JAPAN | Tel: Email: | +81-467-41-2463 Sugimoto.Kazuo@ak.MitsubishiElectric.co.jp |
| Source: | Mitsubishi Electric Corporation |  |  |

_____________________________

# Abstract

This document proposes a mechanism to support parallel processing of ALF and SAO for tiles using Adaptation Parameter Set. Aps_id for each tile is signaled in PPS so that not only prediction and residual coding process but also ALF and SAO process can be performed in parallel. Syntax and semantics of modified PPS are proposed.

# Introduction

Tiles mechanism realizes high level parallel processing by partitioning a picture into rectangular regions whose boundaries break prediction mechanism in the same way as slice boundaries[1]. However, ALF and SAO parameters are signaled at a picture level, therefore these process could not done in parallel. To fully utilize the parallel processing nature of tile mechanism, ALF and SAO should also be processed in parallel. This contribution proposes a mechanism to realize ALF and SAO in parallel for each tile.

# Proposed method

Adaptation Parameter Set(APS) is a picture layer activated parameter set intended for use with picture-adaptive data[2] which contains SAO and ALF parameters. To realize parallel ALF and SAO processing for each tile, we propose to associate an APS for each tile by signaling aps_id at the PPS. The proposed syntax and semantics are shown below.

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| pic_parameter_set_id | ue(v) |
| seq_parameter_set_id | ue(v) |
| entropy_coding_mode_flag | u(1) |
| num_temporal_layer_switching_point_flags | ue(v) |
| for( i = 0; i < num_temporal_layer_switching_point_flags; i++ ) |  |
| temporal_layer_switching_point_flag[ i ] | u(1) |
| num_ref_idx_l0_default_active_minus1 | ue(v) |
| num_ref_idx_l1_default_active_minus1 | ue(v) |
| pic_init_qp_minus26 /* relative to 26 */ | se(v) |
| constrained_intra_pred_flag | u(1) |
| slice_granularity | u(2) |
| shared_pps_info_enabled_flag | u(1) |
| tile_info_present_flag |  |
| if (tile_info_present_flag == 1) { |  |
| num_tile_columns_minus1 |  |
| num_tile_rows_minus1 |  |
| if (num_tile_columns_minus1 != 0 ¦¦ num_tile_rows_minus1 != 0) { |  |
| tile_boundary_independence_idc |  |
| uniform_spacing_idc |  |
| if (uniform_spacing_idc != 1) { |  |
| for (i=0; i<num_tile_columns_minus1 ; i++) |  |
| column_width[i] |  |
| for (i=0; i <num_tile_rows_minus1; i++) |  |
| row_height[i] |  |
| } |  |
| separate_sao_alf_flag | u(1) |
| if (separate_sao_alf_flag){ |  |
| for (i=0; i<num_tile_columns_minus1+1; i++){ |  |
| for (j=0; j<num_tile_rows_minus1+1; j++){ |  |
| aps_id[i][j] | ue(v) |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |
| if( shared_pps_info_enabled_flag ) |  |
| if( adaptive_loop_filter_enabled_flag ) |  |
| alf_param( ) |  |
| if( cu_qp_delta_enabled_flag ) |  |
| max_cu_qp_delta_depth | u(4) |
| rbsp_trailing_bits( ) |  |
| } |  |

| aps_rbsp( ) { | Descriptor |
| --- | --- |
| aps_id | ue(v) |
| aps_sample_adaptive_offset_flag | u(1) |
| aps_adaptive_loop_filter_flag | u(1) |
| if( aps_sample_adaptive_offset_flag ¦¦ aps_adaptive_loop_filter_flag ) { |  |
| aps_cabac_use_flag | u(1) |
| if( aps_cabac_use_flag ) { |  |
| aps_cabac_init_idc | ue(v) |
| aps_cabac_init_qp_minus26 | se(v) |
| } |  |
| } |  |
| /* Insert non-CABAC stuff above this line */ |  |
| if( aps_adaptive_loop_filter_flag ) { |  |
| alf_data_byte_count /* to enable skipping past data without parsing it */ | u(8) |
| /* byte_align() this byte align to happen between the non-CABAC and CABAC parts of the alf_param() Once there is an all CABAC alf_param(), enable this byte_align() */ |  |
| alf_param( ) |  |
| byte_align() |  |
| } |  |
| /* insert CABAC stuff below this line; make sure its byte-aligned */ |  |
| if( aps_sample_adaptive_offset_flag ) { |  |
| sao_data_byte_count /* to enable skipping past data without parsing it */ | u(8) |
| byte_align () |  |
| sao_param( ) |  |
| /* byte_align() this final byte align unnecessary as being taken care of by rbsp_trailing_bits() */ |  |
| rbsp_trailing_bits( ) |  |
| } |  |

separate_sao_alf_flag specifies whether SAO and ALF parameters are signaled for each tile or not. If separate_sao_alf_flag is equal to 0, only one set of SAO and ALF parameters are signaled per picture. If separate_sao_alf_flag is equal to 1, aps_id for each tile are signaled at the PPS.

aps_id identifies the adaptation parameter set that is referred to in the slice header. If separate_sao_alf_flag is equal to 1, all slice headers in a tile located at i-th column and j-th row position shall refer aps_id[i][j] signaled at the PPS.

# Conclusion

A mechanism to support parallel processing of ALF and SAO for tiles using Adaptation Parameter Set is proposed. Aps_id for each tile is signaled in PPS so that not only prediction and residual coding process but also ALF and SAO process can be performed in parallel. We proposed to adopt this mechanism to HEVC standard.

References:

- A. Fuldseth, M. Horowitz, S. Xu, A. Segall and M. Zhou, “Tiles”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-F335, 6th Meeting: Torino, IT, July 2011.

- S. Wenger, J. Boyce, Y.-W. Huang, C.-Y. Tsai, P. Wu and M. Li, “Adaptation Parameter Set(APS)”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-F747, 6th Meeting: Torino, IT, July 2011.

# Patent rights declaration(s)

Mitsubishi Electric Corporation may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).