| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G455 |
| --- | --- |

| Title: | CE6: results of filtering for LM_mode |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Akira Minezawa Kazuo Sugimoto Shun-ichi Sekiguchi Information Technology R&D Center Mitsubishi Electric Corporation 5-1-1, Ofuna, Kamakura, JAPAN | Tel: Email: | +81-467-41-2463 Minezawa.Akira@ds.MitsubishiElectric.co.jp |
| Source: | Mitsubishi Electric Corporation |  |  |

_____________________________

# Abstract

This contribution reports verification results on CE6.a that evaluates modifications of LM mode for intra chroma prediction. The verification work was performed using the CE6.a software. Reported performance results were obtained by building and running the software on 64bit Linux platform. It is reported that applying {1, 2, 1} filter on top neighboring, left neighboring and inner samples achieves 0.15%, 0.39% and 0.17% BD-rate gain for Y, U and V component for AI-HE configuration, respectively. Reported BD-rate gains for AI-LC configuration are 0.13%, 0.63% and 0.39% for Y, U and V component, respectively.

# Introduction

Performance verification on CE6.a approved at the last Torino meeting [1] was conducted. The verified method is several tool combinations of luma down-sampling filters for LM mode. This work was conducted using modified HM4.0 software.

# Simulations & Results

This contribution reports the simulation result of several combinations by applying proposed filters on three parts: the top neighboring line, the left neighboring line, and inner samples of current PU. The proposed filters to be tested are shown in Table 1. Each test case is denoted as “TiLjIk” (i=0 or 1, j=0, 1, or 2, and k=0, 1, or 2).

Table 1  Candidate down-sample filters for each location

| Top neighbors | Filter T0: 121 Filter T1: 101 |
| --- | --- |
| Left neighbors | Filter L0: 11 Filter L1: 101101 Filter L2: 121121 |
| Internal pixels (Current PU) | Filter I0: 11 Filter I1: 101101 Filter I2: 121121 |

Table 2 shows test conditions used in our experiment, which is the test configuration defined in the CE6 document [1]. We built executables from source code provided by the proponent and ran it on 64bit Linux operating system (more specifically Fedora 10 x86_64). Encoding and decoding time for both the anchor and the test case were measured using “time” command in the Linux platform.

Table 2  Test condition

| Codec software | Implementation of the tool proposed method on HM4.0 software |
| --- | --- |
| Test configuration | E900[2] |
| QP setting | QPI=22,37,32,37 |

The test result is shown in Table 3. According to the simulation results, tool combination of Test #8 achieves the best performance in terms of luma BD-rate gain.

Table 3 Simulation results for All Intra Condition

# Conclusion

This contribution reports verification results on CE6.a that evaluates modifications of LM mode for intra chroma prediction. It is reported that applying {1, 2, 1} filter on top neighboring, left neighboring and inner samples which is verified by Test #8 achieves 0.15%, 0.39% and 0.17% BD-rate gain for Y, U and V component for AI-HE configuration, respectively. Reported BD-rate gains for AI-LC configuration are 0.13%, 0.63% and 0.39% for Y, U and V component, respectively. We propose to adopt the combination of Test #8 to HM-5.

# Reference

- Ali Tabatabai, “CE6: Intra Coding Improvements”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-E906, 6th Meeting: Torino, IT, July 2011.

- Frank Bossen, “Common test conditions and software reference configurations”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-E900, 6th Meeting: Torino, IT, July 2011.

# Patent rights declaration(s)

Mitsubishi Electric Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).