| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G457 |
| --- | --- |

| Title: | CE6: results of intra vertical and horizontal prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Akira Minezawa Kazuo Sugimoto Shun-ichi Sekiguchi Information Technology R&D Center Mitsubishi Electric Corporation 5-1-1, Ofuna, Kamakura, JAPAN | Tel: Email: | +81-467-41-2463 Minezawa.Akira@ds.MitsubishiElectric.co.jp |
| Source: | Mitsubishi Electric Corporation |  |  |

_____________________________

# Abstract

This contribution reports verification results on an improvement technique of filtering for vertical and horizontal predictions proposed by JCTVC-F172. The verification work was performed using the CE6.d software. Reported performance results were obtained by building and running the provided software on 64bit Linux platform. It is reported that the proposed scheme improves the 0.29% and 0.28% in BD-rate performance on average with the same complexity for high efficiency and low complexity intra configuration respectively. Additional verification results on harmonization with JCTVC-F122 are also repoted.

# Introduction

Performance verification on CE6.a approved at the last Torino meeting [1] was conducted. The verified method is a modification of luma down-sampling filters for LM mode. This work was conducted using modified HM4.0 software provided by the proponent.

# Simulations & Results

Table 1 shows test conditions used in our experiment, which is the test configuration defined in the CE6 document [1]. We built executables from source code provided by the proponent and ran it on 64bit Linux operating system (more specifically Fedora 10 x86_64). Encoding and decoding time for both the anchor and the test case were measured using “time” command in the Linux platform.

Table 1  Test condition

| Codec software | Implementation of the tool proposed method on HM4.0 software |
| --- | --- |
| Test configuration | E900[2] |
| QP setting | QPI=22,37,32,37 |

The test result of applying intra vertical and horizontal prediction filter proposed by JCTVC-F172 is shown in Table 2. It is reported that the proposed scheme improves the 0.29% and 0.28% in BD-rate performance on average with the same complexity for high efficiency and low complexity intra configuration respectively.

Table 2 Simulation results of JCTVC-F172 for All Intra Condition

We also verified the performance of the harmonization of JCTVC-F122 and JCTVC-F172 described in Table 3. The results are shown in Table 4. In summary, the simulation results are shown that Test #2 is the best tradeoff between coding efficiency and filtering complexity.

Table 3  Harmonization test configuration

| Test number | intraPredAngle=0 | intraPredAngle=2,5 |
| --- | --- | --- |
| #1 | 3-line filtering | 1-line filtering |
| #2 | 2-line filtering | 1-line filtering |
| #3 | 1-line filtering | 1-line filtering |

Table 4 Simulation results of harmonization scheme for All Intra Condition

# Conclusion

This contribution reports cross-verification results on CE6.d that evaluates an improvement technique of filtering for vertical and horizontal predictions. It is reported that tool combination of Test #2 described in Table 3 achieves the best tradeoff between coding efficiency and filtering complexity. We propose the combination of Test #2 to be adopted to HM-5.

# Reference

- Ali Tabatabai, “CE6: Intra Coding Improvements”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-E906, 6th Meeting: Torino, IT, July 2011.

- Frank Bossen, “Common test conditions and software reference configurations”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-E900, 6th Meeting: Torino, IT, July 2011.

# Patent rights declaration(s)

Mitsubishi Electric Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).