| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G471 |
| --- | --- |

| Title: | CE1.A.2: Crosscheck for Panasonic's parallel context processing for coefficient coding using block-based context updates in JCTVC-G349 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Tzu-Der Chuang, Yu-Wen Huang No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 | Tel: Email: | Yu-Wen Huang +886-3-5670766 ext. 26297 {peter.chuang, yuwen.huang }@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

The purpose of this document is to crosscheck the parallel context processing by using block-based context updates in CABAC coefficient coding proposed by Panasonic in JCTVC-G349. The coefficient coding contexts are not updated till the end of the block. The delay in context updating includes last significant coefficient flag, significant coefficient flag, coefficient absolute level greater than 1 flag, and coefficient absolute level greater than 2 flag. The verification task was done successfully, and the results match exactly those provided by Panasonic.

# Verification

Panasonic provided the HM source codes of their proposed codec described in JCTVC-G349 [1]. Five tests are evaluated. The verification is carried out by setting the following parameters in the provided codes:

1st test: all on

#define PCP_BB_CTX_UPDATE_COEFF_LAST  1
#define PCP_BB_CTX_UPDATE_COEFF_SIG    1
#define PCP_BB_CTX_UPDATE_COEFF_LV1    1
#define PCP_BB_CTX_UPDATE_COEFF_LV2    1

2nd test: last and levels

#define PCP_BB_CTX_UPDATE_COEFF_LAST  1
#define PCP_BB_CTX_UPDATE_COEFF_SIG    0
#define PCP_BB_CTX_UPDATE_COEFF_LV1    1
#define PCP_BB_CTX_UPDATE_COEFF_LV2    1

3rd test: only last

#define PCP_BB_CTX_UPDATE_COEFF_LAST  1
#define PCP_BB_CTX_UPDATE_COEFF_SIG    0
#define PCP_BB_CTX_UPDATE_COEFF_LV1    0
#define PCP_BB_CTX_UPDATE_COEFF_LV2    0

4th test: levels

#define PCP_BB_CTX_UPDATE_COEFF_LAST  0
#define PCP_BB_CTX_UPDATE_COEFF_SIG    0
#define PCP_BB_CTX_UPDATE_COEFF_LV1    1
#define PCP_BB_CTX_UPDATE_COEFF_LV2    1

5th test: significance map

#define PCP_BB_CTX_UPDATE_COEFF_LAST  0
#define PCP_BB_CTX_UPDATE_COEFF_SIG    1
#define PCP_BB_CTX_UPDATE_COEFF_LV1    0
#define PCP_BB_CTX_UPDATE_COEFF_LV2    0

Four configurations (intra HE, random access HE, low delay B HE, low delay P HE) were run for the delay context updating in CABAC coefficient coding with the provided codes. The running of the tests includes encoding, decoding, and collecting the run time of encoding and decoding processes.

# Results

The BD-rates from our experimental results of five tests match exactly those provided by Panasonic. The encoding time and decoding time are roughly the same as those provided by Panasonic. The detailed results are listed in JCTVC-G471_test_1.xls, JCTVC-G471_test_2.xls, JCTVC-G471_test_3.xls, JCTVC-G471_test_4.xls, and JCTVC-G471_test_5.xls.

# Conclusion

The results presented by Panasonic in JCTVC-G349 are confirmed.

# References

- H. Sasai, T. Nishi, “CE1: SubsetA: Parallel context processing for coefficient coding using block-based context updates,” Document of Joint Collaborative Team on Video Coding, JCTVC-G349, November 2011.