| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G472 |
| --- | --- |

| Title: | CE1.A.4: Crosscheck for Samsung's line buffer removal for CU split flag context model in JCTVC-G769 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Tzu-Der Chuang, Yu-Wen Huang No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 | Tel: Email: | Yu-Wen Huang +886-3-5670766 ext. 26297 {peter.chuang, yuwen.huang }@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

The purpose of this document is to crosscheck the context model selection for split_coding_unit_flag coding proposed by Samsung in JCTVC-G769. The context model selection of split_coding_unit_flag only depends on the depth information of the left block and the current block. The line buffer for storing CU depth can be removed. The verification task was done successfully, and the results match exactly those provided by Samsung.

# Verification

Samsung provided the HM source codes of their proposed codec described in JCTVC-G769 [1]. The verification is carried out by setting the following parameters in the provided codes:

#define LEFT_CU_SPLIT_FLAG                1
#define DEPTH_CU_SPLIT_FLAG               1

Three configurations (intra HE, random access HE, low delay B HE) were run for the modified context model selection for split_coding_unit_flag coding with the provided codes. The running of the tests includes encoding, decoding, and collecting the run time of encoding and decoding processes.

# Results

The BD-rates from our experimental results match exactly those provided by Samsung. The encoding time and decoding time are roughly the same as those provided by Samsung. The detailed results are listed in JCTVC-G472.xls.

# Conclusion

The results presented by Samsung in JCTVC-G769 are confirmed.

# References

- T. Lee, J. Chen, J. H. Park, “CE1 subtest A: Line buffer removal for CU split flag context model,” Document of Joint Collaborative Team on Video Coding, JCTVC-G769, November 2011.