| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G477 WG11 Number: m22041 |
| --- | --- |

| Title: | Non-CE8.c.6: Cross-check of MediaTek/Panasonic/Qualcomm’s multi-source SAO and ALF virtual boundary processing with cross9x9 JCTVC-G211 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Tomoo Yamakage Takashi Watanabe | Email: | tomoo.yamakage@toshiba.co.jp |
| Source: | TOSHIBA Corporation |  |  |

_____________________________

# Abstract

Cross-check results of JCTVC-G211 (Non-CE8.c.6: Multi-source SAO and ALF virtual boundary processing with cross9x9) are reported.  JCTVC-G211 is further improvement of CE8.c.4-2 and belongs to multi-source SAO and ALF.  Coding efficiency results are matched except one test point, which would be affected by operating environments.  Source code was reviewed and confirmed the code was consistent with the description in JCTVC-G211.  Since this contribution is an improvement of CE8.c.4-2, it is suggested to include this improved contribution for an informal subjective viewing and consider it for adoption.

# Introduction

This contribution is a report of cross-check results of JCTVC-G211, which is an improvement of a CE8 contribution (CE8.c.4 Tool-2).  JCTVC-G211 is further improvement of CE8.c.4-2 and belongs to multi-source SAO and ALF.

Cross-check is performed under the CE8 test conditions that refer to JCTVC-F900 and JCTVC-F908.  Coding efficiency results are matched e SteamLocomotive, LC-RA, QP37, slight difference).  Relative encoding and decoding time are similar to those of proponent’s.

Source code was reviewed and confirmed that it was consistent with the description in JCTVC-G211.

# Experimental Results

Detailed results in Excel sheet are attached.  The following tables are the summary of experimental results:

Table 1  Experimental results

| Test case | High Efficiency |  |  |  | Low Complexity |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | IO | RA | LDB | LDP | IO | RA | LDB | LDP |
| Class A | 0.0% | -0.1% |  |  | 0.0% | 0.0% |  |  |
| Class B | 0.0% | -0.1% | -0.4% | -0.2% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | -0.2% | -0.4% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class D | -0.2% | -0.7% | -0.9% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class E | -0.1% |  | -0.5% | -0.5% | 0.0% |  | -0.2% | 0.0% |
| Average | 0.0% | -0.2% | -0.5% | -0.2% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc time | 99% | 100% | 100% | 100% | 101% | 101% | 101% | 101% |
| Dec time | 102% | 102% | 103% | 103% | 101% | 103% | 103% | 103% |

There is no mismatch between encoder and both decoders.  Relative encoding and decoding time are similar to those of proponent’s.

# Conclusion

This contribution shows verification results of JCTVC-G211. Coding efficiency results are confirmed.  No mismatch between encoder and decoder is observed.  Relative encoding and decoding time are also confirmed.  Source code was reviewed.  Since this contribution is an improvement of CE8.c.4-2, it is suggested to include this improved contribution for an informal subjective viewing and consider it for adoption.

# Patent rights declaration(s)

TOSHIBA Corporation may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).