| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, 21-30 November, 2011 | Document: JCTVC-G488 |
| --- | --- |

| Title: | Non-CE6a : Cross check on use of chroma phase in LM mode (JCTVC-G245) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Thomas Guionnet (INRIA) Laurent Guillo (IRISA/CNRS), | Tel: Email: | +33-2-99-84-71-00 thomas.guionnet@inria.fr laurent.guillo@inria.fr |
| Source: | INRIA. |  |  |

_____________________________

# Abstract

This document reports the results of the cross check for the use of chroma phase in LM mode, proposed by Canon (JCTVC-G245) Error: Reference source not found. This contribution is intended to be included in the core experiment CE6.a on block based intra prediction.

When using the LM mode, an assumption is made on the phase used for chroma subsampling. Although this assumption is valid regarding the current official JCT-VC sequence set, it may not stand in the general case. Canon proposes to take into account phase parameters in order to ensure optimal chroma prediction and coding efficiency. Therefore, in order to reliably assess their proposition performance, Canon provided an appropriate set of sequences with various chroma phases.

The results provided in this contribution have been generated with this set of sequences instead of the current official JCT-VC sequence set. The anchors have also been generated in this context.

- Code verification

## General remarks

Canon provided INRIA with the source code to be tested.

The source codes have been successfully compiled using Linux (Fedora Core 11 64 bits) with gcc 4.4.4.

Tests were run on a cluster grid with the following characteristics:

- CPU: 2 x Intel(R) Xeon(R) E5640 @ 2.67GHz, 4 cores.

- RAM: 12 Go per core.

No memory leaks were found.

Classically, the contribution is activated thanks to a set of macros in the typedef.h file. We found the code to be concise and easy to read.

We noted that the chroma phase information is not transmitted in the slice header. It is rather hard coded (LM_CXCYSLICEHEADER set to 0). Actually, it accounts for a very few bits and should not impact the results presented in this document.

## Bug correction

In the context of the CE6.a, a bug has been spotted in the LM mode code. A correction is available but not integrated in the HM4.0. As it is recommended to use the correction in the context of CE6.a tests, we applied it on the HM4.0 before generating our anchors. The source code provided by Canon already included the correction.

# Results

Three tests have been conducted, corresponding to 3 different chroma phases [1]. Results summaries are reported below.

Note that classes AP, BP, CP and EP correspond respectively to classes A, B, C and E downsampled by two with appropriate chroma phase handling. Class P contains new contents provided by Canon.

Note also that the time measures are not fully reliable, due to the use conditions of our cluster grid.

## Phase Cx = 0 and Cy = 0

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class AP | 0.1% | 0.0% | 0.2% | 0.1% | 0.1% | 0.1% |
| Class BP | 0.0% | -0.8% | 0,2% | 0,0% | -0,6% | 0,3% |
| Class CP | 0.0% | -0.8% | -0,9% | 0,0% | -0,4% | -0,4% |
| Class EP | 0.0% | -0.1% | 0,0% | 0,0% | 0,1% | 0,4% |
| Class P | 0.0% | -1.0% | -3,0% | 0,0% | -0,8% | -2,6% |
| Overall | 0.0% | -0.6% | -0,7% | 0,0% | -0,4% | -0,4% |
|  | 0.0% | -0.6% | -0,6% | 0,0% | -0,3% | -0,5% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 101% |  |  |

Table 1: All intra test with Cx = 0 and Cy = 0.

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class AP | 0.0% | 0.4% | 0.2% | 0.0% | -0.1% | -0.3% |
| Class BP | 0.0% | -0.8% | 0.3% | 0.0% | -0.7% | 0.4% |
| Class CP | 0.0% | -0.6% | -0.6% | 0.0% | -0.4% | -0.4% |
| Class EP |  |  |  |  |  |  |
| Class P | 0.0% | -1.5% | -3.1% | 0.0% | -1.0% | -3.1% |
| Overall | 0.0% | -0.6% | -0.7% | 0.0% | -0.5% | -0.8% |
|  | 0.0% | -0.7% | -0.7% | 0.0% | -0.5% | -0.8% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 102% |  |  |

Table 2: Random access test with Cx = 0 and Cy = 0.

|  | Low delay B HE |  |  | Low delay B LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class AP |  |  |  |  |  |  |
| Class BP | 0.1% | -0.1% | 0.3% | 0.0% | -0.3% | 0.4% |
| Class CP | 0.0% | -0.2% | -0.5% | 0.0% | -0.2% | 0.0% |
| Class EP | 0.0% | -1.4% | 0.7% | -0.1% | -0.3% | -0.1% |
| Class P | 0.0% | -1.3% | -1.3% | -0.2% | -1.2% | -1.7% |
| Overall | 0.0% | -0.7% | -0.2% | -0.1% | -0.5% | -0.3% |
|  | 0.0% | -0.6% | -0.3% | -0.1% | -0.5% | -0.3% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 101% |  |  | 101% |  |  |

Table 3: Low delay B test with Cx = 0 and Cy = 0.

|  | Low delay P HE |  |  | Low delay P LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class AP |  |  |  |  |  |  |
| Class BP | 0.0% | -0.1% | 0.4% | 0.0% | -0.3% | 0.0% |
| Class CP | -0.1% | 0.0% | -0.3% | 0.0% | -0.3% | -0.6% |
| Class EP | 0.1% | -1.1% | 1.6% | -0.2% | 0.6% | -0.2% |
| Class P | 0.0% | -1.1% | -0.9% | -0.2% | -0.9% | -1.6% |
| Overall | 0.0% | -0.5% | 0.1% | -0.1% | -0.3% | -0.6% |
|  | 0.0% | -0.5% | 0.0% | -0.1% | -0.2% | -0.5% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 101% |  |  | 103% |  |  |

Table 4: Low delay P test with Cx = 0 and Cy = 0.

## Phase Cx = 1 and Cy = 0

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class AP | -0.1% | -1.4% | -0.2% | 0.0% | -1.3% | -0.3% |
| Class BP | -0.1% | -1.9% | 0.2% | -0.1% | -1.8% | 0.2% |
| Class CP | -0.3% | -2.5% | -2.5% | -0.3% | -2.1% | -2.1% |
| Class EP | 0.0% | 0.0% | -0.5% | 0.0% | 0.3% | -0.1% |
| Class P | -0.1% | -1.9% | -5.6% | -0.1% | -1.8% | -5.4% |
| Overall | -0.1% | -1.6% | -1.7% | -0.1% | -1.5% | -1.5% |
|  | -0.1% | -1.6% | -1.7% | -0.1% | -1.4% | -1.5% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 99% |  |  | 101% |  |  |

Table 5: All intra test with Cx = 1 and Cy = 0.

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class AP | 0.0% | -2.0% | -0.7% | 0.0% | -1.6% | 0.1% |
| Class BP | 0.0% | -1.8% | 0.2% | 0.0% | -1.9% | 0.1% |
| Class CP | -0.1% | -1.9% | -1.8% | -0.2% | -1.9% | -1.8% |
| Class EP |  |  |  |  |  |  |
| Class P | 0.0% | -3.0% | -5.6% | -0.1% | -2.3% | -5.6% |
| Overall | 0.0% | -2.1% | -1.9% | -0.1% | -1.9% | -1.7% |
|  | 0.0% | -2.1% | -1.9% | -0.1% | -1.9% | -1.7% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 101% |  |  | 100% |  |  |

Table 6: Random access test with Cx = 1 and Cy = 0.

|  | Low delay B HE |  |  | Low delay B LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class AP |  |  |  |  |  |  |
| Class BP | 0.0% | -0.4% | -0.4% | 0.0% | -0.9% | -0.3% |
| Class CP | 0.0% | -0.9% | -0.6% | -0.1% | -0.8% | -0.7% |
| Class EP | 0.0% | -1.0% | -1.8% | 0.0% | -0.8% | -0.2% |
| Class P | 0.0% | -1.9% | -2.2% | -0.2% | -0.6% | -2.5% |
| Overall | 0.0% | -1.0% | -1.2% | -0.1% | -0.8% | -0.9% |
|  | 0.0% | -1.1% | -1.1% | -0.1% | -0.8% | -1.0% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 99% |  |  |

Table 7: Low delay B test with Cx = 1 and Cy = 0.

|  | Low delay P HE |  |  | Low delay P LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class AP |  |  |  |  |  |  |
| Class BP | 0.0% | -0.5% | 0.0% | 0.0% | -0.6% | -0.8% |
| Class CP | 0.1% | -0.8% | -0.9% | -0.1% | -1.1% | -0.8% |
| Class EP | -0.1% | -0.8% | -1.5% | 0.0% | -0.2% | 0.1% |
| Class P | -0.2% | -1.7% | -2.6% | -0.2% | -1.3% | -2.8% |
| Overall | 0.0% | -0.9% | -1.1% | -0.1% | -0.8% | -1.1% |
|  | 0.0% | -0.9% | -1.2% | -0.1% | -0.9% | -1.1% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 102% |  |  | 100% |  |  |

Table 8: Low delay P test with Cx = 1 and Cy = 0.

## Phase Cx = 1 and Cy = 1

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class AP | -0.1% | -2.2% | -0.6% | -0.1% | -2.0% | -0.5% |
| Class BP | -0.1% | -1.1% | -0.1% | -0.1% | -1.3% | -0.1% |
| Class CP | -0.3% | -2.2% | -2.0% | -0.3% | -2.1% | -2.0% |
| Class EP | 0.0% | 0.3% | -0.6% | 0.0% | 0.2% | -0.4% |
| Class P | -0.1% | -1.2% | -3.9% | -0.1% | -1.2% | -3.9% |
| Overall | -0.1% | -1.3% | -1.4% | -0.1% | -1.4% | -1.4% |
|  | -0.1% | -1.3% | -1.4% | -0.1% | -1.3% | -1.4% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 102% |  |  | 101% |  |  |

Table 9: All intra test with Cx = 1 and Cy = 1.

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class AP | -0.1% | -2.8% | -0.7% | 0.0% | -2.0% | -1.2% |
| Class BP | 0.0% | -1.1% | 0.1% | -0.1% | -1.0% | -0.1% |
| Class CP | -0.1% | -1.6% | -1.9% | -0.2% | -1.7% | -1.6% |
| Class EP |  |  |  |  |  |  |
| Class P | 0.1% | -1.5% | -3.6% | 0.0% | -1.6% | -3.1% |
| Overall | 0.0% | -1.7% | -1.4% | -0.1% | -1.5% | -1.4% |
|  | -0.1% | -1.6% | -1.4% | -0.1% | -1.5% | -1.4% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 98% |  |  | 99% |  |  |

Table 10: Random access test with Cx = 1 and Cy = 1.

|  | Low delay B HE |  |  | Low delay B LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class AP |  |  |  |  |  |  |
| Class BP | -0.1% | -0.3% | -1.0% | 0.0% | -0.3% | -0.3% |
| Class CP | 0.0% | -0.5% | -0.4% | 0.0% | -0.8% | -0.7% |
| Class EP | 0.0% | 0.5% | -0.2% | 0.0% | -1.0% | -1.2% |
| Class P | 0.1% | 0.1% | -1.7% | 0.0% | -0.6% | -1.7% |
| Overall | 0.0% | -0.1% | -0.9% | 0.0% | -0.7% | -0.9% |
|  | 0.0% | -0.1% | -0.8% | 0.0% | -0.7% | -0.9% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 101% |  |  | 100% |  |  |

Table 11: Low delay B test with Cx = 1 and Cy = 1.

|  | Low delay P HE |  |  | Low delay P LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class AP |  |  |  |  |  |  |
| Class BP | 0.0% | -0.4% | -0.7% | 0.0% | -0.5% | -0.2% |
| Class CP | -0.1% | -0.7% | -0.7% | 0.0% | -0.5% | -0.5% |
| Class EP | 0.0% | 0.8% | 0.6% | 0.0% | -2.6% | -0.2% |
| Class P | -0.1% | 0.0% | -1.6% | 0.0% | -0.9% | -2.2% |
| Overall | -0.1% | -0.1% | -0.7% | 0.0% | -1.0% | -0.8% |
|  | -0.1% | -0.1% | -0.7% | 0.0% | -1.0% | -0.8% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 98% |  |  | 100% |  |  |

Table 12: Low delay P test with Cx = 1 and Cy = 1.

# Conclusion

The Canon proposal, use of chroma phase in LM mode [1], has been cross-checked successfully. No source code issue was reported. We obtained the same results as those provided by Canon.

Overall, a significant gain in the chroma coding efficiency is observed.

# References

- E. François, C. Gisquet, S. Pautet (Canon) « Non-CE6a: Use of chroma phase in LM mode », Doc. JCTVC-G245, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Geneva, Switzerland, November 2011.