| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, 21-30 November, 2011 | Document: JCTVC-G490 WG11 Number: mxxxxx |
| --- | --- |

| Title: | Modified SAO edge offsets |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kenneth Andersson, Per Wennersten, Rickard Sjöberg Färögatan 6 16480 Stockholm | Tel: Email: | +46 10 7141538 kenneth r andersson @ ericsson com |
| Source: | Ericsson AB |  |  |

_____________________________

# Abstract

In this contribution SAO edge offsets in HM4.0 are modified to be more specific to edge characteristics. The result indicates that edges are enhanced both subjectively and objectively. The average BDR gain versus HM4.0 is 0.2% for luma and 1% for chroma for the common conditions [1]. With YUV BDR (6Y+U+V)/8 the average YUV BDR gain is 0.4%. Best results are obtained for the optional condition low delay P at low complexity where average luma BDR is improved by 1.1%. The encoding time is similar to HM4.0. The average decoding time on a single machine is increased by 1.8% compared to HM4.0.

# Introduction

In HM4.0 SAO edge offsets [2][3][4] can be used in one of 4 directions, e.g. horizontal, vertical, diagonal from top left to bottom right, or diagonal from bottom left to top right. The specific direction is determined by sao_type_idx = 1 .. 4.

In HM4.0 SAO edge offsets are categorized by:

edgeIdx = 2+k=01Sign(recPicture[xC+i, yC+j]-recPicture[xC+i+hPos[k], yC+j+vPos[k]]) where recPicture is the picture after deblocking, hPos and vPos are defined below.

Table I Specification of hPos[2] and vPos[2] according to the type of sample adaptive offset process

| sao_type_idx[ cIdx ][ saoDepth ][ rx ][ ry ] | 1 | 2 | 3 | 4 |
| --- | --- | --- | --- | --- |
| hPos[0] | -1 | 0 | -1 | 1 |
| hPos[1] | 1 | 0 | 1 | -1 |
| vPos[0] | 0 | -1 | -1 | 1 |
| vPos[1] | 0 | 1 | 1 | -1 |

The reconstructed picture buffer is modified as:

recSaoPicture[xC+i, yC+j] = recPicture[xC+i, yC+j] + saoValueArray[edgeTable[edgeIdx]] with

i=0..nS-1 and j=0..nS-1, edgeTable[5]={1, 2, 0, 3, 4} and recSaoPicture is the picture after SAO has been applied. saoValueArray contains the edge offsets for the specific direction.

The edges types that HM4.0 SAO edge offsets address are:

edgeIdx = 0

edgeIdx = 1

edgeIdx = 3

edgeIdx = 4

From above one can see that HM4.0 SAO edge offsets do not distinguish between “left” and “right” edges for edgeIdx=1 or edgeIdx=3 and also require that two pixel values are equal to belong to those categories.

# Proposed edge offsets

To better distinguish between different edges the SAO decoding process for edge offsets is modified as follows:

edgeIdx = 19 + Sign(-2*recPicture[xC+i, yC+j]+recPicture[xC+i+hPos[0], yC+j+vPos[0]]+recPicture[xC+i+hPos[1], yC+j+vPos[1]])+

4k=01(3*k+1) Sign(recPicture[xC+i, yC+j]-recPicture[xC+i+hPos[k], yC+j+vPos[k]])

where hPos and vPos are same as for HEVC as defined in Table I. In comparison with HEVC the new categorization requires one additional sign, 3 additional multiplications and 3 additional additions. The multiplications can be implemented with shifts.

The reconstructed picture buffer is modified as:

recSaoPicture[xC+i,yC+j]=recPicture[xC+i,yC+j]+ saoValueArray[edgeTable[saoEoGroupFlag][edgeIdx]] with i=0..nS-1 and j=0..nS-1, edgeTable[2][39]=

{1, 0, 0, 0, 3, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 3, 0, 0, 0, 0, 0, 4, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 4, 0, 0, 0, 2}

{1, 0, 0, 0, 3, 0, 7, 0, 9, 0, 0, 0, 0, 0, 0, 0, 5, 0, 0, 0, 0, 0, 4, 0, 0, 0, 0, 0, 0, 0, 8, 0, 10, 0, 6, 0, 0, 0, 2}}

The proposed categorization switches between 4 edge offsets and 10 individual edge offsets as indicated by saoEoGroupFlag. In comparison with HEVC saoValueArray can contain 6 additional edge offsets but still less than the maximum which is obtained also with HEVC when 16 band offsets are used. The same categorization is used for luma and chroma components. Example of edges that are supported are shown below:

edgeIdx = 0

edgeIdx = 16

edgeIdx = 22

edgeIdx = 38

edgeIdx = 4

edgeIdx = 34

edgeIdx = 32

edgeIdx = 8

edgeIdx = 6

edgeIdx = 30

4 offsets

10 offsets

From the above it can be seen that edgeIdx=32 and edgeIdx=8 have two pixel values closer to each other than a third pixel value. This can address edges that are similar to those that are categorized by edgeIdx=16 and edgeIdx=4. Similarily edgeIdx=6 and edgeIdx=30 can address edges similar to those that are categorized by edgeIdx=22 and edgeIdx=34. .Since edge offsets for the local maxima (edgeIdx=38) typically are close to the negative value of the edge offset for the local minima (edgeIdx=0), this proposal also uses prediction for the edge offset of the local maxima.

Syntax:

| sao_offset_param( rx, ry, saoDepth , cIdx ) { | Descriptor |
| --- | --- |
| if( sao_split_flag[ cIdx ][ saoDepth ][ rx ][ ry ] ) { |  |
| sao_offset_param( 2*rx + 0, 2*ry + 0, saoDepth + 1 , cIdx ) |  |
| sao_offset_param( 2*rx + 1, 2*ry + 0, saoDepth + 1 , cIdx ) |  |
| sao_offset_param( 2*rx + 0, 2*ry + 1, saoDepth + 1 , cIdx ) |  |
| sao_offset_param( 2*rx + 1, 2*ry + 1, saoDepth + 1 , cIdx ) |  |
| } else { |  |
| sao_type_idx[ cIdx ][ saoDepth ][ rx ][ ry ] | ue(v) ¦ ae(v) |
| if( sao_type_idx[ cIdx ][ saoDepth ][ rx ][ ry ] != 0 ){ |  |
| if(sao_type_idx[ cIdx ][ saoDepth ][ rx ][ ry ] <5){ |  |
| sao_eo_group_flag[ cIdx ][ saoDepth ][ rx ][ ry ] | u(1) ¦ ae(v) |
| if(sao_eo_group_flag[ cIdx ][ saoDepth ][ rx ][ ry ]) |  |
| NumSaoClass[ sao_type_idx ] = 10 |  |
| else |  |
| NumSaoClass[ sao_type_idx ] = 4 |  |
| } |  |
| for( i = 0; i < NumSaoClass[ sao_type_idx ]; i++ ) |  |
| sao_offset[ cIdx ][ saoDepth ][ x0 ][ y0 ][ i ] | se(v) ¦ ae(v) |
| } |  |
| } |  |
| } |  |

Semantics:

saoEoGroupFlag is set equal to sao_eo_goup_flag [ cIdx ][ saoDepth ][ rx ][ ry ] which is defined above.

saoValueArray is set equal to SaoOffsetVal[ cIdx ][ saoDepth ][ rx ][ ry ] which is defined below.

An array SaoOffsetVal is specified as

SaoOffsetVal[ cIdx ][ saoDepth ][ rx ][ ry ][ 0 ] = 0

if(sao_type_idx[ cIdx ][ saoDepth ][ rx ][ ry ]  < 5){

SaoOffsetVal[ cIdx ][ saoDepth ][ rx ][ ry ][ i+1] =

sao_offset[ cIdx ][ saoDepth ][ rx ][ ry ][ i ] << ( bitDepth – Min( bitDepth, 10 ) )

with i = 0, 2..NumSaoCategory – 1

SaoOffsetVal[ cIdx ][ saoDepth ][ rx ][ ry ][ i+1] =

(sao_offset[ cIdx ][ saoDepth ][ rx ][ ry ][ 1]- sao_offset[ cIdx ][ saoDepth ][ rx ][ ry ][ 0]) << ( bitDepth – Min( bitDepth, 10 ) )

with i = 1

}else{

SaoOffsetVal[ cIdx ][ saoDepth ][ rx ][ ry ][ i+1 ] =

sao_offset[ cIdx ][ saoDepth ][ rx ][ ry ][ i ] << ( bitDepth – Min( bitDepth, 10 ) )

with i = 0..NumSaoCategory – 1

}

| - sao_type_idx[ cIdx ][ saoDepth ][ rx ][ ry ] | - NumSaoCategory | - Edge type (informative) |
| --- | --- | --- |
| - 0 | - 0 | - Not applied |
| - 1 | - 4/10 | - 1D 0-degree edge |
| - 2 | - 4/10 | - 1D 90-degree edge |
| - 3 | - 4/10 | - 1D 135-degree edge |
| - 4 | - 4/10 | - 1D 45-degree edge |
| - 5 | - 16 | - Central band |
| - 6 | - 16 | - Side band |

# Results

## BDR and encoding/decoding time measurements vs HM4.0

|  | All Intra HE |  |  |  | All Intra LC |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Y | U | V |  | Y | U | V |
| Class A |  | 0,0% | -0,7% | -0,6% |  | 0,0% | -0,8% | -0,9% |
| Class B |  | 0,0% | -0,5% | -0,9% |  | 0,0% | -0,8% | -1,3% |
| Class C |  | 0,0% | -0,3% | -0,3% |  | -0,1% | -0,4% | -0,4% |
| Class D |  | 0,0% | -0,4% | -0,4% |  | 0,0% | -0,4% | -0,5% |
| Class E |  | -0,1% | -0,9% | -0,6% |  | -0,1% | -2,0% | -2,1% |
| Overall |  | 0,0% | -0,5% | -0,6% |  | 0,0% | -0,8% | -1,0% |
|  |  | 0,0% | -0,5% | -0,6% |  | 0,0% | -0,8% | -1,0% |
| Enc Time[%] |  | 123% |  |  |  | 128% |  |  |
| Dec Time[%] |  | 102% |  |  |  | 102% |  |  |
|  |  | Random Access HE |  |  |  | Random Access LC |  |  |
|  |  | Y | U | V |  | Y | U | V |
| Class A |  | 0,0% | -1,3% | -1,1% |  | -0,3% | -1,9% | -1,3% |
| Class B |  | -0,1% | -1,0% | -1,5% |  | -0,3% | -1,7% | -1,7% |
| Class C |  | 0,0% | -0,3% | -0,3% |  | -0,2% | -1,0% | -0,7% |
| Class D |  | 0,0% | -0,3% | -0,5% |  | -0,1% | -0,8% | -1,6% |
| Class E |  |  |  |  |  |  |  |  |
| Overall |  | 0,0% | -0,7% | -0,9% |  | -0,2% | -1,3% | -1,4% |
|  |  | 0,0% | -0,7% | -0,9% |  | -0,2% | -1,3% | -1,4% |
| Enc Time[%] |  | 101% |  |  |  | 132% |  |  |
| Dec Time[%] |  | 102% |  |  |  | 102% |  |  |
|  |  | Low delay B HE |  |  |  | Low delay B LC |  |  |
|  |  | Y | U | V |  | Y | U | V |
| Class A |  |  |  |  |  |  |  |  |
| Class B |  | -0,1% | -1,6% | -1,7% |  | -0,6% | -3,1% | -2,8% |
| Class C |  | -0,1% | -0,4% | -0,8% |  | -0,6% | -2,2% | -2,3% |
| Class D |  | -0,1% | 0,0% | -1,2% |  | -0,2% | -0,7% | -3,5% |
| Class E |  | 0,0% | -0,6% | 1,4% |  | -0,6% | 0,0% | 1,1% |
| Overall |  | -0,1% | -0,7% | -0,8% |  | -0,5% | -1,7% | -2,1% |
|  |  | -0,1% | -0,8% | -0,7% |  | -0,5% | -1,7% | -2,1% |
| Enc Time[%] |  | 100% |  |  |  | 123% |  |  |
| Dec Time[%] |  | 102% |  |  |  | 101% |  |  |
|  |  | Low delay P HE |  |  |  | Low delay P LC |  |  |
|  |  | Y | U | V |  | Y | U | V |
| Class A |  |  |  |  |  |  |  |  |
| Class B |  | -0,1% | -1,4% | -1,3% |  | -1,5% | -2,2% | -2,2% |
| Class C |  | -0,1% | -0,4% | -0,6% |  | -0,6% | -1,5% | -1,7% |
| Class D |  | 0,0% | -0,2% | -1,4% |  | -0,3% | -0,7% | -3,0% |
| Class E |  | -0,1% | -0,8% | 2,1% |  | -2,4% | -0,5% | 1,9% |
| Overall |  | -0,1% | -0,7% | -0,5% |  | -1,1% | -1,3% | -1,5% |
|  |  | -0,1% | -0,7% | -0,6% |  | -1,1% | -1,3% | -1,5% |
| Enc Time[%] |  | 100% |  |  |  | 133% |  |  |
| Dec Time[%] |  | 103% |  |  |  | 102% |  |  |

# Encoding times have been computed on a linux cluster and are not reliable. The encoding time is expected to be similar to the HM4.0. The decoding time on a single PC is increased by 1.8% for common conditions at a YUV BDR reduction of 0.4%. The decoding time increase including low delay P coding is 2.1% at a YUV BDR reduction of 0.5%.

## Visual inspection

Looking at decoded video sequences it can be seen that the proposed approach can improve the sharpness of local image edges and also remove other coding artefacts.

# Conclusion

It has been shown that by adding the possibility to use more specific edge offsets when needed both the subjective and the objective quality can be improved. The average YUV BDR is improved by 0.4% (average BDR luma 0.2%, chroma 1%). The most improvements are seen for low delay configurations. The encoding time is similar to HM4.0 and decoding time is increased by 1.8% compared to HM4.0.

# References

- F. Bossen, “Common test conditions and software reference configurations” JCTVC-F900, Italy, July 2011

- C.-M. Fu, C.-Y. Chen, C-Y Tsai, Y.-W. Huang, and S. Lei, “CE13: Sample Adaptive Offset with LCU-Independent Decoding”, JCTVC-E049, March 2011

- C.-M. Fu, C.-Y. Chen, Y.-W. Huang, and S. Lei, “Sample Adaptive Offset for Chroma”, JCTVC-F057, July 2011

- “WD4: Working Draft 4 of High-Efficiency Video Coding”, JCTVC-F803, Italy, July 2011

# Patent rights declaration(s)

Ericsson AB may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).