| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G493 |
| --- | --- |

| Title: | CABAC Stream Termination |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information & Proposal |  |  |
| Author(s) or Contact(s): | K. Sharman, J. Gamei, N. Saunders, P. Silcock Sony BPRL Jays Close, Viables, Basingstoke RG22 4SB, UK | Tel: Email: | +44 1256 355011 karl.sharman@eu.sony.com |
| Source: | Sony Europe Limited |  |  |

_____________________________

# Abstract

A technique to terminate a CABAC stream is presented, and it is stated that the method produces a loss of on average 1.5 bits. A second alternative method is also presented which is claimed to produce an average loss of 1 bit. Applications of these techniques have been suggested to include termination of the CABAC stream prior to IPCM data, and termination of the stream for row-per-slice. In the working-draft HEVC design (JCT-VC F803), use of the CABAC stream termination method causes 8 bits to be flushed to the stream. The technique is illustrated with an example where intra frames are terminated after each LCU, allowing the coefficient bypass data (sign bits/escape codes) to be placed into the bit-stream in a raw format.

# Introduction

In the current working-draft and reference software (HM4.0), a process to terminate the CABAC stream is applied at the end of each slice and prior to IPCM data. This process assumes that the probability that the stream is to be terminated is fixed at on average 0.54%.

For the encoder, this process is:

- if the stream is not to be terminated, the CABAC range is decremented by 2, and the CABAC engine is renormalized by 1 place if required; processing on the current CABAC stream continues.

- if the stream is to be terminated, the CABAC ‘low’ is incremented by ‘the range less 2’, the range is set to 2, and the CABAC engine is renormalized by 8 places, followed by outputting a further binary ‘1’. This process is equivalent to a renormalization of 9 places, where the value being renormalized has been forced to be an odd number.

There may be occasions where the above process is not optimal – i.e. where the probability of termination is variable, or fixed at a higher percentage, or even a certainty (probability of 1).

This proposal describes a method whereby the CABAC stream can be immediately terminated with just 2 renormalizations, with a loss of (on average) 1.5 bits and negligible impact on the decoder and encoder complexity. An alternative method is also indicated that can reduce the overhead to just 1 bit, but at the expense of an increase in CABAC decoder complexity. Both methods can then be used in conjunction with a standard adaptive context variable if there is a variable probability of termination, or in conjunction with a fixed percentage mechanism (akin to a non-adaptive context variable).

# Algorithm

## Proposal 1

The steps of the encoder are as follows (using the naming convention of HM4.0):

m_uiLow=(m_uiLow+128)&~127			{or m_uiLow=(m_uiLow+127)&~127}

Force 2 stages of renormalization of m_uiLow and call test_write_out().

Prior to encoding next CABAC stream, set m_uiRange=510, m_uiLow=0.

The steps of the decoder are as follows (using the naming convention of HM4.0):

Rewind input stream by 7 bits (i.e. move read position back by 7 bits).

Prior to decoding next CABAC stream, set m_uiRange=510, and read m_uiLow from bit-stream.

This method has negligible processing impact on the decoder and encoder.

## Proposal 2

This method increases the complexity of the current HM4.0 decoder as it requires that the decoder maintains m_uiLow. If maintenance of m_uiLow is required by other proposals, then this additional complexity is again minimal.

The steps of the encoder are as follows (using the naming convention of HM4.0):

Let test256=(m_uiLow+255)&~255

If (test256+255 < m_uiLow+m_uiRange)

m_uiLow=test256

Force 1 stage of renormalization of m_uiLow and call test_write_out().

Else (as before)

m_uiLow=(m_uiLow+128)&~127   		{ or m_uiLow=(m_uiLow+127)&~127 }

Force 2 stages of renormalization of m_uiLow and call test_write_out().

Prior to encoding next CABAC stream, set m_uiRange=510, m_uiLow=0.

The steps of the decoder are as follows (using the naming convention of HM4.0):

Let test256=(m_uiLow+255)&~255

If (test256+255 < m_uiLow+m_uiRange)

Rewind stream by 8 bits

Else (as before)

Rewind stream by 7 bits

Prior to decoding next CABAC stream, set m_uiRange=510, and read m_uiLow from bit-stream.

## Theory

For the CABAC encoder, the data written to the stream (or buffered) concatenated with m_uiLow is an n-bit value low indicating the lowest value that the final output can be. The highest value, high, is the sum of low and m_uiRange, a variable maintained by the encoder to be within the range 256 (inclusive) to 511 (exclusive). At the end of the stream, any value between low (inclusive) and high (exclusive) can be selected as the final output value, without affecting the decode. If the decode could occur without being dependent on the n LSBs of the value, then the n LSBs could be replaced with data from the next section of the bit-stream.

Let v be a value between low and high where n LSBs are 0, and where if the last n LSBs were 1, the resulting value V would still be less than high. Since "high - low" is at least 256, then there will always be a value v between low and high that has at least 7 LSBs that are 0. i.e. the value v is the first value between low and high that is divisible by 128 without a remainder.

The simplest manner to achieve this is a standard power-of-2 alignment routine, namely:

v=(low+127)&~127

However, since range is at least 256, then:

v=(low+128)&~127

is also sufficient (and results in a slightly smaller encoder).

For the current part of the bit-stream, the encoder would output the value ‘v’, except for the bottom 7 bits, this is achieved by renormalizing m_uiLow by 2 places. At the end of the bit-stream, the decoder would have read 7 bits from the next section of the bit stream, and therefore would have to ‘rewind’ the bit-stream by 7 bits.

There are cases where the bottom 8 bits are not required to fully decode the stream, with the simplest illustration being where "m_uiLow = 0", and these are explored by the alternative algorithm. In this alternative algorithm, the value v between low and high with 8 LSBs of 0 is calculated, and then a test is applied to check if there is a corresponding value V. The decision process requires tests on low and high, and since the decoder must also make the same decision, the decoder would need to track m_uiLow.

In both versions of the encoder algorithm, there is a choice for the 7-bit path, which will result in a different bit-stream, but will be decodable by the same decoder.

# Applications

Possible applications for this include:

- Termination for the last encoded LCU for a slice, especially in a ‘row-per-slice’ style configuration, where the probability may be significantly higher than 0.54%

- Termination for the last possible LCU for a slice (as termination after the last possible LCU of a slice is a certainty).

- Termination prior to IPCM data (possibly in conjunction with a context variable).

- Termination of the stream to prevent the “bits outstanding” mechanism getting too long.

# Simulation Results

In addition to the aforementioned applications, the technique has been demonstrated in a system where the CABAC stream is terminated after each LCU in the intra high-efficiency configuration. The system placed the bypass data (sign bits and escape data) as raw binary data after each LCU. The simulation results below for Proposal 1 demonstrate that the coding loss observed is equivalent to on average 1.5 bits per LCU, as expected. Results for the alternative method (Proposal 2) demonstrate the expected coding loss of on average 1 bit per LCU.

# Conclusion

A method (Proposal 1) has been presented that allows a stream to be terminated with 1.5 bits of loss, with negligible impact on the encoder and decoder, and this method has been extended further to an alternative method (Proposal 2) reducing the loss to 1 bit, albeit with the additional task of tracking the CABAC low value in the decoder. Simulation results of the two methods where the CABAC stream is terminated after each LCU, with bypass data being placed in raw form after the termination point, confirm the 1.5 bit and 1 bit average cost per LCU.

# Patent rights declaration(s)

Sony Corporation or Sony Group companies may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).