| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G494 |
| --- | --- |

| Title: | CABAC Packet-based Stream |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | K. Sharman, J. Gamei, N. Saunders, P. Silcock Sony BPRL Jays Close, Viables, Basingstoke RG22 4SB, UK | Tel: Email: | +44 1256 355011 karl.sharman@eu.sony.com |
| Source: | Sony Europe Limited |  |  |

_____________________________

# Abstract

A method of arranging CABAC data and bypass data into packets is presented, where the bypass data comprises equal-probability bits. This method is stated to allow the insertion of bypass data into the bit-stream in raw binary form, which then allows the bypass data to be read and decoded in parallel with CABAC data. The reported cost of this method is an average of 8 bits per slice with no effect on software encoding and decoding times.

# Introduction

In the present (HM4.0) system, bypass data are coded into the CABAC stream by encoding each bit as if it were a CABAC bit with a fixed probability of 50%. This effectively involves multiplication of the current range by the bypass bits during encode, and requires iterative logic (or a divide) to decode multiple bits. Bypass data cannot be directly introduced into the bit-stream because the HM4.0 decoder has no way of knowing which bits are bypass data and which are CABAC data.

By placing the bypass data at specific positions in the stream that are known to both the encoder and decoder, it not only becomes possible to write bypass data in raw binary form, allowing multiple bits to be read simply, but also allows the bypass bits to be decoded in parallel. This would allow the bypass data (e.g. sign) to be decoded in parallel with the CABAC data (e.g. significance map).

The techniques described below achieve this by arranging the data into packets of defined size, supporting parallel decoding.

# CABAC Packet Algorithm

It is proposed that the CABAC data (i.e. data that use context variables) be written to the bit-stream in fixed-size packets of 16 bits, referred to as 'CABAC packets'. After each CABAC packet, the corresponding 'Bypass packet' is written to the bit-stream.

The Bypass packet (which is variable in size) comprises any bypass bits that attach to CABAC data that can be decoded (including renormalization) using only the bits contained within preceding CABAC packets; this bypass data is inserted directly into the stream.

To generate a CABAC Packet-based stream, the encoder must track how many bits the decoder has read after each renormalization process. The encoder is required to start counting at a number of bits equal to the decoder’s initial read (nine in the HM4.0 system).

## CABAC Last Packet Termination

If the final CABAC packet has any associated bypass data, this final CABAC packet is padded out to its fixed size of 16 bits, ensuring that the bypass data is in the expected position for the decoder.

If there is no bypass data for the final CABAC packet, no padding is necessary and the decoder can detect that it has reached the end of the stream.

The last CABAC packet padding bits are the only source of loss with this proposal, giving an average loss of 8 bits (because padding between 0 and 16 bits may be required).

## Outstanding Bits

In order to output the stream in the correct order, the Bypass packets must be saved by the encoder until their corresponding CABAC packets have been generated.

Since the encoder may defer writing bits for a long time, even up to the entire stream, due to those bits being outstanding, the number of Bypass packets that must be buffered is potentially unbounded.

To limit the number of Bypass packets that must be buffered, it is necessary to limit the number of outstanding bits. This can be achieved using methods such as those presented in JCTVC-F059 “CABAC with Constrained Outstanding Bits” or in the following section.

# Limitation of Outstanding Bits using Stream Termination

This method allows the outstanding bits to be limited to a fixed number without requiring that they are checked after every renormalization. To enable this method, the decoder must track m_uiLow as it would appear in the encoder.

Bits renormalized from m_uiLow are accumulated in a buffer. If, after renormalization, at least a certain number of bits have accumulated in the buffer, they are considered to form a 'group'. The minimum group size is proposed to be 15 bits. The maximum group size would therefore be equal to 22 (14 + 8 (8 bits being the maximum possible renormalization)).

If all the bits in a group are ones and there is no carry, the group is considered to be outstanding. The encoder and decoder each keep a count of the number of outstanding groups. The encoder also keeps a count of the sum of the outstanding group sizes so that it knows how many outstanding bits need to be flushed when a non-outstanding group is encountered.

In the encoder, the last non-outstanding group is stored in a buffer. If a subsequent group is not outstanding, the group currently in the buffer is flushed to the stream along with any outstanding bits. The new group is then stored in the buffer.

If a number of outstanding groups greater than or equal to a defined limit is encountered, the stream is terminated according to the method presented in JCTVC-G493 “CABAC Stream Termination”. This ensures the next group will not be outstanding, allowing the accumulated outstanding groups to be flushed.

Using this method, the maximum possible count of outstanding bits is equal to the maximum group size multiplied by the limiting number of outstanding groups.

# Illustrative Decoder Hardware Implementation

A CABAC Packet-based stream can be decoded using a shift register as the buffer. Each time data is read from the buffer, the remaining bits are shifted to fill the space occupied by the read bits, and new data from the stream is added to the end of the register.

- CABAC data is read into m_uiValue from the front of the shift register, both at the start of decoding and after each renormalization.

- Bypass data is read from a position indicated by a bypass index. This index is initially set to 16 (the start of the first ‘Bypass packet’) and decremented on each read of CABAC data by the number of bits read.

- In a parallel system, both CABAC and bypass read-and-shifts happen simultaneously.

- At the end of a CABAC packet (i.e. when a CABAC read passes the bypass index), the bypass index is incremented by 16, jumping it past the next CABAC packet.

- This can be achieved without a stall in a parallel system by speculatively reading from a position equal to the bypass index plus 16 as well as the bypass index itself.

- The data in a Bypass packet attaches only to CABAC data within the preceding CABAC packet. This ensures that, by the time a CABAC read would pass the end of the current CABAC packet, all the data in the associated Bypass packet will have been read and therefore the end of the current CABAC packet is directly adjacent to the start of the next.

  - This allows CABAC data to always be safely read past the end of the current CABAC packet as though it were one continuous stream.

The size of the shift register is determined by the need to read the maximum possible number of bypass bits from the furthest possible position and is calculated to be 67 bits ((8 – 1 + 16) {furthest bypass read index} + 44 {largest possible bypass read}), assuming coefficients have maximum magnitude of 32768 (16 bit signed).

# Illustrative Encoder Hardware Implementation

To encode a CABAC Packet-based stream, an output buffer is required that can be written to in multiple places. However, a buffer is already required for HM4.0 encoder implementation due to outstanding bits.

- Two write pointers are used to index this buffer, the first indicating where to write CABAC data (starting at zero) the second indicating where to write bypass data (starting at 16).

- Each time the encoder detects that the decoder has read 16 bits, the bypass pointer’s position is noted and incremented by 16. When the encoder finishes writing the current CABAC packet, the CABAC pointer is set equal to the noted previous position of the bypass pointer.

  - In this way, each pointer “jumps over” the other’s data.

- After any given renormalization, the difference between the total number of bits the decoder has read and the total number of bits the encoder has written can be greater than the size of a packet.

- Therefore it is necessary to store multiple previous bypass pointer locations. The required number of pointers is bounded by the maximum difference and is the reason for limiting outstanding bits.

- Non-word-aligned writes to the buffer can be executed in a single write cycle by caching the bytes surrounding the target region.

# Simulation Results

The results presented below show BD-rate changes for a packet-based CABAC system in HM4.0, both with and without group-based limitation of outstanding bits; average {Y, U, V} BD-rate changes are {+0.01%, +0.01%, +0.01%} and {+0.01%, +0.02%, +0.02%} for each system, respectively.

We would like to thank Qualcomm for cross-checking these results.

- Conclusion

A method has been presented of arranging the CABAC and bypass data into packets, allowing the bypass data to be inserted directly into the stream and decoded in parallel with the CABAC data. This method has been shown to incur a minimal BD-rate cost. It has also been shown to require only a small increase in complexity provided a limit is placed on outstanding bits.

A method that limits outstanding bits without requiring that the count of outstanding bits is checked after each renormalization has also been presented.

This system is also used in another proposal where the three context variables for each coefficient are combined (JCTVC-G504 “CABAC with Combined Context Variables”).

# Patent rights declaration(s)

Sony Corporation or Sony Group companies may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).