| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G496 |
| --- | --- |

| Title: | Core transform design for HEVC with 7 bit coefficients |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | A. Fuldseth, Cisco Systems G. Bjøntegaard, Cisco Systems M. Budagavi, Texas Instruments Inc., USA V. Sze, Texas Instruments Inc., USA | Tel: Email: Tel: Email Tel: Email Tel: Email | +47 92414082 arild.fuldseth@cisco.com gbjonteg@cisco.com +1-214-480-1861 madhukar@ti.com sze@ti.com |
| Source: | Cisco Systems, Texas Instruments Inc |  |  |

_____________________________

# Abstract

This contribution proposes a set of 7 bit transform matrices for HEVC, covering all transform sizes from 4x4 to 32x32. The proposed transform matrices have the same properties as the 8 bit transform matrices currently used in the HM transforms. The transform matrices and the associated transform operations described in this contribution are proposed for the core transform design in HEVC. The proposed transform design has the following properties; 16 bit data representation before and after each transform stage (independent of the internal bit depth), 16 bit multipliers for all internal multiplications, no need for correction of different norms of basis vectors during quantization/dequantization, all transform sizes above 4x4 can reuse arithmetic operations for smaller transform sizes, and implementations using either pure matrix multiplication or a combination of matrix multiplication and butterfly structures are possible. hardware complexity analysis shows that the gate count can be reduced by X% by using 7 bit matrices instead of 8 bit matrices. BD-rate results vary between -0.1% and 0.2% over all test cases and low (-1,5,9,13), normal (22,27,32,37) and high (36,42,47,51) QP ranges. The 7 bit transform matrices offers between 8% and 16% reduction in hw costs when compared to the 8 bit versions currently used in HM.

# Introduction

In [1] and [2] transform designs facilitating matrix multiplication and 16 bit intermediate data representations were proposed independently by Cisco and TI. These contributions were combined into joint proposals in [3], [4], and [5]. In the 5th meeting of the JCT-VC it was decided to adopt the transform design of [3] in the HM software. This contribution proposes a transform structure with exactly the same properties as in [3], [4], and [5] but with 7 bit coefficients instead of 8 bit.

The proposed transform design has the following properties.

- Bit width of 16 for all transform buffers. This is true for all values of internal bit depth parameter for the encoder/decoder.

- 16 bit multipliers for all internal calculations. This is true for all values of the internal bit depth parameter.

- The norms of the basis vectors are sufficiently close so that the same scaling can be used for all transform coefficients during quantization/dequantization. Also, the scaling is the same for all transform sizes, except for right shifts that depend on the transform size. This eliminates the need for large quantization/dequantization matrices.

- Coefficients and basis vectors of the smaller transforms are a subset of coefficients and basis vectors of larger transforms. This allows for re-use of logic for multiply/adds in hardware design.

- The proposed transform design is specified in terms of NxN matrices and also the associated right shift to be used after each transform stage. This allows for more additional flexibility in implementations, such as direct matrix multiplications with SIMD instruction for the innermost loop, or multi-stage butterfly implementations utilizing symmetry properties.

- 7 bit representation of transform coefficients.

- Bit width of accumulators for matrix multiplication (before right shift and storage) does not exceed 32 bit.

- Same symmetry/anti-symmetry properties as the DCT transform.

- The scaled transform coefficients are close to the DCT transform.

- The basis vectors are almost orthogonal.

- The same quantization/dequantization scheme can be used for all transform sizes. Quantization/dequantization is implemented with multipliers and shift operations, where the multipliers depend only on the value of QP and the shifts depend only on the transform size.

# Description

2.1 Basis vectors

The basis vectors of the 4x4 and 8x8 transforms are shown below. The basis vectors of the 16x16 and 32x32 transforms are shown in Appendix A.

4x4 transform:

{32, 32, 32, 32}
		{41, 19,-19,-41}
		{32,-32,-32, 32}
		{19,-41, 41,-19}

8x8 transform:

{32, 32, 32, 32, 32, 32, 32, 32}

{44, 38, 25,  9, -9,-25,-38,-44}

{41, 19,-19,-41,-41,-19, 19, 41}

{38, -9,-44,-25, 25, 44,  9,-38}

{32,-32,-32, 32, 32,-32,-32, 32}

{25,-44,  9, 38,-38, -9, 44,-25}

{19,-41, 41,-19,-19, 41,-41, 19}

{ 9,-25, 38,-44, 44,-38, 25, -9}

2.2 Intermediate scaling and quantization/dequantization.

In this subsection, intermediate scaling and quantization/dequantization are described in more detail. In addition to specifying the scaling in the dequantization and the inverse transform, we also specify the forward transform and quantizer that we have used for our simulation results.

Definitions

B	= internal bit depth (as specified by InternalBitDepth in the common config files)

N 	= transform size

M	= log2(N)

Q	= f(QP%6), 	where f(x)   =   {26214,23302,20560,18396,16384,14564}, x=0,…,5

IQ 	= g(QP%6), 	where g(x)  =   {40,45,51,57,64,72}, x=0,…,5

Forward transform

The following applies to the implementation of the forward transform.

- The forward transform is specified as a matrix multiplication in two stages, horizontal and vertical.

- After the first stage, the resulting transform coefficients are scaled as follows

y  =  (x + offset)>>(M-2+(B-8)),		offset = (1<<(M-3+ (B-8)))

- After the second stage, the resulting transform coefficients are scaled as follows

y  =  (x + offset)>>(M+5),		offset = (1<<(M+4))

This ensures a bit width of 16 after each transform stage.

Quantization

Quantization can be implemented as follows (RDOQ=off):

level  =  (coeff*Q + offset)>>(21+QP/6–M–(B-8))

For RDOQ, the scaling and cost function calculations are adjusted accordingly.

Dequantization

The dequantizer is specified as follows

coeffQ 	=  ((level*IQ << (QP/6)) + offset)>>(M-1+(B-8)),  offset = 1<<(M-2+(B-8))

coeffQ = min(32767,max(-32768,coeffQ))

The clipping ensures that the value of the quantized transform coefficients, coeffQ, can be represented with 16 bits. The scaling is chosen so that clipping only occur for extreme input values and/or unreasonable quantizer behaviour.

Inverse transform

The following applies to the implementation of the inverse transform.

- The inverse transform is specified as a matrix multiplication in two stages, horizontal and vertical.

- After the first stage, the resulting transform coefficients are scaled as follows

y  =  (x + offset)>>6			offset = 1<<(6-1)

This ensures a bit width of 16 after the first inverse transform stage assuming the quantization/dequantization scheme above.

- After the second stage, the resulting transform coefficients are scaled as follows

y  =  (x + offset)>>(11-(B-8))		offset = 1<<(10-(B-8))

Quantization step size and QP

The quantizer multipliers, f(x) and dequantizer multipliers, g(x) are chosen so that

f(x) * g(x)  (1<<20), 	x = 0,1,…,5

Furthermore, assume that we have an N*N DC block with pixel values equal to X.  If this block is input to an orthonormal transformation containing a DC basis vector (e.g. the Discrete Cosine Transform), the value of the DC coefficient is equal to X*N.  With the proposed transforms and quantization/ dequantiztion scheme, the same relationship between input pixel values and the value of level applies when QP = 4. In other words, if all pixel values in an NxN block are equal to X, and QP=4, this will result in a DC coefficient with level = X*N after quantization. The range of QP values maps to approximately the same range of quantization step sizes as in HM2.0.

# Results

BD-Rate results for the 7-bit vs. 8-bit (anchor) are provided in Table1, Table 2, and Table 3 for normal QP, low QP, and high QP ranges respectively.

Table 1 BD-rate results for 7 bit transform matrices, normal QP range (22,27,32,37)

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0 % | 0.0 % | -0.1 % | 0.2 % | 0.0 % | -0.2 % |
| Class B | 0.0 % | 0.0 % | 0.0 % | 0.2 % | -0.1 % | -0.1 % |
| Class C | 0.0 % | 0.0 % | 0.0 % | 0.0 % | -0.1 % | 0.0 % |
| Class D | 0.0 % | 0.0 % | 0.0 % | 0.0 % | 0.0 % | 0.0 % |
| Class E | 0.0 % | 0.1 % | 0.1 % | 0.1 % | 0.0 % | 0.1 % |
| Class F | 0.0 % | 0.0 % | 0.0 % | 0.0 % | 0.0 % | 0.0 % |
| Overall | 0.0 % | 0.0 % | 0.0 % | 0.1 % | 0.0 % | -0.1 % |
|  | 0.0 % | 0.0 % | 0.0 % | 0.1 % | 0.0 % | -0.1 % |
| Enc Time[%] | 100.0 % |  |  | 100.0 % |  |  |
| Dec Time[%] | 100.0 % |  |  | 100.0 % |  |  |
|  | Random Access HE |  |  | Random Access LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0.0 % | -0.3 % | -0.7 % | 0.1 % | -0.1 % | -0.2 % |
| Class B | 0.0 % | -0.1 % | -0.1 % | 0.1 % | -0.1 % | -0.1 % |
| Class C | 0.0 % | -0.1 % | 0.1 % | 0.0 % | -0.3 % | -0.1 % |
| Class D | 0.0 % | 0.0 % | -0.2 % | 0.0 % | -0.2 % | -0.2 % |
| Class E |  |  |  |  |  |  |
| Class F | 0.0 % | 0.0 % | 0.0 % | 0.0 % | 0.0 % | 0.0 % |
| Overall | 0.0 % | -0.1 % | -0.2 % | 0.0 % | -0.2 % | -0.1 % |
|  | 0.0 % | -0.1 % | -0.2 % | 0.1 % | -0.2 % | -0.1 % |
| Enc Time[%] | 100.0 % |  |  | 100.0 % |  |  |
| Dec Time[%] | 100.0 % |  |  | 100.0 % |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0 % | -0.1 % | -0.5 % | 0.0 % | 0.1 % | 0.2 % |
| Class C | 0.0 % | -0.2 % | -0.2 % | 0.0 % | 0.0 % | -0.1 % |
| Class D | 0.0 % | -0.3 % | -0.2 % | 0.0 % | 0.0 % | -0.2 % |
| Class E | 0.0 % | 0.2 % | 0.4 % | -0.1 % | -0.5 % | -0.3 % |
| Class F | 0.0 % | 0.0 % | 0.0 % | 0.0 % | 0.0 % | 0.0 % |
| Overall | 0.0 % | -0.1 % | -0.2 % | 0.0 % | -0.1 % | -0.1 % |
|  | 0.0 % | -0.2 % | -0.2 % | 0.0 % | -0.1 % | -0.1 % |
| Enc Time[%] | 100.0 % |  |  | 100.0 % |  |  |
| Dec Time[%] | 100.0 % |  |  | 100.0 % |  |  |

Table 2 BD-rate results for 7 bit transform matrices, low QP range (1,5,9,13)

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0 % | 0.1 % | 0.1 % | 0.2 % | 0.0 % | 0.1 % |
| Class B | 0.2 % | 0.1 % | 0.1 % | 0.2 % | 0.1 % | 0.1 % |
| Class C | 0.0 % | 0.0 % | 0.1 % | 0.3 % | 0.1 % | 0.1 % |
| Class D | 0.0 % | 0.0 % | 0.1 % | 0.3 % | 0.1 % | 0.2 % |
| Class E | 0.0 % | 0.1 % | 0.1 % | 0.1 % | 0.1 % | 0.1 % |
| Class F | 0.0 % | 0.0 % | 0.0 % | 0.0 % | 0.0 % | 0.0 % |
| Overall | 0.1 % | 0.1 % | 0.1 % | 0.2 % | 0.1 % | 0.1 % |
|  | 0.0 % | 0.1 % | 0.1 % | 0.2 % | 0.1 % | 0.1 % |
| Enc Time[%] | 100.0 % |  |  | 100.0 % |  |  |
| Dec Time[%] | 100.0 % |  |  | 100.0 % |  |  |
|  | Random Access HE |  |  | Random Access LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0.0 % | 0.1 % | 0.1 % | 0.1 % | 0.0 % | 0.0 % |
| Class B | 0.1 % | 0.0 % | 0.0 % | 0.1 % | 0.0 % | 0.0 % |
| Class C | 0.0 % | 0.0 % | 0.0 % | 0.0 % | 0.0 % | 0.0 % |
| Class D | 0.0 % | 0.0 % | 0.0 % | 0.1 % | 0.0 % | 0.0 % |
| Class E |  |  |  |  |  |  |
| Class F | 0.0 % | 0.0 % | 0.0 % | 0.0 % | 0.0 % | 0.0 % |
| Overall | 0.0 % | 0.0 % | 0.0 % | 0.1 % | 0.0 % | 0.0 % |
|  | 0.0 % | 0.0 % | 0.0 % | 0.1 % | 0.0 % | 0.0 % |
| Enc Time[%] | 100.0 % |  |  | 100.0 % |  |  |
| Dec Time[%] | 100.0 % |  |  | 100.0 % |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.1 % | 0.0 % | 0.0 % | 0.1 % | 0.0 % | 0.0 % |
| Class C | 0.0 % | 0.0 % | 0.0 % | 0.0 % | 0.1 % | 0.1 % |
| Class D | 0.0 % | 0.0 % | -0.1 % | 0.0 % | 0.1 % | 0.1 % |
| Class E | 0.0 % | -0.1 % | -0.1 % | 0.0 % | 0.1 % | 0.1 % |
| Class F | 0.0 % | 0.0 % | 0.0 % | 0.0 % | 0.0 % | 0.0 % |
| Overall | 0.0 % | 0.0 % | -0.1 % | 0.0 % | 0.1 % | 0.1 % |
|  | 0.0 % | 0.0 % | 0.0 % | 0.0 % | 0.2 % | 0.3 % |
| Enc Time[%] | 100.0 % |  |  | 100.0 % |  |  |
| Dec Time[%] | 100.0 % |  |  | 100.0 % |  |  |

Table 3 BD-rate results for 7 bit transform matrices, high QP range (36,42,47,51)

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.1 % | 0.1 % | -0.3 % | 0.1 % | 0.3 % | 0.0 % |
| Class B | 0.0 % | 0.3 % | 0.1 % | 0.1 % | -0.1 % | -0.3 % |
| Class C | 0.0 % | -0.2 % | 0.1 % | 0.0 % | 0.0 % | -0.2 % |
| Class D | 0.0 % | -0.1 % | -0.4 % | 0.0 % | 0.3 % | -0.5 % |
| Class E | 0.0 % | -1.7 % | -0.9 % | 0.0 % | 0.7 % | 0.5 % |
| Class F | 0.0 % | 0.0 % | 0.0 % | 0.0 % | 0.0 % | 0.0 % |
| Overall | 0.0 % | -0.2 % | -0.2 % | 0.0 % | 0.2 % | -0.1 % |
|  | 0.0 % | 4.7 % | 0.4 % | 0.0 % | 152.1 % | 2.5 % |
| Enc Time[%] | 100.0 % |  |  | 100.0 % |  |  |
| Dec Time[%] | 100.0 % |  |  | 100.0 % |  |  |
|  | Random Access HE |  |  | Random Access LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0.0 % | -3.4 % | -2.3 % | 0.0 % | -0.7 % | -1.1 % |
| Class B | -0.1 % | -0.5 % | 2.0 % | 0.0 % | -0.5 % | -0.3 % |
| Class C | -0.1 % | 0.1 % | -0.3 % | -0.2 % | 0.4 % | -1.0 % |
| Class D | 0.0 % | 2.5 % | 2.6 % | 0.0 % | 0.2 % | -2.2 % |
| Class E |  |  |  |  |  |  |
| Class F | 0.0 % | 0.0 % | 0.0 % | 0.0 % | 0.0 % | 0.0 % |
| Overall | -0.1 % | -0.3 % | 0.6 % | 0.0 % | -0.2 % | -1.1 % |
|  | -0.1 % | -3.0 % | 4.8 % | -0.1 % | -0.2 % | 0.9 % |
| Enc Time[%] | 100.0 % |  |  | 100.0 % |  |  |
| Dec Time[%] | 100.0 % |  |  | 100.0 % |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.2 % | 0.9 % | -0.7 % | -0.1 % | -0.4 % | -0.8 % |
| Class C | -0.2 % | -1.6 % | -0.9 % | 0.0 % | 0.4 % | 0.4 % |
| Class D | 0.1 % | 5.3 % | 1.8 % | 0.3 % | 2.9 % | 1.2 % |
| Class E | -0.6 % | -1.5 % | 1.5 % | -0.1 % | -6.4 % | 5.8 % |
| Class F | 0.0 % | 0.0 % | 0.0 % | 0.0 % | 0.0 % | 0.0 % |
| Overall | -0.1 % | 0.9 % | 0.3 % | 0.0 % | -0.5 % | 1.2 % |
|  | -0.1 % | 1.1 % | -1.2 % | 0.0 % | -4.2 % | 5.9 % |
| Enc Time[%] | 100.0 % |  |  | 100.0 % |  |  |
| Dec Time[%] | 100.0 % |  |  | 100.0 % |  |  |

# Hardware complexity analysis

Integer transforms for HEVC with 7 bit transform matrices was implemented in RTL using partial butterfly architecture. Table below provides a comparison of hardware area estimates for HM4 and 7 bit transform.

|  | 32-pt HM4 inverse only |  |  |  | 32-pt 7-bit transform inverse only |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| frequency (MHz) \ L | 2 | 4 | 6 | 8 | 2 | 4 | 6 | 8 |
| 150 | 123 | 116 | 122 | 129 | 114 | 103 | 109 | 115 |
| 200 | 152 | 122 | 127 | 133 | 137 | 108 | 113 | 119 |
| 250 | 234 | 130 | 133 | 139 | 213 | 115 | 119 | 125 |
| 300 |  | 140 | 141 | 147 |  | 123 | 125 | 132 |
| 350 |  | 165 | 150 | 155 |  | 143 | 133 | 139 |
| 400 |  |  | 167 | 164 |  |  | 150 | 147 |
| 450 |  |  | 195 | 175 |  |  |  | 156 |
| 500 |  |  |  | 189 |  |  |  | 169 |

|  | 32-pt HM4 forward only |  |  |  | 32-pt 7-bit transform forward only |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| frequency (MHz) \ L | 2 | 4 | 6 | 8 | 2 | 4 | 6 | 8 |
| 150 | 126 | 129 | 135 | 141 | 109 | 110 | 117 | 123 |
| 200 | 167 | 139 | 141 | 147 | 137 | 119 | 122 | 128 |
| 250 |  | 156 | 148 | 154 | 286 | 132 | 128 | 134 |
| 300 |  | 190 | 157 | 163 |  | 161 | 136 | 142 |
| 350 |  |  | 170 | 172 |  |  | 144 | 149 |
| 400 |  |  | 192 | 182 |  |  | 162 | 157 |
| 450 |  |  |  | 195 |  |  | 189 | 166 |
| 500 |  |  |  | 216 |  |  |  | 182 |

|  | 32-pt HM4 inverse + forward |  |  |  | 32-pt 7-bit transform inverse + forward |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| frequency (MHz) \ L | 2 | 4 | 6 | 8 | 2 | 4 | 6 | 8 |
| 150 | 211 | 139 | 145 | 151 | 199 | 125 | 131 | 137 |
| 200 |  | 146 | 150 | 156 |  | 133 | 137 | 143 |
| 250 |  | 166 | 156 | 162 |  | 152 | 143 | 149 |
| 300 |  | 209 | 169 | 170 |  | 188 | 152 | 156 |
| 350 |  |  | 181 | 182 |  |  | 162 | 163 |
| 400 |  |  | 202 | 192 |  |  | 182 | 174 |
| 450 |  |  |  | 206 |  |  | 213 | 189 |
| 500 |  |  |  | 226 |  |  |  | 208 |

The percent area savings based on the frequency/latency combinations tested is as follows:

- Inverse transform only area savings: 10-12%

- Forward transform only area savings: 13-16%

- Joint forward + inverse transform area savings: 8-10%

# Conclusions

Integer transforms for HEVC with 7 bit transform matrices have been proposed. The proposed transforms have several implementation benefits compared to the existing transforms, including

- 16 bit data representation before and after each transform stage independent of IBDI settings.

- Only 16 bit multipliers are needed for internal calculations.

- No need for correction of different norms of basis vectors during quantization/dequantization.

- Reusability of smaller transforms in larger transforms.

- Allows both matrix multiplication and partial butterfly structure.

The 7-bit matrices result in marginal BD-Rate gains/losses compared to the 8-bit versions currently in HM, while offering a significant reduction in HW costs.

# References

- A. Fuldseth and G. Bjøntegaard, “Unified transform design for HEVC with 16 internal data representation,” JCTVC-D224, Daegu, January, 2011.

- M. Sadafale and M. Budagavi, “Matrix multiplication specification for HEVC transforms,” JCTVC-D036, Daegu, January, 2011.

- A. Fuldseth, G. Bjøntegaard, M. Sadafale, M. Budagavi, “Transform design for HEVC with 16 bit intermediate data representation,” JCTVC-E243, Geneva, March, 2011.

- A. Fuldseth, G. Bjøntegaard, M. Sadafale, M. Budagavi, “Core transform design for HEVC,” JCTVC-F446, Torino, July, 2011.

- A. Fuldseth, G. Bjøntegaard, M. Budagavi, “Core transform design for HEVC,” JCTVC-G495, Geneva, November, 2011.

# Patent rights declaration(s)

Cisco and Texas Instruments may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Appendix A

16x16 transform

{32 32 32 32 32 32 32 32 32 32 32 32 32 32 32 32}
{44 44 39 35 30 22 13  5 -5-13-22-30-35-39-44-44}
{44 38 25  9 -9-25-38-44-44-38-25 -9  9 25 38 44}
{44 30  5-22-39-44-35-13 13 35 44 39 22 -5-30-44}
{41 19-19-41-41-19 19 41 41 19-19-41-41-19 19 41}
{39  5-35-44-13 30 44 22-22-44-30 13 44 35 -5-39}
{38 -9-44-25 25 44  9-38-38  9 44 25-25-44 -9 38}
{35-22-44  5 44 13-39-30 30 39-13-44 -5 44 22-35}
{32-32-32 32 32-32-32 32 32-32-32 32 32-32-32 32}
{30-39-13 44 -5-44 22 35-35-22 44  5-44 13 39-30}
{25-44  9 38-38 -9 44-25-25 44 -9-38 38  9-44 25}
{22-44 30 13-44 35  5-39 39 -5-35 44-13-30 44-22}
{19-41 41-19-19 41-41 19 19-41 41-19-19 41-41 19}
{13-35 44-39 22  5-30 44-44 30 -5-22 39-44 35-13}
{ 9-25 38-44 44-38 25 -9 -9 25-38 44-44 38-25  9}
{ 5-13 22-30 35-39 44-44 44-44 39-35 30-22 13 -5}

32x32 transform

{32 32 32 32 32 32 32 32 32 32 32 32 32 32 32 32 32 32 32 32 32 32 32 32 32 32 32 32 32 32 32 32}

{46 46 43 42 41 38 36 33 30 28 23 20 15 11  8  3 -3 -8-11-15-20-23-28-30-33-36-38-41-42-43-46-46}

{44 44 39 35 30 22 13  5 -5-13-22-30-35-39-44-44-44-44-39-35-30-22-13 -5  5 13 22 30 35 39 44 44}

{46 41 33 23 11 -3-15-28-36-42-46-43-38-30-20 -8  8 20 30 38 43 46 42 36 28 15  3-11-23-33-41-46}

{44 38 25  9 -9-25-38-44-44-38-25 -9  9 25 38 44 44 38 25  9 -9-25-38-44-44-38-25 -9  9 25 38 44}

{43 33 15 -8-28-41-46-38-23 -3 20 36 46 42 30 11-11-30-42-46-36-20  3 23 38 46 41 28  8-15-33-43}

{44 30  5-22-39-44-35-13 13 35 44 39 22 -5-30-44-44-30 -5 22 39 44 35 13-13-35-44-39-22  5 30 44}

{42 23 -8-33-46-36-11 20 41 43 28 -3-30-46-38-15 15 38 46 30  3-28-43-41-20 11 36 46 33  8-23-42}

{41 19-19-41-41-19 19 41 41 19-19-41-41-19 19 41 41 19-19-41-41-19 19 41 41 19-19-41-41-19 19 41}

{41 11-28-46-30  8 38 42 15-23-46-33  3 36 43 20-20-43-36 -3 33 46 23-15-42-38 -8 30 46 28-11-41}

{39  5-35-44-13 30 44 22-22-44-30 13 44 35 -5-39-39 -5 35 44 13-30-44-22 22 44 30-13-44-35  5 39}

{38 -3-41-36  8 42 33-11-43-30 15 46 28-20-46-23 23 46 20-28-46-15 30 43 11-33-42 -8 36 41  3-38}

{38 -9-44-25 25 44  9-38-38  9 44 25-25-44 -9 38 38 -9-44-25 25 44  9-38-38  9 44 25-25-44 -9 38}

{36-15-46-11 38 33-20-46 -8 41 30-23-43 -3 42 28-28-42  3 43 23-30-41  8 46 20-33-38 11 46 15-36}

{35-22-44  5 44 13-39-30 30 39-13-44 -5 44 22-35-35 22 44 -5-44-13 39 30-30-39 13 44  5-44-22 35}

{33-28-38 20 42-11-46  3 46  8-43-15 41 23-36-30 30 36-23-41 15 43 -8-46 -3 46 11-42-20 38 28-33}

{32-32-32 32 32-32-32 32 32-32-32 32 32-32-32 32 32-32-32 32 32-32-32 32 32-32-32 32 32-32-32 32}

{30-36-23 41 15-43 -8 46 -3-46 11 42-20-38 28 33-33-28 38 20-42-11 46  3-46  8 43-15-41 23 36-30}

{30-39-13 44 -5-44 22 35-35-22 44  5-44 13 39-30-30 39 13-44  5 44-22-35 35 22-44 -5 44-13-39 30}

{28-42 -3 43-23-30 41  8-46 20 33-38-11 46-15-36 36 15-46 11 38-33-20 46 -8-41 30 23-43  3 42-28}

{25-44  9 38-38 -9 44-25-25 44 -9-38 38  9-44 25 25-44  9 38-38 -9 44-25-25 44 -9-38 38  9-44 25}

{23-46 20 28-46 15 30-43 11 33-42  8 36-41  3 38-38 -3 41-36 -8 42-33-11 43-30-15 46-28-20 46-23}

{22-44 30 13-44 35  5-39 39 -5-35 44-13-30 44-22-22 44-30-13 44-35 -5 39-39  5 35-44 13 30-44 22}

{20-43 36 -3-33 46-23-15 42-38  8 30-46 28 11-41 41-11-28 46-30 -8 38-42 15 23-46 33  3-36 43-20}

{19-41 41-19-19 41-41 19 19-41 41-19-19 41-41 19 19-41 41-19-19 41-41 19 19-41 41-19-19 41-41 19}

{15-38 46-30  3 28-43 41-20-11 36-46 33 -8-23 42-42 23  8-33 46-36 11 20-41 43-28 -3 30-46 38-15}

{13-35 44-39 22  5-30 44-44 30 -5-22 39-44 35-13-13 35-44 39-22 -5 30-44 44-30  5 22-39 44-35 13}

{11-30 42-46 36-20 -3 23-38 46-41 28 -8-15 33-43 43-33 15  8-28 41-46 38-23  3 20-36 46-42 30-11}

{ 9-25 38-44 44-38 25 -9 -9 25-38 44-44 38-25  9  9-25 38-44 44-38 25 -9 -9 25-38 44-44 38-25  9}

{ 8-20 30-38 43-46 42-36 28-15  3 11-23 33-41 46-46 41-33 23-11 -3 15-28 36-42 46-43 38-30 20 -8}

{ 5-13 22-30 35-39 44-44 44-44 39-35 30-22 13 -5 -5 13-22 30-35 39-44 44-44 44-39 35-30 22-13  5}

{ 3 -8 11-15 20-23 28-30 33-36 38-41 42-43 46-46 46-46 43-42 41-38 36-33 30-28 23-20 15-11  8 -3}