| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G498 |
| --- | --- |

| Title: | CE8: ALF with low latency and reduced complexity for HEVC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | A. Fuldseth, Cisco Systems G. Bjøntegaard, Cisco Systems | Tel: Email: Tel: Email | +47 92414082 arild.fuldseth@cisco.com gbjonteg@cisco.com |
| Source: | Cisco Systems |  |  |

_____________________________

# Abstract

The document describes a low complexity ALF technique suitable for low latency applications. One single set of ALF filter coefficients are computed quantized and transmitted sequentially for each block using a single pass technique. The proposed ALF also has low complexity by using only a 5x5 diamond shape and no decoder-side variance calculations. When applied to low complexity configurations, BD-rate gains between 1.4 % and 3.7% are reported. When applied to high efficiency configurations, BD-rate losses between 0.5% and 1.1% are reported.

# Introduction

ALF as specified in HM 4.0 brings a coding gain of typically 2-5% measured in BDR.  The operations are mainly performed on a frame basis.  This may lead to additional latency caused by the coding process.  This makes ALF less attractive for low (sub-frame) delay real time applications like video conferencing.  Furthermore ALF represents a considerable implementation complexity.  The present document therefore describes a low complexity ALF version that is useful for low latency applications. A first version of this work was presented in [1].

# Description

ALF is performed for luma only. In the following the main aspects of the tested method are briefly described.

Units for performing ALF

HM 4.0 perform ALF on a frame basis.  According to the present contribution ALF is performed on smaller units. In particular, 64x64 and 64x128 luma pixels have been used for low and high resolutions respectively.  This will be referred to as an ALF-unit.

Pixel matrix

The 5x5 diamond shaped pixel matrix has been used.

Pixel classifications

No pixel classification based on variance classes has been used.  With the smaller ALF-units classification is considered to be less important.

Calculating ALF coefficients

Calculation of covariance data and solving for optimal coefficients are performed in a similar way to HM 4.0.

Quantization of ALF coefficients

The ALF coefficients are quantized more coarsely than in HM 4.0.  This is mainly in order to save coding bits since coefficients may have to be sent for every ALF-unit.  For the 5x5 diamond shape structure 7 distinct ALF coefficients are calculated.  With the coarse quantization, especially the DC response of the filter may suffer.  To overcome this only 6 out of 7 coefficients are quantized and transmitted.  The last coefficient – representing the center position of the filter is calculated to give a final DC response of 1.

Coding of ALF coefficients

The coefficients are encoded using structured CAVLC tables for the magnitude and sign jointly. In addition, the following techniques are used:

- One bit to enable/disable the ALF for an ALF unit.

- One bit to enable/disable the ALF for one row of LCUs.

- Spatial prediction between the coefficients of the current ALF unit and the coefficients of the previous ALF unit being enabled in raster scan order.

- One bit to signal no change in ALF coefficients between current ALF unit and the previous ALF unit having ALF enabled.

Overall procedure of the ALF process

Calculation and quantization of a set of filter coefficient is performed for an ALF-unit.  Then an RD test is made to decide if the filter shall be used.  1 bit is used to signal whether the ALF is enabled or disabled.  If the outcome of the RD test is positive, filtering is performed with dequantized filter coefficients.  No further switch between filters is performed.

# Simulation results

Table 1 shows BD-rate results for the proposed low delay low complexity ALF.

Table 1. BD-rate results for proposed ALF.

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.7 % | 1.9 % | 1.4 % | -2.3 % | 0.4 % | 0.4 % |
| Class B | 0.4 % | 2.7 % | 2.2 % | -1.4 % | 0.5 % | 0.5 % |
| Class C | 0.5 % | 3.8 % | 4.0 % | -0.9 % | 0.6 % | 0.6 % |
| Class D | 0.2 % | 3.1 % | 3.5 % | -0.4 % | 0.3 % | 0.3 % |
| Class E | 0.7 % | 4.5 % | 6.1 % | -1.9 % | 0.9 % | 0.9 % |
| Overall | 0.5 % | 3.1 % | 3.2 % | -1.4 % | 0.5 % | 0.5 % |
|  | 0.5 % | 3.1 % | 3.2 % | -1.3 % | 0.5 % | 0.5 % |
| Enc Time[%] | 66 % |  |  | 106 % |  |  |
| Dec Time[%] | 88 % |  |  | 110 % |  |  |
|  | Random Access HE |  |  | Random Access LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 1.5 % | 7.5 % | 6.5 % | -3.6 % | 1.4 % | 0.7 % |
| Class B | 1.0 % | 4.1 % | 2.6 % | -2.4 % | 0.5 % | 0.5 % |
| Class C | 0.9 % | 2.6 % | 3.0 % | -1.5 % | 0.9 % | 0.9 % |
| Class D | 0.9 % | 2.2 % | 2.5 % | -1.6 % | 0.5 % | 0.4 % |
| Class E |  |  |  |  |  |  |
| Overall | 1.1 % | 4.1 % | 3.6 % | -2.3 % | 0.8 % | 0.6 % |
|  | 1.1 % | 4.0 % | 3.5 % | -2.3 % | 0.8 % | 0.6 % |
| Enc Time[%] | 91 % |  |  | 101 % |  |  |
| Dec Time[%] | 89 % |  |  | 108 % |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 1.1 % | 3.6 % | 2.5 % | -2.2 % | 0.6 % | 0.7 % |
| Class C | 1.0 % | 3.2 % | 3.5 % | -2.0 % | 1.0 % | 0.8 % |
| Class D | 0.7 % | 3.5 % | 3.7 % | -1.1 % | 0.9 % | 1.0 % |
| Class E | 1.3 % | 3.1 % | 3.4 % | -3.1 % | 1.3 % | 0.9 % |
| Overall | 1.0 % | 3.4 % | 3.2 % | -2.0 % | 0.9 % | 0.8 % |
|  | 1.0 % | 3.4 % | 3.2 % | -2.0 % | 0.9 % | 0.8 % |
| Enc Time[%] | 94 % |  |  | 101 % |  |  |
| Dec Time[%] | 91 % |  |  | 108 % |  |  |
|  | Low delay P HE |  |  | Low delay P LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 1.2 % | 3.9 % | 2.6 % | -5.1 % | 0.2 % | -0.3 % |
| Class C | 0.4 % | 2.4 % | 2.7 % | -2.5 % | 0.6 % | 0.7 % |
| Class D | 0.1 % | 2.1 % | 2.1 % | -0.9 % | 0.3 % | 0.2 % |
| Class E | 1.3 % | 4.3 % | 3.8 % | -7.0 % | -0.2 % | 0.4 % |
| Overall | 0.7 % | 3.2 % | 2.7 % | -3.7 % | 0.3 % | 0.2 % |
|  | 0.7 % | 3.2 % | 2.7 % | -3.7 % | 0.3 % | 0.2 % |
| Enc Time[%] | 90 % |  |  | 101 % |  |  |
| Dec Time[%] | 90 % |  |  | 110 % |  |  |

# Conclusions

Results have been presented for a low complexity ALF version that is particularly useful for applications requiring low latency.

# References

- A. Fuldseth and G. Bjøntegaard, “ALF with low latency and reduced complexity for HEVC,” JCTVC-F542, Torino, July, 2011.

# Patent rights declaration(s)

Cisco Systems may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).