| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G501 |
| --- | --- |

| Title: | CABAC with Arithmetic Context Variables |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | K. Sharman, J. Gamei, N. Saunders, P. Silcock Sony BPRL Jays Close, Viables, Basingstoke RG22 4SB, UK | Tel: Email: | +44 1256 355011 karl.sharman@eu.sony.com |
| Source: | Sony Europe Limited |  |  |

_____________________________

# Abstract

This proposal reports on a technique to remove the look-up tables within the CABAC engine, replacing them with arithmetic operations. Claims are made that this could potentially reduce hardware size and complexity even if the Context Variables used are all increased from 7 to 8-bit. Results are reported for two different variants of the system, showing average {Y, U, V} BD-rate changes of {0.0%, 1.1%, 1.1%} and {0.0%, -0.9%, and -1.0%} respectively; encoder and decoder processing times were 101% and 100% respectively for both systems.

# Background

In the HEVC (working draft) CABAC coder, context variables (CVs) are used by the CABAC engine to encode binary values. CVs, in effect, represent the expected probability of the binary value, v, that they are about to encode (or decode), with the expectation held within the 7-bit state of the CV. The state is split into a 6-bit code ‘valueCV-HM4’ and a 1-bit Most Probable Symbol (MPS) field:

if P(v=1)>50%, the MPS=1;

if P(v=0)>50%, the MPS=0;

if P(1)=P(0)=50%, the MPS is either 1 or 0, depending on previous state transitions.

It is important to note that the state of the CV does not directly represent the probability/expectation of the value ‘v’; rather, it is used as an index to a table that determines the split between Least Probable Symbol (LPS) and the Most Probable Symbol (MPS).

After encoding (or decoding) a binary value, the state is updated, to reflect the change in expectation. This is achieved in HM4.0 with tables; if the MPS is encoded/decoded, the table m_aucNextStateMPS is indexed with the current state to give the next state, but if the LPS is encoded/decoded, the table m_aucNextStateLPS is used instead. These tables each have 128 elements, each 7-bits (in hardware) or 8-bits (in software).

The CVs are not transmitted in the bit-stream between encoder and decoder: the encoder and decoder initialize the CVs to the same value, and are updated in the same manner after a CV has been used to encode or decode a value.

CABAC’s coding engine uses an internal variable called m_uiRange that is a 9-bit number, always kept within the range of 256-511. When the number falls below 256, it is ‘renormalized’ (shifted left). The range is an indication of the current fractional scale that is available for the next symbol. i.e., the value m_uiRange should be split into two parts representing the MPS and LPS, with the splitting made according to the probabilities.

When encoding/decoding, the split-point is calculated using the 2D table sm_aucLPSTable, which gives the space allocated to the LPS, uiLPS. The current 6-bit state of a CV is used to access a row in the table; bits {7, 6} of m_uiRange are used to indicate the column (bit 8 is always ‘1’). The MPS takes the split from 0 to "m_uiRange – uiLPS"; the LPS takes the split from "m_uiRange – uiLPS" to m_uiRange.

The use of three tables may not be ideal for hardware applications, especially as “speculation” is required for high throughput (which may lead to multiple instances of the tables). Instead a means to update the context variables and produce the range split with arithmetic operations may be useful. This proposal leads to such a design, where the context variables will contain the actual probability: a potentially useful property explored in a separate proposal (JCTVC-G504 “CABAC with Combined Context Variables”).

# Algorithm

This proposal:

- changes the CVs from a 7-bit state to an 8-bit state,

- replaces the CV table-based update procedure with an arithmetic function,

- replaces the range-split calculation: from a table-based system to another arithmetic function.

In this document, the following methods are demonstrated:

- one common method for the CV updating procedure,

- two methods for the range-split calculation.

## Context Variable updating

### Summary

The CV’s 8-bit state state0 directly represents the probability/expectation of the binary value being 0:

P(0)=state0/256

The adaptation of the CVs state uses the mathematical function “update” on the current state, given the binary value v that was encoded/decoded:

nextState0	= update(state0,v)

The “update” function may be expressed in the following pseudo-code:

Function update(state0,v):
		Let xorVal   =(v==1?255:0)
		Let prob_v   =(state0 XOR xorVal)
		Let increment=Max(1, 13-(((prob_v<<3)-(prob_v))>>7))
		Return Min(251, prob_v+increment) XOR xorVal

This function requires the use of a 4-bit, a 12-bit and an 8-bit adder.

### Derivation of the Update Function

The update method described in 2.1.1 uses the state to model the expectation; therefore, the update equation has to model the updating of the expectation. The update equation was derived from the tables used by HM4.0 CVs, by plotting the current state’s fraction of the range assigned to ‘0’ against the corresponding fraction of the range after encoding a ‘0’.

i.e. calculate the fraction of the current range given to symbol ‘0’ using a CV; update the CV; calculate
      the new fraction given to symbol ‘0’.

The fraction is derived from the fourth column (index 3) of TComCABACTables::sm_aucLPSTable,

i.e.		 TComCABACTables::sm_aucLPSTable[state][3]/480

and the adaptation uses ContextModel::m_aucNextStateMPS[valueCV-HM4] or ContextModel::m_aucNextStateLPS[valueCV-HM4], depending on whether ‘0’ was the LPS or MPS for the CV state.

Figure 1: Current % range of symbol '0' vs. next % range of symbol '0', given '0' was encoded/decoded

This yields an approximate straight line, from (0%, 5%) to (100%,100%), as shown in Figure 1, which is approximately (0,13) to (256,256) for an 8-bit fixed point representation:

i.e.

y=nextState0=f(state0)=(state0 × (256-13) / 256) +13 = state0 + 13 – ((state0×13)>>8)		Eq.1

However, if this equation is used, then the number of steps that a CV goes through from the point where a ‘0’ is least expected to where it is most expected is significantly shorter than in HM4.0, as illustrated below in Table 1.

Considering this, instead the following equation 9 (also illustrated in Table 1) was used:

y=nextState0=f(state0)= state0 + 13 – ((state0×14)>>8)					Eq.2

Note that when state0 becomes 238 or greater, the increment becomes 0, due to integer arithmetic. Therefore, the equation is modified so that the increment cannot be 0, and is also simplied, as follows:

y=nextState0=f(state0)= state0 + Max(1, 13 – ((state0×7)>>7))					Eq.3

Note that the multiplier can be implemented with a subtractor:

y=nextState0=f(state0)= state0 + Max(1, 13 – (((state0<<3)-state0)>>7))			Eq.4

Table 1: CV state transitions from 0 being extremely unlikely to extremely likely

| HM4.0 |  | Arithmetic CVs |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | state0 + 13 – ((state0×13)>>8). |  | state0 + 13 – ((state0×14)>>8). |  |
| State | Fraction | State | Fraction | State | Fraction |
| 62 MPS=1 | 1.88% | 4 | 1.56% | 4 | 1.56% |
| 38 MPS=1 | 6.88% | 17 | 6.64% | 17 | 6.64% |
| 28 MPS=1 | 11.67% | 30 | 11.72% | 30 | 11.72% |
| 22 MPS=1 | 15.83% | 42 | 16.41% | 42 | 16.41% |
| ... | ... | ... | ... | ... | ... |
| 44 MPS=0 | 95.00% | 253 | 98.83% | 239 | 93.36% |
| 45 MPS=0 | 95.21% | 254 | 99.22% | 240 | 93.75% |
| 46 MPS=0 | 95.42% | 255 | 99.61% | 241 | 94.14% |
| 47 MPS=0 | 95.63% | - | - | 242 | 94.53% |
| ... | ... | ... | ... | ... | ... |
| 58 MPS=0 | 97.50% | - | - | 253 | 98.83% |
| 59 MPS=0 | 97.71% | - | - | 254 | 99.22% |
| 60 MPS=0 | 97.71% | - | - | 255 | 99.61% |
| 61 MPS=0 | 97.92% | - | - | - | - |
| 62 MPS=0 | 98.13% | - | - | - | - |

The update process above describes the next probability of 0 (nextState0/256) given the current probability of 0 (state0/256) and the fact that a 0 has been encoded/decoded (v=0). By symmetry, the same process would describe the next probability of 1 (nextState1/256) given the current probability of 1 (state1/256) and the fact that a 1 has been encoded/decoded (v=1). The probability of a 1 is “1-P(0)”, and therefore "state1=256-state0" and "nextState0=256-nextState1".

As an approximation, it can be said that "state1≈255-state0" and "nextState0≈255 - nextState1", allowing the subtractions to be replaced with XORs.

Finally, the probability is clipped so that the range will be at least ‘4’ (the state is in the range of 4 to 251), similar to the limits placed in the HM4.0 tables.

These stages therefore give rise to the “update” function described above in section 2.1.1.

## Range Calculation

### Method 1: Multiplier-based Range Calculation

Calculating the amount of the range (m_uiRange) allocated to the value "v=0", given the current state of the current CV is achieved with a single 9-bit multiplier:

zeroRange=(m_uiRange × state0) >> 8

The amount of the range allocated to the value v=1 is calculated using:

oneRange=m_uiRange-zeroRange

The range is therefore divided into two sections, the first being zeroRange in length, and allocated to the case where v=0, and the second being oneRange in length and allocated to the case where v=1.

### Method 2: Adder-based Range Calculation

Utilizing a multiplier on m_uiRange is not ideal when the throughput of the entropy decoder is considered; it is better if m_uiRange is required for a minimal number of simple operations thereby allowing multiple decodes within a single clock cycle.

In HM4.0, the CV’s state is used to index a row in table ‘sm_aucLPSTable’, giving four potential next range values for the LPS. The choice of the next range value is a simple multiplexer operation utilizing two bits of the current range.

The following method pre-calculates the 4 possible next range values, and then, as in HM4.0, the choice is made later utilizing the same two bits of the current range. The method can be thought of as a multiplier operation, but using a two bit operand and rounding.

In summary, the operation is:

If state0≥128		lps=1, lpFraction=256-state0
	else				lps=0, lpFraction=state0

	candidateLPSRanges[4]={	(lpFraction ×9)>>3,     	(lpFraction ×11)>>3 ,
						(lpFraction ×13)>>3,   	(lpFraction ×15)>>3}
	lpsRange= candidateLPSRanges[m_uiRange bits 7,6]

	As in HM4.0, encode / decode in terms of LPS/MPS.

where the 4 multipliers can be implemented with adders/subtractors – and only 4 adders would be required to calculate all 4 candidate entries.

#### Theory

As in HM4.0, the first of the four values will be utilized when the current range, m_uiRange, is between 256 (0b100000000) and 319 (0b100111111) (inclusive), i.e. on average 288 (0b100100000), where the two bits that are used for the choice of result are highlighted.

The second of the four values will be utilized when m_uiRange is between 320 and (0b101000000) and 383 (0b101111111) (inclusive), i.e. on average 352 (0b101100000).

Similarly, the third and the fourth values will be used when, on average m_uiRange 416 and 480 respectively. The four candidate values are therefore calculated as the current probability (state0) multiplied by the 4 average m_uiRange values:

i.e.

candidateZeroRange[4]={(state0 × 288)>>8, (state0 × 352)>>8, (state0 × 416)>>8, (state0 × 480)>>8}

The 4 multipliers can be implemented as 4 adders, and since the 5 LSBs of each of the constant multiplicands are 0, the multiplicands and shifts can be reduced.

Once the value of m_uiRange is known, the zero range can be selected from the four candidates, and the process could proceed as with the multiplier-based range method.

However, as an expected m_uiRange value is used, it is possible for the entries to exceed the actual m_uiRange value. e.g. if "state0 = 240" (P(0)=93.75%), then "candidateZeroRange[0] = 270". However, if m_uiRange was a value between 256 and 269 (inclusive), then the resulting zero range would exceed the total available range. One method to alleviate the problem is to clip the resulting zeroRange value, ensuring that there is adequate oneRange. However, it is better if a least-probable-symbol mechanism is employed, as this will in effect average out the errors caused by the truncation. Therefore, if "state0≥128" (P(0)≥50%), the candidate range values will be the ‘one’ ranges:

candidateOneRange[4]={(state1 × 288)>>8, (state1 × 352)>>8, (state1 × 416)>>8, (state1 × 480)>>8}

where state1 = 256 – state0.

Therefore, this system returns to essentially the same as HM4.0, except with arithmetic updating instead of tables: a LPS is identified (test on bit-7 of the state), and the next LPS range is selected (using two bits from the m_uiRange) from 4 candidate LPS ranges. The MPS can take the first section of the range, and the LPS can take the second section of the range.

# Discussion

In the HM4.0 Encoder, the RDOQ table has 128 table entries, corresponding to each of the 128 states of a CV. However, the CVs have been extended to 256 possible states, and therefore the RDOQ table has 256 table entries, by default. It was noticed that this new table can be reduced to 128 entries, and indexed using the 7 MSBs of the state, without having any noticeable effect on the coding efficiency.

In HM4.0, initialization of the CVs uses a table of values and the target QP to linearly interpolate:

CV_initial_state=f(mCVQp+cCV)

Two methods have been investigated for initializing arithmetically updated CVs, those being:

- The conversion of the CV_initial_state, by using the same stateHM4 to probability curve as described earlier.

- The conversion of mCV and cCV table values, by fitting a straight line through a curve defined by the first method at QP=22, 27, 32 and 37.

There is no significant difference in terms of coding efficiency between the two methods; however, the latter would be simpler for implementation, and is the chosen configuration for the simulated system.

It should be noted that the arithmetic values can be pre-calculated and stored in tables, if required, resulting in a similar system to HM4.0.

Finally, although the replacement of look-up tables (used to update the CVs and calculate the range allocations) has the potential to reduce hardware size and simplify the design, there is potentially a cost for hardware by increasing the CV state from 7 to 8 bits. However, often internal RAMs, which are likely to be used to store the CVs, are in multiples of 4-bits internally, and therefore this increase from 7 to 8-bits may be less critical.

# Simulation Results

Simulations for systems using a multiply-based range calculation and an adder-based range calculation are shown below, comparing against HM4.0.

In all cases, the C model used a single 256-element pre-calculated table for the CV update procedure for state0, using the approximation of “state1 = 255 - state0” when a ‘1’ was encoded.

In addition, a set of results of the adder-based method that illustrates the effects of reducing the size of the RDOQ table to128 elements is given:

We would like to thank Samsung for cross-checking our results (JCTVC-G934).

# Conclusion

An alternative context variable is proposed, which, although the number of bits is increased from 7 to 8, removes the requirement of look-up tables, replacing them with arithmetic.

Two systems are described, with both using the same 3-adder update system.

The first system utilizes a multiplier on the CABAC range, and the encoder and decoder processing times are, to within error, the same as HM4.0. Results show that this system has an average BD-rate change of 0.0%, -1.1%, and -1.1% for Y, U and V respectively.

The second system utilizes 5 adders without requiring CABAC range, and then a multiplexer selects one of four values. It is believed that this system is capable of higher throughput than the first, although there is an increase in encoder processing time of 1%. Results show that this system has a coding efficiency change of 0.0%, -0.9%, and -1.0% for Y, U and V respectively.

Advantages of a CV that directly indicates probability are explored in contribution JCTVC-G504 “CABAC with Combined Context Variables”.

# Patent rights declaration(s)

Sony Corporation or Sony Group companies may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).