| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G503 |
| --- | --- |

| Title: | CE4 Subtest 3.2: Crosscheck report of compact representation of quantization matrices proposed by TI (JCTVC-G083) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Masato Shima Canon Inc. 30-2, Shimomaruko 3-chome, Ohta-ku, Tokyo 146-8501, Japan | Tel: Email: | +81-3-3758-2111 shima.masato@canon.co.jp |
| Source: | Canon Inc. |  |  |

_____________________________

# Abstract

This contribution contains the cross check result of CE4 Subtest 3.2 (JCTVC-G083/F085), compact representation of quantization matrices.

# Cross Check Procedure

The source code corresponds to CE4 Subtest 3.2 was provided by TI, and it was compiled with GCC 3.4.6.  All the simulation was done on a Linux cluster.  The results of CE4 Subtest 3.1 (reported as in JCTVC-G434, cross checked as in JCTVC-G502) were used as reference data.  Both symmetry and asymmetry quantization matrices are used to measure the performance of proposed method with Test 1 and Test 2 configuration defined in JCTVC-F904.

In order to generate the results of the proposed method, the following settings are added to the configuration file:

QMatxCompressionOption0      : 1                 # Quantization Matrix Compression Option for 4x4

QMatxCompressionOption1      : 1                 # Quantization Matrix Compression Option for 8x8

QMatxCompressionOption2      : 1                 # Quantization Matrix Compression Option for 16x16

QMatxCompressionOption3      : 1                 # Quantization Matrix Compression Option for 32x32

# Results

## Results of Test 1: Results of compression performance of matrix itself

Cross check results of Test 1 are summarized in Table 1, where proposed method is compared with AVC based coding method.

|  |  |  |  | Symmetry |  |  | Asymmetry |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Options |  |  |  | bits | comp ratio | average error | bits | comp ratio | average error |
| down S. | UEK | sym45 | sym135 |  |  |  |  |  |  |
| 0 | 0 | 0 | 0 | 8856 | 1 | 0 | 9512 | 1 | 0 |
| 0 | 0 | 0 | 1 | 4864 | 1.82 | 0 | 4894 | 1.94 | 3.53 |
| 0 | 0 | 1 | 0 | 5052 | 1.75 | 5.06 | 4558 | 2.09 | 3.14 |
| 0 | 0 | 1 | 1 | 2852 | 3.11 | 5.06 | 2626 | 3.62 | 5.25 |
| 0 | 1 | 0 | 0 | 3390 | 2.61 | 3.78 | 3442 | 2.76 | 6.68 |
| 0 | 1 | 0 | 1 | 2102 | 4.21 | 3.53 | 2126 | 4.47 | 6.83 |
| 0 | 1 | 1 | 0 | 2012 | 4.4 | 7.91 | 1968 | 4.83 | 6.9 |
| 0 | 1 | 1 | 1 | 1352 | 6.55 | 7.52 | 1286 | 7.4 | 6.95 |
| 1 | 0 | 0 | 0 | 3631 | 2.44 | 0.43 | 3784 | 2.51 | 0.32 |
| 1 | 0 | 0 | 1 | 2137 | 4.14 | 0.43 | 2094 | 4.54 | 3.76 |
| 1 | 0 | 1 | 0 | 2263 | 3.91 | 4.73 | 1923 | 4.95 | 3.54 |
| 1 | 0 | 1 | 1 | 1399 | 6.33 | 4.73 | 1249 | 7.62 | 5.52 |
| 1 | 1 | 0 | 0 | 1391 | 6.37 | 3.95 | 1380 | 6.89 | 6.35 |
| 1 | 1 | 0 | 1 | 1001 | 8.85 | 3.47 | 998 | 9.53 | 6.65 |
| 1 | 1 | 1 | 0 | 933 | 9.49 | 7.28 | 905 | 10.51 | 6.81 |
| 1 | 1 | 1 | 1 | 774 | 11.44 | 6.69 | 693 | 13.73 | 7.07 |

Table 1: Test 1 results of proposed method vs. anchor AVC method

## Results of Test 2: Impact on overall HM performance

Cross check results with symmetry quantization matrices are summarized in Table 2, where proposed method is compared with AVC based coding method.

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -1.4% | -1.2% | -1.2% | -1.3% | -1.1% | -1.1% |
| Class B | -3.0% | -2.6% | -2.5% | -2.7% | -2.3% | -2.3% |
| Class C | -7.2% | -6.6% | -6.7% | -6.7% | -6.2% | -6.2% |
| Class D | -15.5% | -14.6% | -14.7% | -14.8% | -14.0% | -14.1% |
| Class E | -8.4% | -7.7% | -7.7% | -7.5% | -6.9% | -7.0% |
| Class F |  |  |  |  |  |  |
| Overall | -6.8% | -6.3% | -6.3% | -6.3% | -5.9% | -5.9% |
|  | -6.8% | -6.3% | -6.3% | -6.3% | -5.9% | -5.9% |
| Enc Time[%] | 101% |  |  | 101% |  |  |
| Dec Time[%] | 102% |  |  | 100% |  |  |
|  | Random Access HE |  |  | Random Access LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A | -8.9% | -8.2% | -8.2% | -8.3% | -7.6% | -7.4% |
| Class B | -16.4% | -15.0% | -14.4% | -15.5% | -14.1% | -13.6% |
| Class C | -24.9% | -23.9% | -23.9% | -24.3% | -23.4% | -23.4% |
| Class D | -34.8% | -34.1% | -34.2% | -34.5% | -33.9% | -34.0% |
| Class E |  |  |  |  |  |  |
| Class F |  |  |  |  |  |  |
| Overall | -21.0% | -20.0% | -19.9% | -20.4% | -19.4% | -19.2% |
|  | -21.0% | -20.0% | -19.9% | -20.4% | -19.4% | -19.3% |
| Enc Time[%] | 100% |  |  | 101% |  |  |
| Dec Time[%] | 102% |  |  | 103% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -17.1% | -15.3% | -14.8% | -16.3% | -14.5% | -14.2% |
| Class C | -24.9% | -23.6% | -23.6% | -24.3% | -23.2% | -23.3% |
| Class D | -34.5% | -33.7% | -33.8% | -34.3% | -33.4% | -33.6% |
| Class E | -36.2% | -35.2% | -35.2% | -35.5% | -34.4% | -34.7% |
| Class F |  |  |  |  |  |  |
| Overall | -27.0% | -25.7% | -25.6% | -26.4% | -25.1% | -25.2% |
|  | -27.0% | -25.7% | -25.6% | -26.4% | -25.1% | -25.2% |
| Enc Time[%] | 102% |  |  | 101% |  |  |
| Dec Time[%] | 104% |  |  | 103% |  |  |
|  | Low delay P HE |  |  | Low delay P LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -16.7% | -14.9% | -14.4% | -15.7% | -14.0% | -13.6% |
| Class C | -24.5% | -23.2% | -23.2% | -23.9% | -22.8% | -22.8% |
| Class D | -34.2% | -33.3% | -33.4% | -34.0% | -33.1% | -33.2% |
| Class E | -35.9% | -34.8% | -34.9% | -35.1% | -34.1% | -34.3% |
| Class F |  |  |  |  |  |  |
| Overall | -26.6% | -25.3% | -25.2% | -26.0% | -24.7% | -24.7% |
|  | -26.6% | -25.3% | -25.2% | -26.0% | -24.7% | -24.7% |
| Enc Time[%] | 103% |  |  | 102% |  |  |
| Dec Time[%] | 105% |  |  | 105% |  |  |

Table 2: BD-rate results for proposed method vs. anchor AVC based method with symmetry QM

Cross check results with asymmetry quantization matrices are summarized in Table 3, where proposed method is compared with AVC based coding method.

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.2% | -2.7% | -1.5% | -1.3% | -5.0% | -3.6% |
| Class B | 0.5% | -5.4% | -5.3% | 0.2% | -5.9% | -6.5% |
| Class C | -3.1% | -9.3% | -9.4% | -2.5% | -8.6% | -8.8% |
| Class D | -13.3% | -17.4% | -17.2% | -12.0% | -16.5% | -16.3% |
| Class E | -8.1% | -9.4% | -9.3% | -7.1% | -9.5% | -9.4% |
| Class F |  |  |  |  |  |  |
| Overall | -4.4% | -8.6% | -8.4% | -4.2% | -8.9% | -8.8% |
|  | -4.4% | -8.6% | -8.3% | -4.2% | -8.9% | -8.7% |
| Enc Time[%] | 99% |  |  | 97% |  |  |
| Dec Time[%] | 101% |  |  | 99% |  |  |
|  | Random Access HE |  |  | Random Access LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A | -8.6% | -8.3% | -5.3% | -8.5% | -7.6% | -3.6% |
| Class B | -15.5% | -15.2% | -15.2% | -14.5% | -14.8% | -15.1% |
| Class C | -24.3% | -25.1% | -25.3% | -23.5% | -24.4% | -24.6% |
| Class D | -35.9% | -36.0% | -35.9% | -35.4% | -35.6% | -35.6% |
| Class E |  |  |  |  |  |  |
| Class F |  |  |  |  |  |  |
| Overall | -20.7% | -20.8% | -20.1% | -20.1% | -20.3% | -19.5% |
|  | -20.8% | -20.8% | -20.1% | -20.1% | -20.2% | -19.4% |
| Enc Time[%] | 101% |  |  | 100% |  |  |
| Dec Time[%] | 104% |  |  | 102% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -16.5% | -14.8% | -14.7% | -16.0% | -14.0% | -13.6% |
| Class C | -24.7% | -24.4% | -24.5% | -24.0% | -23.9% | -23.9% |
| Class D | -35.8% | -35.4% | -35.3% | -35.4% | -35.0% | -34.8% |
| Class E | -38.1% | -37.2% | -37.0% | -37.3% | -36.1% | -36.5% |
| Class F |  |  |  |  |  |  |
| Overall | -27.4% | -26.5% | -26.5% | -26.9% | -25.9% | -25.8% |
|  | -27.4% | -26.5% | -26.5% | -26.9% | -25.9% | -25.8% |
| Enc Time[%] | 100% |  |  | 101% |  |  |
| Dec Time[%] | 102% |  |  | 103% |  |  |
|  | Low delay P HE |  |  | Low delay P LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -15.9% | -14.4% | -14.5% | -15.5% | -13.4% | -13.2% |
| Class C | -24.2% | -24.1% | -24.2% | -23.5% | -23.4% | -23.5% |
| Class D | -35.3% | -34.9% | -35.0% | -35.0% | -34.6% | -34.4% |
| Class E | -37.6% | -36.7% | -36.6% | -36.8% | -35.8% | -36.1% |
| Class F |  |  |  |  |  |  |
| Overall | -26.9% | -26.1% | -26.2% | -26.4% | -25.4% | -25.4% |
|  | -26.9% | -26.1% | -26.2% | -26.4% | -25.4% | -25.4% |
| Enc Time[%] | 100% |  |  | 98% |  |  |
| Dec Time[%] | 103% |  |  | 102% |  |  |

Table 3: BD-rate results for proposed method vs. anchor AVC based method with asymmetry QM

# Conclusion

It has been verified that the bitrate and PSNR results of Test 1 and option 1 of Test 2 provided by TI on JCTVC-G083 are accurate.