| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G504 |
| --- | --- |

| Title: | CABAC with Combined Context Variables |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | K. Sharman, J. Gamei, N. Saunders, P. Silcock Sony BPRL Jays Close, Viables, Basingstoke RG22 4SB, UK | Tel: Email: | +44 1256 355011 karl.sharman@eu.sony.com |
| Source: | Sony Europe Limited |  |  |

_____________________________

# Abstract

This proposal presents an adaption of the CABAC entropy coder that is claimed to be able to decode multiple coefficients per clock and therefore capable of decoding even the sustained worst case for 4:4:4 data.

The reported method encodes or decodes all of the required CABAC context variables (CV) for each coefficient in a single cycle using a quaternary arithmetic coding function; the bypass (sign and escape codes) are not explicitly discussed in this report, although a method whereby the bypass data can be decoded in parallel to the CV CABAC data is assumed, such as claimed in JCTVC-G494 “CABAC Packet-based Stream”.

The reported method encodes or decodes coefficients in a single pass of the data. Problems regarding the speculation required to maintain the throughput are discussed, and it is indicated that with changes to the CV selection system, speculation for large blocks (16x16, 32x32) can be completely removed. For small blocks (4x4 and 8x8), two methods of CV selection are presented that reduce speculation to 2 levels and 3 levels respectively (that is, a system with 2 levels of speculation would prepare 2 possible values per coefficient, requiring less than 2x the amount of logic than that of a system with no speculation). Results are presented that indicate average BD-rate changes of +0.1%, -1.1% and -1.2% for Y, U and V respectively for the 3x speculation system, and average BD-rate changes of +0.2%, -1.0% and -1.1% for Y, U and V respectively for the 2x speculation system. In both systems, the context variables have not been re-optimized/adjusted from the values in HM4.0.

In addition, results are presented for an unoptimized packet-based system with 2 levels of speculation that indicates average BD-rate changes of 0.2%, -1.0% and -1.1% for Y, U and V respectively; the software times have increased by 3% and 0.5% for the encoder and decoder, with increases believed to be caused by the deferred ACV update mechanism and the neighbourhood-based method to select context variables ACV1 and ACV2, both designed to reduce the speculation requirements in hardware.

# Introduction

In HM4.0, the process of encoding data is as follows:

- Determine and encode the coordinates of the first coefficient in the reverse scan order.

- Encode significance map (sig-map) using reverse coefficient scan order, starting at the coefficient after the first coefficient.

- Encode a ‘magnitude is >1’ map using reverse coefficient scan order, starting at the first coefficient, considering only coefficients that were significant.

- Encode a ‘magnitude is >2’ map using reverse coefficient scan order, starting at the first coefficient, considering only coefficients that whose ‘magnitude is >1’.

- Encode a sign bit for all values where the sig-map is non-zero, in reverse coefficient scan order.

- Encode an escape code (Golomb-Rice and possibly an Exponential-Golomb code) for all coefficients where the ‘magnitude is >2’ map is non-zero, in reverse coefficient scan order.

The HM4.0 CABAC engine encodes a binary value in combination with a context variable (CV), where the CV is selected from a set using various criteria.

The HM4.0 CABAC engine requires multiple passes of the data, and may require 3 context variables (for the three maps) to be encoded per coefficient, as well as bypass data (sign and escape codes). Designing an encoder and decoder that can cope with the sustained worst case for 4:2:0, let alone 4:4:4 data is a challenge, making the CABAC engine a potential bottleneck in the HEVC coder.

Building upon the proposal to use Arithmetic Context Variables (ACVs) (JCTVC-G501 “CABAC with Arithmetic Context Variables”) (where the 8-bit context variables state the probability of a symbol being a ‘0’), the technique described in this proposal is designed to encode/decode one coefficient in one step, and therefore in one pass of the Transform Unit (TU) data.

Increasing the system to encode/decode up to 4 coefficients per clock-cycle is believed to be potentially viable, allowing the sustained worst case 4:4:4 to be supported. The problems of speculation in a parallel hardware system are discussed, and finally, the results of various configurations of the system are presented.

# Core Algorithm

## Quaternary Symbol Encoding

In HM4.0, ignoring the order of steps 2 to 4 in the Introduction, each coefficient is encoded into the CABAC stream with up to 3 CVs. Building upon the Arithmetic Context Variables (ACVs) proposal JCTVC-G501 “CABAC with Arithmetic Context Variables”, these 3 CVs can be thought of as probabilities of the magnitude, m, of the coefficient:

P(m=0)

P(m=1 | m>0)

P(m=2 | m>1)

From there, the probability of the four possible cases being encoded can be produced:

P(m=0) = P(m=0)

P(m=1) = (1-P(m=0)) (P(m=1 | m>0)

P(m=2) = (1-P(m=0)-P(m=1)) P(m=2 | m>2)

P(m>2) = (1-P(m=0)-P(m=1)-P(m=2))

The probability space is split into 4 symbols, which is then represented in an 8-bit integer 'fractional' space, as illustrated below:

As described in proposal JCTVC-G501 “CABAC with Arithmetic Context Variables”, the proportion of the CABAC range that a symbol is allocated is proportional to the probability of a symbol; the CABAC ‘low’ part of the range is incremented by the cumulative of the allocation of ranges of the ‘preceding’ symbols.

Given the three 8-bit arithmetic context variables (ACV0, ACV1, ACV2) (where ACV0=256 P(m=0)) that could potentially encode a symbol, the proposed Combined Context Variable (CCV) encoding/decoding process is as follows:

- Form the fractional space F0,F1,F2,F3 (see below):

F0=ACV0

F1=(256-F0) ×(ACV1) >> 8

F2=(256-F0-F1) ×(ACV2) >> 8

F3=(256-F0-F1-F2)

- Calculate the cumulative fractional (CF) space:

CF0=F0

CF1=F1+CF0

CF2=F2+CF1

- Multiply the cumulative fractions against the current 9-bit CABAC range, to get cumulative ranges (CR):

CR0=CF0 × CABACrange

CR1=CF1 × CABACrange

CR2=CF2 × CABACrange

- Encode/Decode symbol:

To encode,

If (m=0), increase CABAClow by 0, set CABACrange to CR0

If (m=1), increase CABAClow by CR0, set CABACrange to CR1-CR0

If (m=2), increase CABAClow by CR1, set CABACrange to CR2-CR1

If (m=3), increase CABAClow by CR2, set CABACrange to CABACrange-CR2

Then, renormalize.

To decode,

If CABACvalue < CR0,	m=0, decrement CABACvalue by 0

else if CABACvalue < CR1, 	m=1, decrement CABACvalue by CR0

else if CABACvalue < CR2, 	m=2, decrement CABACvalue by CR1

else 			m=3, decrement CABACvalue by CR2

In all cases, CABACrange is updated as for encoding, and renormalization occurs.

Note that with integer arithmetic the initial fractions, F1, F2 or F3, may be 0, and therefore this must be prevented as all possible symbols must have a space in the fractional space. Therefore the cumulative range of CF0 is clipped to a maximum of “256-3”  (this test is not required given that ACVs cannot exceed 251); the cumulative range of CF1 is clipped to “256-2”; the cumulative range of CF2 is clipped to “256-1”.

## Quaternary Symbol Encoding Without Multipliers

As with the ACV proposal JCTVC-G501 “CABAC with Arithmetic Context Variables”, the above technique requires multiplies on CABACrange, which would possibly impact the ability to decode multiple quaternary symbols in one clock cycle. Therefore, an adder-based system is recommended, which again needs to consider the Most Probable Symbol mechanism, thereby averaging out the errors in the system.

However, instead of searching for the Most Probable Symbol (MPS), it is only necessary to identify a symbol that has at least 25% of the symbol range. i.e., the first of the fractions F0, F1, F2 and F3 that is at least 64 is declared as the ‘First Most Probable Symbol’ (FMPS).

The processing steps then become:

- Form the fractional space F0,F1,F2,F3 as before, except preventing 0 length symbols:

F0=ACV0

F1=Max(1, (256-F0) ×(ACV1) >> 8)

If (F1+F0 >=254)

F1=254-F0; F2=1

else

F2=Max(1, (256-F0-F1) ×(ACV2) >> 8)

If (F2+F1+F0 > 255) F2=255-F1-F0

F3=Max(1, (256-F0-F1-F2) )

- Calculate the FMPS:

If F0>=256/4	FMPS=0

Else if F1>=256/4	FMPS=1

Else if F2>=256/4	FMPS=2

Else  		FMPS=3

- Calculate the Cumulative Fractions (CFs) and a Vector of potential Cumulative Ranges (VCRs)

CF0=(FMPS==0?0:F0)		VCR0=(CF0.[9,11,13,15])>>3

CF1=CF0+(FMPS==1?0:F1)	 	VCR1=(CF1.[9,11,13,15])>>3

CF2=CF1+(FMPS==2?0:F2)	 	VCR2=(CF2.[9,11,13,15])>>3

CF3=CF2+(FMPS==3?0:F3) 		VCR3=(CF3.[9,11,13,15])>>3

Note that the vector multiplies used for the VCRs can be done with 4 adders each.

- Select 4 cumulative ranges, one from each vector, given 2 bits of CABACrange.

For a 9-bit CABACrange system, use bits 7,6 to select (bit 8 is always 1).

- Encode/Decode symbol

To encode,

If (m=FPMS), increase CABAClow by CR3, set CABACrange to CABACrange-CR3

Else If (m=0), increase CABAClow by 0, set CABACrange to CR0

Else If (m=1), increase CABAClow by CR0, set CABACrange to CR1-CR0

Else If (m=2), increase CABAClow by CR1, set CABACrange to CR2-CR1

Else If (m=3), increase CABAClow by CR2, set CABACrange to CR3-CR2

Then, renormalize.

To decode,

If CABACvalue < CR0,	m=0, decrement CABACvalue by 0

else if CABACvalue < CR1, 	m=1, decrement CABACvalue by CR0

else if CABACvalue < CR2, 	m=2, decrement CABACvalue by CR1

else if CABACvalue < CR3, 	m=3, decrement CABACvalue by CR2

else 			m=FMPS, decrement CABACvalue by CR3

In all cases, CABACrange is updated as for encoding, and renormalization occurs.

This system is illustrated in Figure 1, which also indicates the processing delays at various points in the processing. It should be noted that the context variable CV0 could be expressed as the probability of ‘1’ rather than ‘0’, thereby reducing the delay of the critical path. Depending on implementation, this may produce/require a different bit-stream.

## Accuracy of the Fractions

Combining the ACVs while keeping a 9-bit CABACrange system (8-bit fractional space) reduces the accuracy of the system. For this reason, the proposed system is actually an 11-bit CABACrange system (10-bit fractional space, but still with 8-bit ACVs). The simulation results for a 9 and 11-bit CABACrange system are shown in Section 4 for comparative purposes.

Figure 1: Illustrative 11-bit CCV system

## Bypass Data

The context variable data only fully describes magnitudes that are less than 3 – to fully describe larger magnitudes escape data is required, in addition to sign bits. The final proposed system is that bypass data (escape and sign data) is placed in the bit-stream so that it can be decoded in parallel with the associated context variable data for a specific coefficient. Proposal JCTVC-G494 “CABAC Packet-based Stream” is one such method by which this can be achieved.

## Transform Unit Coding

Using the quaternary symbol coding algorithm (without multipliers), the proposed system would be:

Reverse scan the entire block,

In the encoder, find the first non-zero coefficient, and encode the coordinate as in HM4.0; in the decoder, decode the coordinates, and check if that position has been reached.

For each subsequent coefficient, determine the 3 context variables that HM4.0 would have potentially have used to encode that coefficient.

Use the quaternary encoding method to encode the CABAC data.

Decode/encode the bypass data using proposal JCTVC-G494 “CABAC Packet-based Stream”.

# Speculation

For a 4:4:4 system, a minimum of 3 coefficients must be decoded per pixel rate clock cycle in order to cope with the sustained worst case. Similarly for 4:2:2 and 4:2:0 systems, the minimum must be 2 and 1.5 coefficients per pixel rate clock cycle respectively. The system shown in Figure 1 illustrates the decoding of a single coefficient, and therefore if multiple coefficients need to be decoded, this effectively increases the size of the logic by the same multiple: the preparation phase of the VCRs would be done in parallel; the decoding phase would be done sequentially.

However, decoding coefficient N may impact decoding coefficient N+1, and if a system based upon Figure 1 was required to decode 3 coefficients in parallel, when decoding coefficient N, coefficient N+5 is being prepared: interactions between the coefficients mean that speculation on the result of coefficient N, N+1, N+2, N+3 and N+4 must be taken into account when preparing coefficient N+5 for decode. With each value have 4 different outcomes, this would be impractical to implement, as it has 45 possibilities.

Therefore, a number of methods have been sought to reduce speculation, and indeed, a method to completely remove speculation for larger blocks (16x16 and 32x32) will now be described, leaving only a maximum of 3 levels of speculation for smaller blocks – i.e. a system where 3 different values per coefficient are prepared, and therefore only 3x the logic of the base system.

## Deferred Update

One method to reduce speculation is to defer the updating of the ACVs until a later point, and therefore the update that an ACV undergoes does not affect any of the coefficients being decoded/prepared up until the update point. The ACV selection (as in HM4.0) is based around a 16-coefficient group, and therefore waiting until the group of 16 is fully encoded/decoded seems to be a practical opportunity to update the context variables; deferring the update may also have the benefit of ‘smoothing’ out the updates caused by high-frequency noise in the data. (It may be possible, as discussed in the previous JCT-VC meeting in Turin, to defer the update until the end of the TU itself.)

## Speculation on ACV0

As indicated above, a 4:2:0, 4:2:2 and 4:4:4 system needs to decode at least 1.5, 2 and 3 coefficients per cycle respectively. Consider instead decoding 2, 3 and 4 coefficients per clock cycle respectively for each system. This would result in a system where it is possible to force a break in the processing every 16 coefficients. The breaks would allow the pipeline to be flushed so that the update in context variables can occur, along with changes to other states of the context selection (such as the context sets).

For smaller blocks (8x8 and 4x4), the ACV0 context variable is a function based solely upon its position in the block, and therefore not dependent on the data. With 8x8 blocks, 4 pixels in the block can update the same ACV0, but with the deferred update system, the shared use of the ACV0 does not impede the decoding of any coefficients within the same group of 16.

For larger blocks, it is important to note that the system of decoding 2, 3 or 4 coefficients per clock for the 4:2:0, 4:2:2 and 4:4:4 systems respectively, even with flushes every 16 coefficients, would also result in some spare processing cycles being available. These can be used to insert ‘pauses’ into the processing so that coefficients that are “below or to the right” of the current coefficient are sufficiently old that they will not affect the current decode.

Therefore, increasing the levels of parallelism along with the “deferred update” system can potentially remove the requirement of speculation on ACV0. This is illustrated in Figure 2, for 4:2:0, Figure 3 for 4:2:2 and Figure 4 for 4:4:4, which suggest the possible clock cycles at which the coefficients can be  decoded and indicates the ‘youngest age’ of the neighbours required for the ACV0 context model selection.

In these three scenarios, a gap in the processing is enforced every 16 coefficients (2 clock cycles for 4:2:0; 1 clock cycle for 4:2:2 and 4:4:4), meaning that the values that are being ‘prepared’ can be ignored. In the three diagrams, the youngest ‘age’ of the neighbours is always at least 2 clock cycles, and therefore, the values being prepared will not be dependent on the values being currently decoded. i.e. the 2 clock cycle process of Figure 1 would not require speculation on ACV0 for the calculation.

Note that the values highlighted in the “Youngest age” section of the figures use location based context variables, and are therefore not relevant to the neighbourhood study.

In addition, for the sustained worst case, the 4:2:0, 4:2:2 and 4:4:4 systems require 164, 125 and 91 cycles to decode all the coefficients, and at pixel rate, the 256 coefficients of a 16x16 block would need to be completed in 256/1.5=170, 256/2=128 and 256/3=85 clocks.

i.e.

For 4:2:0, a system decoding 2 coefficients per clock can decoder faster than pixel rate with limited speculation: 164 clock cycles are required to decode 256 coefficients, with 170 pixel-rate clock cycles available.

For 4:2:2, a system decoding 3 coefficients per clock can decode faster than pixel rate with limited speculation: 125 clock cycles are required to decode 256 coefficients, with 128 pixel-rate clock cycles available.

For 4:4:4, a system decoding 4 coefficients per clock cannot decode faster than pixel rate with limited speculation: 91 clock cycles are required to decode 256 coefficients, with 85 pixel-rate clock cycles available. In this case, the clock rate would have to be 107% (91/85) that of pixel rate.

Figure 2: Coefficient processing times for an illustrative 4:2:0 system

Figure 3: Coefficient processing times for an illustrative 4:2:2 system

Figure 4: Coefficient processing times for an illustrative 4:4:4 system

## Use of Neighbourhood for ACV1 and ACV2 Selection

In HM4.0, selection of context variables ‘ACV1’ and ‘ACV2’ requires details of the previously decoded coefficients: ACV1 is selected based upon the length of the end run of ‘1’s in the current group of 16; the choice of ACV2 is changed each time another value whose magnitude is greater than 1 is encountered. For each group of 16 coefficients, there are only 8 valid combinations of ACV1 and ACV2.

More formally, that is to say (in HM4.0) there is a superset of ACVs for ACV1 and a superset for ACV2 from which for any group of 16 coefficients, a subset of 5 ACVs for ACV1 and a subset of 5 ACVs for ACV2 are chosen. This selection is based upon the slice and channel type, and a function of the number of values whose ‘magnitude >1’ (the ‘uiNumOne’ scheme) decoded prior to the current group of 16 coefficients.

In HM4.0, the variable ‘c1’ counts the number of trailing magnitude 1’s in the current group of 16 coefficients (noting that c1 is initialised to ‘1’ and is clipped to a maximum of 4); the variable ‘c2’ counts the number of values in the group whose ‘magnitude is >1’ (where c2 is initialised to ‘0’ and clipped to a maximum of 4). Then for each coefficient encode/decode:

ACV1 	= ACV_set1[0] 		if c2!=0
				= ACV_set1[c1] 	if c2=0

ACV2	= ACV_set2[c2]

There are only 8 possible pairings for ACV1 and ACV2 given the two limited sized subsets.

An alternative scheme is to use the same coefficient neighbourhood as in the significance map, counting the number of magnitude 1’s (and starting at ‘c1=1’, as for HM4.0) and ‘magnitude is >1’ (starting with ‘c2=0’). Both counters are limited to 4, and if c2 is non-zero after checking the neighbourhood, c1 is set to 0. Note that the scheme still uses the HM4.0 ‘uiNumOne’ scheme, which affects the subset of ACVs on each 16 coefficient boundary. (The neighbourhood is defined as the two coefficients to the right, the two coefficients below, and the coefficient below and to the right; if the neighbour coefficient is outside the TU, it is assumed to be 0.)

The mechanism of using the same neighbourhood as the significance map (ACV0) therefore removes speculation on ACV1 and ACV2 for larger blocks, since the neighbouring values are sufficiently ‘old’. However, note that in Figure 2, Figure 3 and Figure 4, the highlighted top-left (low frequency) areas of the ‘youngest age’ indicate position-based context variables; no such rule has been applied for ACV1 and ACV2, and the three top left values use the same rule of context variable selection as the other coefficients. This increases the number of clock cycles to process the data to 166, 128 and 95 for the 3 described systems, potentially increasing the system clock rate to at least 111% of pixel rate for the 4:4:4 scenario.

For smaller blocks, a neighbourhood system does not reduce speculation, because there are not enough spare cycles available to ensure the coefficients below or to the right are sufficiently old. This would therefore leave 8 possible pairings for ACV1 and ACV2. i.e. until the decoding phase, the selection of c1 and c2 would not be known, and therefore it would be necessary to prepare the VCRs for the coefficients for all 8 combinations.

Therefore, for smaller blocks (i.e. 4x4 and 8x8), a new superset of ACVs is used, from which a subset is picked, based upon the same slice type and channel type scheme (the uiNumOne scheme is ignored for these smaller blocks). Two methods of ACV1/ACV2 selection are proposed for smaller blocks. The selection of c1 and c2 is calculated from the 3-coefficient neighbourhood (right, below right and below).

Method 1 (3x Speculation):

ACV1	= ACV_set1[0]		if c2!=0
				= ACV_set1[1]		if c2==0 and c1=1 or 2
				= ACV_set1[2]		if c2==0 and c1=3 or 4

ACV2	= ACV_set2[0]		if c2=0
		ACV2	= ACV_set2[1]		if c2!=0

This has 3 possible pairings, and therefore only 3 different possibilities for the fractions would need to be handled. i.e. for a 4:2:0 system decoding 2 coefficients in parallel, the amount of logic for the entropy decoder would be less than 6 times that shown in Figure 1 (i.e. 12x 10-bit multipliers); for a 4:4:4 system decoding 4 coefficients in parallel, this would be 12 times that shown in Figure 1 (i.e. 24x 10-bit multipliers).

Method 2 (2x Speculation):

If channel is Luma, and coefficient is DC (no neighbourhood check):

ACV1 	= ACV_set1[4]

ACV2 	= ACV_set2[4]

Else:

ACV1	= ACV_set1[0]		if c2!=0
				= ACV_set1[1]		if c2==0

ACV2	= ACV_set2[0]		if c2==0
		ACV2	= ACV_set2[1]		if c2!=0

This has a maximum of 2 possible pairings, and therefore 2 different possibilities for the fractions would need to be handled. Compared with Method 1, the entropy logic requirements for the decoder shown in Figure 1 for 4:2:0 and 4:4:4 systems are reduced to 4 and 8 times respectively.

Note that the new superset of ACVs are currently unoptimized, and are initialized to the same values as those corresponding to the first subset (uiCtxSet=0) of the larger blocks for each slice type and channel combination; in addition, the ACV supersets for the larger blocks also have not been optimized from those in HM4.0, even though they are now being used with a different selection method.

# Simulation Results

## CCV 11-bit System with Multiplier

Below is a summary of the simulation results of a CCV 11-bit system with multiplier (see Section 2.1). These show a small increase in encoding time (1%), and no significant impact on decoding time, whilst giving an average BD rate change of 0.0%, -1.0% and -1.1% for Y, U, and V respectively (classes A-E).

## CCV 9-bit System (without multiplier on m_uiRange)

Below is a summary of the simulation results of a CCV 9-bit system without multiplier (see Section 2.2). These show a small increase in encoding time (2%), and no significant impact on decoding time, whilst giving an average BD rate change of 0.0%, -1.0% and -1.1% for Y, U, and V respectively (classes A-E).

## CCV 11-bit System (without multiplier on m_uiRange)

Below is a summary of the simulation results of a CCV 11-bit system without multiplier (see Section 2.2). This is the recommended base system. These show a small increase in encoding time (1%), and no significant impact on decoding time, whilst giving a combined BD rate change of +0.1%, -1.0% and -1.1% for Y, U, and V respectively (classes A-E).

## CCV with 3x Speculation System

Below is a summary of the 3x speculation system (i.e. 11-bit CCV system without multiplier but with deferred update and neighbourhood CV selection). The software is slower by 3% and 0.5% for the encoder and decoder respectively, not helped by the neighbourhood calculation now required for smaller blocks. The results however indicate an average BD rate change of +0.1%, -1.1% and -1.2% for Y, U, and V respectively (classes A-E).

## CCV with 2x Speculation System

Below is a summary of the 2x speculation system (i.e. 11-bit CCV system without multiplier but with deferred update and neighbourhood CV selection). The software is slower by 3% and 0.5% for the encoder and decoder respectively, again not helped by the neighbourhood calculation now required for smaller blocks. The results however indicate an average BD rate change of +0.2%, -1.0% and -1.1% for Y, U, and V respectively (classes A-E).

## Complete CCV System

Below is a summary of the 2x speculation system in combination with the packet-based bit-stream (JCTVC-G494 “CABAC Packet-based Stream”) with outstanding bits limited to two groups. As expected, the results are similar to the CCV system with 2x speculation, The results however indicate an average BD rate change of +0.2%, -1.0% and -1.1% for Y, U, and V respectively (classes A-E).

We would like to thank Samsung for cross-checking our results (JCTVC-G934).

- Discussion

The Combined Context Variable system describes the quaternary process being used on combined context variables for coefficient data. However, the system could be extended to code CABAC header data.

If the entropy decoder (ED) has to decode data at pixel rate, then the wavefront processing concept (eg JCTVC-E196 “Wavefront Parallel Processing”) could be implemented within a slice unit as a reordering of LCUs: the ED would decode the LCUs in series in a wavefront-based order, but could then pass the raw coefficients onto other parallel modules that would run at a slower rate.

Note that this report analyzes the worst sustained possible case, alleviating the requirement to buffer the decoded results as they are decoded at pixel rate. However, it would be possible to rely on sufficient coefficients having been removed from the stream, i.e. for QP37, QP32, QP27, QP22, QP12 and QP7 with the ‘all-intra’ A-F test conditions, 91%, 87%, 80%, 67%, 32% and 19% (respectively) of the coefficients are trailing 0’s; such reliance would require additional buffering.

The processing times are affected by the reduced speculation algorithms, and in particular the neighbourhood check.

# Conclusion

A “Combined Context Variable” CCV system has been proposed where Arithmetic CVs are combined and written to the CABAC stream using a quaternary arithmetic coding process, within a single pass of the TU data. It is claimed that a system is possible that would be capable of decoding multiple coefficients in a single clock cycle and thereby allowing the entropy decoder to support the sustained worst case of even 4:4:4 data. The 11-bit no-multiplier CCV system yields an average BD rate change of +0.1%, -1.0% and -1.1% for Y, U, and V respectively (classes A-E), with encoding times 1% slower and decoding times similar to the HM4.0 system. No effort has been made to optimize the new functions added to the software.

The problems of speculation have been discussed, and methods to reduce the speculation to 3 levels for small blocks and remove speculation entirely for large blocks have been described, giving an average BD rate change of +0.1%, -1.1% and -1.2% for Y, U, and V respectively (classes A-E); the processing times have increased by +3% and +0.5% for the encoder and decoder respectively compared against HM4.0.

A method to further reduce speculation for small blocks to 2 levels has also been described, giving an average BD rate change of +0.2%, -1.0% and -1.1% for Y, U, and V respectively (classes A-E); processing times have increased by +3% and +0.5% for encoder and decoder respectively. The increase in processing time is due to the deferred update and neighbourhood checks now required for smaller blocks. The context variables for the two methods to reduce speculation have not been optimized.

- Patent rights declaration(s)

Sony Corporation or Sony Group companies may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).