| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G508 |
| --- | --- |

| Title: | CE4: cross-check result of subtest 1.2.c |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Kazushi Sato Sony City Osaki 9F 2-10-1 Osaki, Shinagawa, 141-8610 Tokyo, Japan | Tel: Email: | +81 50 3750 2741 Kazushi.Sato@jp.sony.com |
| Source: | Sony Corp. |  |  |

_____________________________

# Abstract

This document reports cross-check result of CE4: subtest 1.2.c.

# Introduction

In AVC and HEVC relation between QP (Quantization Parameter) and QS (Quantization Scale) is QS(QP+6) / QS(QP) =2.

JCTVC-F495 [1] proposed to add functionality to control granularity of QP for more accurate rate control. Experiment of this proposal is a part of CE4 (subtest 1.2.c).

# Simulation Condition

On top of HM-4.0, this contains TM5 step-3 as an encoding method, the proponent implemented the proposed method and provided to the author. Their modification includes extension of dQP CABAC binarization that was proposed by JCTVC-F745 [3] and adopted into HM-4.0. “Scale1” is test with finer granularity ( QS(QP+12) / QS(QP) = 2) and “scale3” is with coarser granularity ( QS(QP+3) / QS(QP) = 2).

For QP adaptation method described in [3] is applied. IT/IT_Loco/RA/RA_Loco/LD/LD_Loco conditions described in [2] have been tested.

# Simulation Result

In Table 3.1 summary of result with scale = 1 is shown. Complete results are shown in JCTVC-G514_scale1.xls.

In Table 3.2 summary of result with scale = 3 is shown. Complete results are shown in JCTVC-G514_scale3.xls.

The result provided by the proponent has been verified.

Table 3.1

|  | All Intra HE |  |  |  | All Intra LC |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | dQP incr. | Y | U | V | dQP incr. |
| Class A | 1.1% | 0.6% | 0.7% | 30.5% | 1.2% | 0.7% | 0.6% | 35.2% |
| Class B | 1.3% | 0.8% | 0.9% | 32.8% | 1.4% | 0.9% | 0.9% | 36.3% |
| Class C | 1.4% | 0.8% | 0.8% | 31.4% | 1.7% | 1.0% | 1.1% | 36.3% |
| Class D | 1.2% | 0.8% | 0.8% | 29.6% | 1.6% | 0.9% | 1.0% | 34.9% |
| Class E | 1.7% | 1.6% | 1.5% | 30.9% | 2.0% | 1.6% | 1.6% | 35.7% |
| Overall | 1.3% | 0.9% | 0.9% | 31.1% | 1.6% | 1.0% | 1.0% | 35.7% |
|  | 1.3% | 0.9% | 0.9% |  | 1.6% | 1.0% | 1.0% |  |
| Enc Time[%] | 100% |  |  |  | 99% |  |  |  |
| Dec Time[%] | 100% |  |  |  | 100% |  |  |  |
|  | Random Access HE |  |  |  | Random Access LC |  |  |  |
|  | Y | U | V | dQP incr. | Y | U | V | dQP incr. |
| Class A | 1.0% | 0.4% | 0.0% | 28.8% | 1.3% | 0.5% | 0.3% | 34.2% |
| Class B | 1.2% | 0.4% | 0.5% | 30.7% | 1.5% | 0.6% | 0.6% | 35.6% |
| Class C | 1.3% | 0.6% | 0.7% | 29.4% | 1.7% | 0.9% | 1.1% | 34.9% |
| Class D | 1.1% | 0.6% | 0.6% | 29.1% | 1.6% | 0.9% | 0.6% | 34.3% |
| Class E |  |  |  |  |  |  |  |  |
| Overall | 1.2% | 0.5% | 0.4% | 29.6% | 1.5% | 0.8% | 0.7% | 34.8% |
|  | 1.2% | 0.5% | 0.4% |  | 1.5% | 0.8% | 0.7% |  |
| Enc Time[%] | 100% |  |  |  | 100% |  |  |  |
| Dec Time[%] | 101% |  |  |  | 100% |  |  |  |
|  | Low delay B HE |  |  |  | Low delay B LC |  |  |  |
|  | Y | U | V | dQP incr. | Y | U | V | dQP incr. |
| Class A |  |  |  |  |  |  |  |  |
| Class B | 1.4% | 1.2% | 1.4% | 29.7% | 1.8% | 1.5% | 1.2% | 35.3% |
| Class C | 1.4% | 1.0% | 1.3% | 28.7% | 1.9% | 1.5% | 1.6% | 34.6% |
| Class D | 1.3% | 0.8% | 1.9% | 28.8% | 1.9% | 1.3% | 1.3% | 33.8% |
| Class E | 1.6% | 1.2% | 1.0% | 29.2% | 1.9% | 1.2% | 1.2% | 34.2% |
| Overall | 1.4% | 1.0% | 1.4% | 29.1% | 1.9% | 1.4% | 1.3% | 34.5% |
|  | 1.4% | 1.0% | 1.4% |  | 1.9% | 1.5% | 1.3% |  |
| Enc Time[%] | 100% |  |  |  | 100% |  |  |  |
| Dec Time[%] | 101% |  |  |  | 100% |  |  |  |

Table 3.2

|  | All Intra HE |  |  |  | All Intra LC |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | dQP incr. | Y | U | V | dQP incr. |
| Class A | -0.6% | -0.8% | -0.8% | -17.0% | -0.5% | -0.9% | -0.9% | -17.4% |
| Class B | -0.6% | -1.0% | -1.0% | -17.6% | -0.6% | -1.0% | -1.0% | -17.4% |
| Class C | -0.6% | -1.2% | -1.1% | -17.2% | -0.7% | -1.3% | -1.2% | -17.7% |
| Class D | -0.5% | -0.9% | -0.9% | -16.5% | -0.6% | -1.1% | -1.0% | -17.5% |
| Class E | -0.9% | -0.9% | -0.8% | -16.5% | -0.8% | -1.2% | -1.2% | -17.0% |
| Overall | -0.6% | -1.0% | -0.9% | -17.0% | -0.6% | -1.1% | -1.1% | -17.4% |
|  | -0.6% | -1.1% | -1.0% |  | -0.6% | -1.2% | -1.2% |  |
| Enc Time[%] | 100% |  |  |  | 99% |  |  |  |
| Dec Time[%] | 100% |  |  |  | 100% |  |  |  |
|  | Random Access HE |  |  |  | Random Access LC |  |  |  |
|  | Y | U | V | dQP incr. | Y | U | V | dQP incr. |
| Class A | -0.5% | -1.3% | -0.9% | -16.2% | -0.6% | -1.3% | -1.3% | -17.4% |
| Class B | -0.6% | -1.4% | -1.3% | -16.9% | -0.7% | -1.6% | -1.5% | -17.6% |
| Class C | -0.6% | -1.2% | -1.3% | -16.4% | -0.7% | -1.6% | -1.2% | -17.6% |
| Class D | -0.5% | -1.1% | -1.0% | -16.5% | -0.7% | -1.5% | -1.6% | -17.7% |
| Class E |  |  |  |  |  |  |  |  |
| Overall | -0.6% | -1.2% | -1.1% | -16.5% | -0.7% | -1.5% | -1.4% | -17.6% |
|  | -0.6% | -1.3% | -1.2% |  | -0.7% | -1.5% | -1.4% |  |
| Enc Time[%] | 100% |  |  |  | 100% |  |  |  |
| Dec Time[%] | 101% |  |  |  | 100% |  |  |  |
|  | Low delay B HE |  |  |  | Low delay B LC |  |  |  |
|  | Y | U | V | dQP incr. | Y | U | V | dQP incr. |
| Class A |  |  |  |  |  |  |  |  |
| Class B | -0.8% | -0.7% | -0.8% | -16.5% | -0.8% | -0.9% | -1.0% | -17.8% |
| Class C | -0.8% | -1.2% | -0.7% | -16.2% | -0.9% | -1.3% | -1.1% | -17.5% |
| Class D | -0.6% | -1.3% | -0.5% | -16.5% | -0.8% | -1.4% | -1.3% | -17.7% |
| Class E | -0.6% | 0.1% | -1.4% | -16.3% | -0.9% | -2.1% | -0.7% | -17.4% |
| Overall | -0.7% | -0.8% | -0.8% | -16.4% | -0.9% | -1.4% | -1.1% | -17.6% |
|  | -0.7% | -0.8% | -0.8% |  | -0.9% | -1.3% | -1.1% |  |
| Enc Time[%] | 100% |  |  |  | 100% |  |  |  |
| Dec Time[%] | 101% |  |  |  | 100% |  |  |  |

# Conclusion

This document reports cross-check result of CE4: subtest 1.2.c.

The result shows that with scale=1 loss in coding efficiency is observed, and with scale=3 gain is observed.

However, this is not a proposal for coding efficiency but for better controllability. The author recommends discussing the necessity of this functionality at the meeting.

# References

[1] “Higher granularity of quantization parameter scaling and adaptive delta QP signaling”, JCTVC-F495, July 2011

[2] “Common test conditions and software reference configurations”, JCTVC-F900, July 2011

[3] “Description of Core Experiment 4: Quantization,” JCTVC-F904, July 2011.

[4] “BoG report on efficient binary representation of cu_qp_delta syntax for CABAC”, JCTVC-F745, July 2011

# Patent rights declaration(s)

Sony Corp. does not have any current or pending patent rights relating to the technology described in this contribution.