| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G509r1 |
| --- | --- |

| Title: | Support of ChromaQPOffset in HEVC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Shan Liu 2860 Junction Ave. San Jose, CA95134, USA Kazushi Sato Sony City Osaki 9F 2-10-1 Osaki, Shinagawa, 141-8610 Tokyo, Japan | Email: | shan.liu@mediatek.com Kazushi.Sato@jp.sony.com |
| Source: | MediaTek USA / Sony Corp. |  |  |

_____________________________

# Abstract

In AVC, the default relationship between QPY and QPC is defined but a user can tune the balance of luma-chroma image quality by controlling the value of ChromaQPOffset. This functionality has been proved very useful for real applications, and thus, this contribution proposes to include this functionality also in HEVC.

Two types of experiments have been conducted based on HM4.0 to show its usefulness in HEVC standard:

- Uniform Offset throughout the sequences

- Adaptive Offset depending on the temporal hierarchy of the temporal layer

We recommend that this functionality be adopted in HM-5.0.

# Introduction

In standardization activities image quality of luma tends to be respected. However in real applications chroma quality is also important.

For example CE equipments like camcorders support xvYCC (Extended-gamut YCC color space for video applications) color space, that uses wider range of color components. It causes increase in chroma information in a sequence, and if QP for chroma is not fine enough degradation will be noticeable. Especially human eyes are sensitive to red portions. ChromaQPOffset is a useful tool to control the balance of luma-chroma image quality in AVC and it should be inherited to HEVC.

# Simulation Condition

On top of HM-4.0 the authors have implemented ChromaQPOffset functionality. Software is attached with this contribution.

With this software 2 kind tests can be executed as follows:

A: Uniform Offset

This test can be executed by setting the macro in “Typedef.h” as follows:

#define CHROMA_QP_OFFSET 1

With this setting the value of ChromaQPOffset (for U) and ChromaQPOffset2nd (for V) can be set in the encoder configuration file as:

ChromaQpOffset                : -2          # ChromaQPOffset

ChromaQpOffset2nd             : -2          # ChromaQPOffset2nd

With this test the value of ChromaQPOffset remains same throughout the sequences.

Setting of CQO=CQO2=0 provides same result of HM-4.0, except CQO/CQO bits contained in PPS.

B: Adaptive Offset

This test can be executed by setting the macro in “Typedef.h” as follows:

#define CHROMA_QP_OPT 1

With this setting the value of ChromaQPOffset (for U) and ChromaQPOffset2nd (for V) will be either of {2, 1, 0, -1, -1}, depends on the temporal layer in a GOP.

Test condition not described in this section is same as the one defined in [2]. HM-4.0 is used as anchor.

# Simulation Result

## Test A: Uniform Offset

Summary of the result with CQO=CQO2=-2 and +2 are shown in Table 3.1.1 and Table3.1.2 respectively. Complete results are shown in JCTVC_G509_minus2.xls and JCTVC_G509_plus2.xls respectively.

The authors would like to thank Panasonic for cross-checking test A [3].

As can be seen quality balance between luma-chroma can be controlled by adjusting CQO&CQO2.

Table 3.1.1

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.9% | -4.1% | -4.0% | 0.9% | -4.3% | -4.7% |
| Class B | 1.1% | -5.7% | -5.6% | 1.1% | -6.1% | -6.3% |
| Class C | 1.1% | -4.8% | -4.8% | 1.2% | -5.3% | -5.4% |
| Class D | 1.1% | -4.7% | -4.9% | 1.2% | -5.3% | -5.6% |
| Class E | 0.5% | -4.5% | -4.4% | 0.7% | -6.3% | -5.9% |
| Class F |  |  |  |  |  |  |
| Overall | 1.0% | -4.8% | -4.8% | 1.1% | -5.5% | -5.6% |
|  | 1.0% | -4.7% | -4.7% | 1.1% | -5.2% | -5.3% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 101% |  |  |
|  | Random Access HE |  |  | Random Access LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0.5% | -8.8% | -9.7% | 0.6% | -8.4% | -8.7% |
| Class B | 0.7% | -9.8% | -9.7% | 0.8% | -10.3% | -10.3% |
| Class C | 0.7% | -7.4% | -7.2% | 0.8% | -8.1% | -7.5% |
| Class D | 0.6% | -7.9% | -7.9% | 0.8% | -9.0% | -8.6% |
| Class E |  |  |  |  |  |  |
| Class F |  |  |  |  |  |  |
| Overall | 0.6% | -8.6% | -8.7% | 0.8% | -9.0% | -8.9% |
|  | 0.6% | -8.4% | -8.5% | 0.8% | -8.8% | -8.6% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 101% |  |  | 102% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.5% | -5.1% | -5.0% | 0.7% | -6.3% | -6.8% |
| Class C | 0.5% | -3.8% | -3.9% | 0.7% | -4.5% | -4.8% |
| Class D | 0.4% | -4.5% | -4.6% | 0.6% | -5.8% | -6.1% |
| Class E | 0.2% | -7.1% | -5.0% | 0.4% | -11.3% | -8.7% |
| Class F |  |  |  |  |  |  |
| Overall | 0.4% | -5.0% | -4.6% | 0.6% | -6.7% | -6.5% |
|  | 0.4% | -4.9% | -4.5% | 0.6% | -6.4% | -6.1% |
| Enc Time[%] | 99% |  |  | 100% |  |  |
| Dec Time[%] | 104% |  |  | 102% |  |  |

Table 3.1.2

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.8% | 7.0% | 7.3% | -0.8% | 7.9% | 8.1% |
| Class B | -1.0% | 9.8% | 9.9% | -1.0% | 10.5% | 10.9% |
| Class C | -1.0% | 8.2% | 8.1% | -1.1% | 9.0% | 9.0% |
| Class D | -1.0% | 8.0% | 8.1% | -1.0% | 9.3% | 9.4% |
| Class E | -0.4% | 7.1% | 6.8% | -0.6% | 8.9% | 9.0% |
| Class F |  |  |  |  |  |  |
| Overall | -0.9% | 8.2% | 8.2% | -0.9% | 9.2% | 9.4% |
|  | -0.9% | 8.5% | 8.5% | -0.9% | 9.6% | 9.8% |
| Enc Time[%] | 100% |  |  | 99% |  |  |
| Dec Time[%] | 100% |  |  | 101% |  |  |
|  | Random Access HE |  |  | Random Access LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A | -0.3% | 10.1% | 11.4% | -0.5% | 10.4% | 11.5% |
| Class B | -0.5% | 11.8% | 12.2% | -0.7% | 12.3% | 13.1% |
| Class C | -0.5% | 8.5% | 8.2% | -0.6% | 9.2% | 9.0% |
| Class D | -0.4% | 9.3% | 9.0% | -0.6% | 10.6% | 10.4% |
| Class E |  |  |  |  |  |  |
| Class F |  |  |  |  |  |  |
| Overall | -0.4% | 10.0% | 10.3% | -0.6% | 10.7% | 11.1% |
|  | -0.4% | 10.2% | 10.4% | -0.6% | 10.8% | 11.2% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 101% |  |  | 97% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.4% | 6.9% | 6.7% | -0.6% | 9.2% | 10.0% |
| Class C | -0.4% | 4.9% | 4.4% | -0.7% | 6.9% | 7.0% |
| Class D | -0.3% | 5.7% | 5.6% | -0.6% | 9.0% | 8.8% |
| Class E | 0.0% | 8.1% | 8.3% | -0.4% | 10.3% | 11.7% |
| Class F |  |  |  |  |  |  |
| Overall | -0.3% | 6.3% | 6.2% | -0.6% | 8.8% | 9.3% |
|  | -0.3% | 6.4% | 6.2% | -0.6% | 8.7% | 9.3% |
| Enc Time[%] | 99% |  |  | 100% |  |  |
| Dec Time[%] | 87% |  |  | 102% |  |  |

## Test B: Adaptive Offset

Summary of the result is shown in Table 3.2.1. Joint YUV BD-rate is shown in Table 3.2.2. Complete results are shown in JCTVC_G509_adaptive.xls.

The authors would like to thank InterDigital for cross-checking test B [4].

As can be seen luma-chroma BD-performance can be jointly optimized by adjusting CQO&CQO2 depending on temporal layer.

Table 3.2.1

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.43 | -1.94 | -2.63 | 0.42 | -0.96 | -1.46 |
| Class B | 0.51 | -3.02 | -3.07 | 0.51 | -2.97 | -2.79 |
| Class C | 0.50 | -2.02 | -1.81 | 0.56 | -1.92 | -1.79 |
| Class D | 0.49 | -2.12 | -1.77 | 0.56 | -2.06 | -1.82 |
| Class E | 0.21 | -1.10 | -0.86 | 0.24 | -1.22 | -0.75 |
| All | 0.44 | -2.14 | -2.14 | 0.47 | -1.91 | -1.82 |
| Enc Time[%] | 101% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |
|  |  | Random access |  |  | Random access LoCo |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.2 | -4.3 | -4.6 | 0.3 | -2.9 | -2.9 |
| Class B | 0.3 | -4.2 | -5.2 | 0.4 | -4.1 | -4.8 |
| Class C | 0.3 | -3.4 | -3.5 | 0.4 | -3.5 | -3.4 |
| Class D | 0.3 | -4.1 | -4.2 | 0.4 | -4.1 | -3.9 |
| Class E |  |  |  |  |  |  |
| All | 0.3 | -4.0 | -4.4 | 0.3 | -3.7 | -3.8 |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |
|  | Low delay B |  |  | Low delay B LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 0.3 | -2.9 | -3.0 | 0.4 | -2.8 | -2.8 |
| Class C | 0.3 | -2.2 | -2.3 | 0.4 | -1.9 | -2.4 |
| Class D | 0.2 | -2.7 | -2.5 | 0.3 | -2.4 | -2.4 |
| Class E | 0.2 | -3.6 | -3.0 | 0.3 | -5.6 | -3.8 |
| All | 0.2 | -2.8 | -2.7 | 0.3 | -3.0 | -2.8 |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |
|  | Low delay P |  |  | Low delay P LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 0.3 | -3.2 | -3.1 | 0.4 | -3.1 | -3.2 |
| Class C | 0.3 | -2.3 | -2.5 | 0.3 | -2.5 | -2.3 |
| Class D | 0.2 | -2.7 | -3.2 | 0.2 | -2.7 | -2.7 |
| Class E | 0.1 | -3.9 | -3.5 | 0.4 | -6.8 | -4.7 |
| All | 0.2 | -3.0 | -3.0 | 0.3 | -3.5 | -3.1 |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

Table 3.2.2

|  | Random access | Random Access LoCo |
| --- | --- | --- |
|  | YUV BD-rate | YUV BD-rate |
| Class A | -0.3 | -0.1 |
| Class B | -0.6 | -0.4 |
| Class C | -0.4 | -0.3 |
| Class D | -0.4 | -0.4 |
| Class E |  |  |
| All | -0.4 | -0.3 |
|  | Low delay B | Low delay B LoCo |
|  | YUV BD-rate | YUV BD-rate |
| Class A |  |  |
| Class B | -0.3 | -0.2 |
| Class C | -0.2 | -0.1 |
| Class D | -0.3 | -0.2 |
| Class E | -0.5 | -0.5 |
| All | -0.3 | -0.3 |
|  | Low delay P | Low delay P LoCo |
|  | YUV BD-rate | YUV BD-rate |
| Class A |  |  |
| Class B | -0.4 | -0.3 |
| Class C | -0.3 | -0.2 |
| Class D | -0.3 | -0.3 |
| Class E | -0.6 | -0.6 |
| All | -0.4 | -0.3 |

# Proposed Change of WD

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| pic_parameter_set_id | ue(v) |
| seq_parameter_set_id | ue(v) |
| entropy_coding_mode_flag | u(1) |
| num_temporal_layer_switching_point_flags | ue(v) |
| for( i = 0; i < num_temporal_layer_switching_point_flags; i++ ) |  |
| temporal_layer_switching_point_flag[ i ] | u(1) |
| num_ref_idx_l0_default_active_minus1 | ue(v) |
| num_ref_idx_l1_default_active_minus1 | ue(v) |
| pic_init_qp_minus26 /* relative to 26 */ | se(v) |
| constrained_intra_pred_flag | u(1) |
| slice_granularity | u(2) |
| shared_pps_info_enabled_flag | u(1) |
| if( shared_pps_info_enabled_flag ) |  |
| if( adaptive_loop_filter_enabled_flag ) |  |
| alf_param( ) |  |
| if( cu_qp_delta_enabled_flag ) |  |
| max_cu_qp_delta_depth | u(4) |
| ChromaQPOffset |  |
| ChromaQPOffset2nd |  |
| rbsp_trailing_bits( ) |  |
| } |  |

The value of QP’Y is derived as

QP’Y = QPY + QpBdOffsetY		(7-1)

The value of QP’c is derived as

QP’C=QPCtoQPY[ clip(0,51,QP’Y+Shift ) ]

Where

QPCtoQPY= {

0, 1, 2, 3, 4, 5, 6, 7, 8, 9,10,11,

12,13,14,15,16,17,18,19,20,21,22,23,24,25,26,27,

28,29,29,30,31,32,32,33,34,34,35,35,36,36,37,37,

37,38,38,38,39,39,39,39}

Shift = ChromaQPOffset (for U) or ChromaQPOffset2nd (for V)

Values of ChromaQPOffset and ChromaQPOffset2nd are in the range of [-12, 12]

# Conclusion

On top of HM-4.0 ChromaQPOffset functionally has been implemented and 2 kind of tests have been conducted to show that it can

- Control image quality balance between luma-chroma

- Jointly optimize BD-performance of luma-chroma

We recommend this functionality be adopted into HM-5.0.

# References

[1] “Method for deriving Chroma QP from Luma QP”, JCTVC-F277, Jul 2011

[2] “Common test conditions and software reference configurations”, JCTVC-F900, Jul 2011

[3] “Cross-check report for Support of ChromaQPOffset in HEVC (JCTVC-G509)”, JCTVC-G759, Nov 2011

[4] “Crosscheck of JCTVC-G509 - Support of ChromaQPOffset in HEVC”, JCTVC-G907, Nov 2011

# Patent rights declaration(s)

MediaTek USA may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Sony Corp. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).