| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G510 |
| --- | --- |

| Title: | CE6.a: Result of Subtest 4.1.2.2 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kazushi Sato Sony City Osaki 9F 2-10-1 Osaki, Shinagawa, 141-8610 Tokyo, Japan | Tel: Email: | +81 50 3750 2741 Kazushi.Sato@jp.sony.com |
| Source: | Sony Corp. |  |  |

_____________________________

# Abstract

This document reports result of CE6.a: Subtest 4.1.2.2.

# Introduction

In JCTVC-E266 [3] luma-based chroma intra prediction [LM_CHROMA] was proposed and adopted into HM. It contributes to gain in chroma by as much as much as -7.8% for U and -6.1% for V in Intra and Intra_Loco cases.

At 6th JCTVC mtg, there were several contributions on the simplification of LM_CHROMA [4]. Most simplifications are on calculation of correlation factor α&β, but storage of reconstructed luma (Rec’L) is also concern on implementation. In the worst case it reaches as much as 16x16x10bit. This data will be stored only for LM_CHROMA, as the position is different from the one of original luma pixels.

To solve this problem the author proposed to store Rec’L not in 10 but 8 bit accuracy for HE case and LC case (Just for 10-bit input, that is, SteamLocomotiveTrain and Nebuta). Additionally Rec’L not in 8 but 6 bit accuracy is tested for LC case except SteamLocomotiveTrain and Nebuta.

Fig 1.1

# Simulation Condition

HM-4.0 with bugfix (alpha-7bits) is used both for the anchor.

On top of this software the proposed method has been implemented. To test the method the following macro in “Typedef.h” should be set as follows:

#define RECL_REDUCTION 1

Common test condition described in [5] is applied and IT/IT_LOCO conditions are tested.

# Simulation Result

Summary of the result is shown in Table 3.1. Complete result is shown in JCTVC-Gxxd.xls.

The author would like to thank Panasonic and Qualcomm for cross-checking.

As can be seen loss in coding efficiency is as small as 0.1% for HE case and 0.2% for LC case.

Especially with HE case, 10-bit processing is concern with software implementation where byte-alignment of data is important, so by reducing bit accuracy to 8 storage of Rec’L can be greatly reduced.

Table 3.1

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.1% | 0.1% | 0.0% | 0.3% | 0.1% |
| Class B | 0.0% | 0.1% | 0.0% | 0.0% | 0.3% | 0.2% |
| Class C | 0.0% | 0.0% | 0.1% | 0.0% | 0.2% | 0.3% |
| Class D | 0.0% | 0.0% | 0.1% | 0.0% | 0.1% | 0.2% |
| Class E | 0.0% | 0.1% | 0.1% | 0.0% | 0.1% | 0.2% |
| Class F |  |  |  |  |  |  |
| Overall | 0.0% | 0.0% | 0.1% | 0.0% | 0.2% | 0.2% |
|  | 0.0% | 0.0% | 0.1% | 0.0% | 0.2% | 0.2% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 101% |  |  | 102% |  |  |

# Proposed Change in WD

##### Specification of Intra_FromLuma prediction mode

Inputs to this process are:

–	a sample location ( xB, yB ) specifying the top-left sample of the current block relative to the top-left sample of the current picture,

–	neighbouring samples p[ x, y ], with x, y = -1..2*nS-1,

–	a variable nS specifying the prediction size.

Output of this process is:

–	predicted samples predSamples[ x, y ], with x, y =0..nS-1.

This intra prediction mode is invoked when intraPredMode is equal to 35.

The values of the prediction samples predSamples[ x, y ], with x, y = 0..nS-1, are derived as the following ordered steps:

- Variable k3 and the sample array pY’ are derived as:

k3 = Max( 0, BitDepthC + log2( nS ) – 14 )		(8-1)

pY’[ x, y ] = ( recSamplesL[ 2x, 2y ] + recSamplesL[ 2x, 2y+1 ] ) >> 1, with x, y = -1..2*nS	(8-2)

if internal bit-depth is 10 bit, not pY’[ x, y ] but p’’[x,y] = (pY’[ x, y ]+2) >> 2 is stored in a buffer.

- Variables L, C, LL, LC and k2 are derived as follows:

L = y=0nS−1pY'[−1, y]+x=0nS−1pY'[x, −1]>>k3		(8-3)

C = y=0nS−1p[−1, y]+x=0nS−1p[x, −1]>>k3		(8-4)

LL = y=0nS−1pY'[−1, y]2+x=0nS−1pY'[x, −1]2>>k3	(8-5)

LC = y=0nS−1pY'[−1, y]∗p[−1, y]+y=0nS−1pY'[x, −1]∗p[x,−1]>>k3	(8-6)

k2 = log2( (2*nS) >> k3 )		(8-7)

- Variables a and b are derived as:

a1 	= ( LC << k2 ) – L*C		(8-8)
a2 	= ( LL << k2 ) – L*L		(8-9)
k 	= Max( 0, log2( abs( a2 ) ) – 5 ) – Max( 0, log2( abs( a1 ) ) – 14 ) + 2	(8-10)
a1s	= a1 >> Max(0, log2( abs( a1 ) ) – 14 )		(8-11)
a2s	= abs( a2 >> Max(0, log2( abs( a2 ) ) – 5 ) ) + 1	(8-12)

[Ed. (WJ): bug in software – a2s should be abs( a2 >> Max(0, log2( abs( a2 ) ) – 5 ) ) – without ‘+1’]

a	= Clip3( -215, 215-1, a1s*lmDiv + ( 1 << ( k1 – 1 ) ) >> k1 )	(8-13)
b	= ( L – ( ( a*C ) >> k1 ) + ( 1 << ( k2 – 1 ) ) ) >> k2	(8-14)

where lmDiv is specified in Error: Reference source not found with the input a2s.

- The values of the prediction samples predSamples[ x, y ] are derived as:

predSamples[ x, y ] = Clip1C( ( ( pY’[ x, y ] * a ) >> 13 ) + b ), with x, y = 0..nS-1	(8-15)

if internal bit-depth is 10bit p’[x,y] is reconstructed as p’[x,y] = p’’[x,y]<<2.

# Conclusion

On luma-based chroma intra prediction not only calculation of α&β but also Rec’L storage size is concern on implementation.

This document proposes to store it with reduced bit-accuracy like not 10 but 8 for HE and not 8 but 6 for LC. The loss caused by the proposed reduction is as small as 0.1% and 0.2% for HE and LC cases.

It is recommended to adopt the proposed reduction into HM-5.0 at least for HE and LC with 10bit-input. This is a meaningful reduction especially for software implementation, where byte-aligned is important.

# References

[1] “WD4: Working Draft 4 of High-Efficiency Video Coding”, JCTVC-F803, July 2011

[2] “CE6: Intra Coding Improvement”, JCTVC-F906, July 2011

[3] “CE6.a.4: Chroma intra prediction by reconstructed luma samples”, JCTVC-E266, Mar 2011

[4] “BoG report on intra mode coding with fixed number of MPM candidates”, JCTVC-F765, Jul 2011

[5] “Common test conditions and software reference configurations”, JCTVC-F900, July 2011

[6] “Complexity Reduction of Chroma Intra Prediction by Reconstructed Luma Samples”, JCTVC-F431, July 2011

# Patent rights declaration(s)

Sony Corp. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).