| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G511 |
| --- | --- |

| Title: | Performance Evaluation of Luma-based Chroma Intra Prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Kazushi Sato Sony City Osaki 9F 2-10-1 Osaki, Shinagawa, 141-8610 Tokyo, Japan | Tel: Email: | +81 50 3750 2741 Kazushi.Sato@jp.sony.com |
| Source: | Sony Corp. |  |  |

_____________________________

# Abstract

This document provides data on BD-performance on luma based chroma intra prediction. The original proposal JCTVC-E266 contains result with longer GOP as well as all-intra conditions, but in CE6 performances of related proposals are evaluated just with IT/IT_Loco.

As simplification methods have been adopted, it is necessary to evaluate its performance once again not only with IT/IT_Loco but with all conditions.

# Introduction

Luma-based chroma intra prediction [LM_CHROMA] proposed by JCTVC-E266 [2] has been adopted at 4th JCTVC mtg. In the 5th JCTVC mtg several simplification methods [4] have been proposed and as a result of performance evaluation at BoG some of them have been adopted into HM-4.0.

[2] contains result not only with IT/IT_Loco cases but with longer GOP like RA/RA_Loco/LD/LD_Loco, but as of simplification proposals only IT/IT_Loco results are presented.

The purpose of this document is to provide information on the performance evaluation of LM_CHROMA not only with IT/IT_Loco cases but also with longer-GOP cases.

# Simulation Condition

HM-4.0 with bugfix (alpha-7bits) is used both for anchor and tested.

For tested macro setting in “Typedef.h” is as follows:

#define LM_CHROMA 0

Common test condition described in [3] is applied and IT/IT_LOCO/RA/RA_Loco/LD/LD_Loco conditions are tested with class A to E sequences.

# Simulation Result

Summary of the result is shown in Table 3.1. Complete results are shown in JCTVC-G511.xls.

As can be seen large gain can be obtained not only with IT/IT_Loco cases but also with RA/RA_Loco cases. With LD/LD_Loco cases gain is rather limited in comparison with other conditions.

Probably this is because in RA/RA_Loco conditions maximum distance between referenced B pictures is 16 frames and with such long-distance reference motion estimation does not work well, and more PUs drop into intra mode. In this case LM_CHROMA also works. On the contrary the distance between referenced B pictures are rather short in LD/LD_Loco cases and more PUs are coded with inter mode thus not so much gain can be obtained with improved intra coding mode.

Table 3.1

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 1.2% | 25.6% | 15.9% | 1.2% | 25.5% | 15.3% |
| Class B | 0.5% | 7.1% | 3.8% | 0.7% | 7.2% | 3.7% |
| Class C | 0.9% | 6.8% | 7.7% | 1.1% | 6.5% | 7.6% |
| Class D | 0.4% | 4.7% | 4.7% | 0.6% | 4.7% | 4.7% |
| Class E | 0.1% | 1.8% | 2.2% | 0.1% | 3.1% | 4.0% |
| Class F |  |  |  |  |  |  |
| Overall | 0.6% | 9.5% | 6.9% | 0.7% | 9.6% | 7.0% |
|  | 0.6% | 9.3% | 6.7% | 0.7% | 9.4% | 6.9% |
| Enc Time[%] | 99% |  |  | 98% |  |  |
| Dec Time[%] | 99% |  |  | 100% |  |  |
|  | Random Access HE |  |  | Random Access LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | 29.5% | 18.9% | 0.2% | 32.6% | 16.8% |
| Class B | 0.2% | 8.8% | 4.3% | 0.3% | 8.5% | 3.9% |
| Class C | 0.3% | 6.4% | 6.8% | 0.5% | 6.3% | 7.0% |
| Class D | 0.2% | 4.8% | 4.5% | 0.4% | 4.9% | 4.5% |
| Class E |  |  |  |  |  |  |
| Class F |  |  |  |  |  |  |
| Overall | 0.2% | 12.2% | 8.4% | 0.3% | 12.8% | 7.8% |
|  | 0.2% | 12.1% | 8.1% | 0.4% | 12.8% | 7.8% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 98% |  |  | 99% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | 2.4% | 1.7% | 0.1% | 2.9% | 1.8% |
| Class C | 0.1% | 2.6% | 2.9% | 0.2% | 2.7% | 3.0% |
| Class D | 0.0% | 2.1% | 2.0% | 0.2% | 1.8% | 2.1% |
| Class E | -0.1% | 2.4% | 3.4% | 0.0% | 2.9% | 5.3% |
| Class F |  |  |  |  |  |  |
| Overall | 0.0% | 2.4% | 2.4% | 0.1% | 2.6% | 2.8% |
|  | 0.0% | 2.4% | 2.4% | 0.1% | 2.5% | 2.8% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 102% |  |  | 100% |  |  |

# Conclusion

Gain by 10% or more with U component and 7% or more with V component can be observed with LM_CHROMA not only with IT/IT_Loco cases but also with RA/RA_Loco cases where distance between referenced B picture reaches as long as 16 frames in the worst case and more PUs in inter pictures are coded with intra mode. For LD/LD_Loco cases more PUs in inter pictures are coded with inter mode, and gain is around 2-3%.

The author would like to voice that gain with LM_CHROMA is large but increase in implementation cost is also large, so its trade-off between gain and implantation cost should be carefully studied.

# References

[1] “WD4: Working Draft 4 of High-Efficiency Video Coding”, JCTVC-F803, July 2011

[2] “CE6.a.4: Chroma intra prediction by reconstructed luma samples”, JCTVC-E266, Mar 2011

[3] “Common test conditions and software reference configurations”, JCTVC-F900, July 2011

[4] “BoG report on simplification of intra_chromaFromLuma mode prediction”, JCTVC-F760, Jul 2011

# Patent rights declaration(s)

Sony Corp. does not have any current or pending patent rights relating to the technology described in this contribution.