| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G513 |
| --- | --- |

| Title: | CE9: Cross-check result of Sel01&02 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Kazushi Sato Sony City Osaki 9F 2-10-1 Osaki, Shinagawa, 141-8610 Tokyo, Japan | Tel: Email: | +81 50 3750 2741 Kazushi.Sato@jp.sony.com |
| Source: | Sony Corp. |  |  |

_____________________________

# Abstract

This document reports cross-check result of CE9: Sel01&02.

# Introduction

To find spatial motion vector predictor both non-scaled and scaled motion vectors of neighboring PUs are scanned. CE9: Sel01&02 are test on the order of scanning spatial neighboring motion information. Sel01 is one-pass and Sel02 is two pass.

Scanning order of each method are as follows:

HM-4.0:

A:

A0 non-scaled, A1 non-scaled, A0 scaled, A1 scaled

B:

B0 non-scaled, B1 non-scaled, B2 non-scaled, B0 scaled (if A is not scaled), B1 scaled (if A is not scaled), B2 scaled (if A is not scaled)

Sel01:

B0 non-scaled, B0 scaled (if S0 is not scaled), A0 non-scaled, A0 scaled (if S0 is not scaled), A1 non-scaled, A1 scaled (if S0 is not scaled), B1 non-scaled, B1 scaled (if S0 is not scaled), B2 non-scaled, B2 scaled (if S0 is not scaled)

Sel02:

first pass: 	B0 non-scaled, A0 non-scaled, A1 non-scaled, B1 non-scaled, B2 non-scaled

second pass: 	B0 scaled (if S0 is not scaled), A0 scaled (if S0 is not scaled), A1 scaled (if S0 is not scaled), B1 scaled (if S0 is not scaled), B2 scaled (if S0 is not scaled)

Fig 1.1

This document reports cross-check result of CE9: Sel01&02.

# Simulation Condition

On top of HM-4.0 the proposed methods have been implemented by the proponent.

Macro setting of each test in “Typedef.h” are as follows:

AMVP_SEL01:

#define JVC_AMVP_SEL01                    1       // 1: enable, 0: disable

#define JVC_AMVP_SEL02                    0       // 1: enable, 0: disable

AMVP_SEL02:

#define JVC_AMVP_SEL01                    0       // 1: enable, 0: disable

#define JVC_AMVP_SEL02                    1       // 1: enable, 0: disable

Both for SEL01 and 02 LD/LD_Loco/LP/LP_Loco/RA/RA_Loco conditions as described in [2] have been tested with class A to E sequences.

# Simulation Result

Summary of the results with Sel01 and Sel02 are shown in Table 3.1 and 3.2 respectively. Complete results are shown in JCTVC-Gxxf_SEL01.xls and JCTVC-Gxxf_SEL02.xls respectively.

The result matches the one provided by the proponent.

Table 3.1

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | -0.2% | -0.2% | 0.0% | 0.2% | 0.1% |
| Class B | 0.0% | -0.1% | 0.0% | 0.0% | -0.1% | 0.0% |
| Class C | 0.0% | -0.1% | 0.1% | 0.0% | 0.0% | 0.0% |
| Class D | 0.1% | 0.1% | 0.0% | 0.0% | -0.1% | 0.0% |
| Class E |  |  |  |  |  |  |
| Class F |  |  |  |  |  |  |
| Overall | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | -0.1% | 0.0% | 0.0% | 0.1% | 0.0% |
| Class C | 0.0% | 0.0% | -0.1% | 0.0% | 0.2% | -0.1% |
| Class D | 0.0% | 0.0% | 0.3% | 0.0% | 0.1% | -0.2% |
| Class E | 0.1% | -0.1% | 0.6% | -0.1% | 0.4% | -0.4% |
| Class F |  |  |  |  |  |  |
| Overall | 0.0% | 0.0% | 0.2% | 0.0% | 0.2% | -0.1% |
|  | 0.0% | -0.1% | 0.1% | 0.0% | 0.2% | -0.1% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |
|  | Low delay P HE |  |  | Low delay P LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | 0.1% | 0.3% | 0.0% | 0.1% | -0.2% |
| Class C | 0.0% | 0.2% | 0.3% | 0.0% | 0.0% | 0.1% |
| Class D | 0.0% | 0.2% | 0.2% | 0.0% | 0.3% | 0.1% |
| Class E | 0.0% | -0.4% | -0.2% | 0.0% | -0.3% | -0.3% |
| Class F |  |  |  |  |  |  |
| Overall | 0.0% | 0.0% | 0.2% | 0.0% | 0.1% | -0.1% |
|  | 0.0% | 0.0% | 0.1% | 0.0% | 0.1% | -0.1% |
| Enc Time[%] | 103% |  |  | 101% |  |  |
| Dec Time[%] | 103% |  |  | 100% |  |  |

Table 3.2

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | -0.2% | 0.0% | 0.1% | 0.1% | 0.1% |
| Class B | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | -0.1% |
| Class D | 0.0% | -0.1% | 0.0% | 0.0% | -0.2% | 0.0% |
| Class E |  |  |  |  |  |  |
| Class F |  |  |  |  |  |  |
| Overall | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 101% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | -0.2% | -0.2% | 0.0% | 0.0% | -0.1% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | 0.0% |
| Class D | 0.0% | 0.1% | 0.2% | 0.0% | 0.4% | -0.1% |
| Class E | 0.0% | 0.1% | 0.4% | -0.1% | 0.0% | 0.0% |
| Class F |  |  |  |  |  |  |
| Overall | 0.0% | 0.0% | 0.1% | 0.0% | 0.1% | -0.1% |
|  | 0.0% | -0.1% | 0.0% | 0.0% | 0.1% | -0.1% |
| Enc Time[%] | 99% |  |  | 100% |  |  |
| Dec Time[%] | 99% |  |  | 100% |  |  |
|  | Low delay P HE |  |  | Low delay P LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | 0.0% | 0.2% | 0.0% | 0.1% | -0.2% |
| Class C | -0.1% | -0.1% | 0.0% | 0.0% | -0.1% | -0.1% |
| Class D | 0.0% | 0.0% | -0.2% | 0.0% | -0.2% | -0.2% |
| Class E | -0.1% | 0.3% | 0.0% | 0.1% | -0.3% | -0.3% |
| Class F |  |  |  |  |  |  |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | -0.2% |
|  | 0.0% | 0.0% | -0.1% | 0.0% | -0.1% | -0.2% |
| Enc Time[%] | 101% |  |  | 98% |  |  |
| Dec Time[%] | 101% |  |  | 98% |  |  |

# Conclusion

Performance both in coding efficiency and enc/dec time is similar with Sel01 and Sel02.

In terms of complexity Sel02 may be preferable because it will require less scaling especially with software implementation.

# References

[1] “WD4: Working Draft 4 of High-Efficiency Video Coding”, JCTVC-F803, July 2011

[2] “Common test conditions and software reference configurations”, JCTVC-F900, July 2011

[3] “Description of Core Experiment 9: MV Coding and Skip/Merge Operations”, July 2011

[4] “Unification of derivation process for merge mode and MVP”, JCTVC-F419, July 2011

# Patent rights declaration(s)

Sony Corp. does not have any current or pending patent rights relating to the technology described in this contribution.