| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G515 |
| --- | --- |

| Title: | Cross-check of JCTVC-G552: Simplification of temporal motion vector (TMVP) candidate derivation for Merge and AMVP |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Kazushi Sato Sony City Osaki 9F 2-10-1 Osaki, Shinagawa, 141-8610 Tokyo, Japan | Tel: Email: | +81 50 3750 2741 Kazushi.Sato@jp.sony.com |
| Source: | Sony Corp. |  |  |

_____________________________

# Abstract

Motion vector coding (AMVP/Merge) contains co-located center specification for temporal motion vector predictor derivation. Finding a center-block in the reference picture requires increase in complexity.

Enabling/disabling of this functionality is studies in CE9, but alternative simplification method is proposed by JCTVC-G552. This document reports verification result of JCTVC-G552.

# Introduction

In the current HM specification motion information of CR and H is used as temporal motion vector predictor.

By setting macros in “Typedef.h” as follows, not CR but T is used for temporal prediction. Performance evaluation with this setting will be studied as CE9: subtest SP01.

#define FT_TCTR_MRG	0           ///< central colocated in MERGE

#define FT_TCTR_AMVP	0           ///< central colocated in AMV

Even with this setting H is still used as one of the temporal motion vector predictor.

JCTVC-G552 proposes to simplify temporal motion vector derivation as just using T, not CR or H.

Fig 1.1

# Simulation Condition

HM-4.0 is used as anchor.

On top of anchor software the proposed method is implemented by the proponent and provided to the author.

To test the proposed method the following macro in “Typedef.h” are enabled:

#define TCTR_AMVP_MRG_OFF                  1

LD/LD_Loco/LP/LP_Loco/RA/RA_Loco conditions as described in [2] are tested with class A to E sequences.

# Simulation Result

Summary of the result is shown in Table 3.1. Complete results are shown in JCTVC-G515.xls.

The result provided by the proponent is verified.

As can be seen average loss in each test condition is less than or equal to 0.1%.

Table 3.1

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% |
| Class B | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | -0.1% | 0.0% | 0.0% | 0.1% | 0.0% |
| Class E |  |  |  |  |  |  |
| Class F |  |  |  |  |  |  |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | -0.1% | 0.0% | 0.0% | -0.1% | 0.2% |
| Class C | 0.0% | -0.1% | -0.1% | 0.0% | 0.2% | -0.2% |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | 0.0% |
| Class E | 0.0% | 0.4% | 0.5% | 0.1% | -0.1% | 0.3% |
| Class F |  |  |  |  |  |  |
| Overall | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.1% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 103% |  |  | 101% |  |  |
|  | Low delay P HE |  |  | Low delay P LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.2% | 0.1% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | -0.2% |
| Class D | 0.1% | -0.1% | 0.0% | 0.0% | 0.1% | 0.1% |
| Class E | 0.0% | 0.2% | -0.3% | 0.0% | -0.1% | 0.0% |
| Class F |  |  |  |  |  |  |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% |
|  | 0.0% | 0.1% | -0.1% | 0.0% | 0.0% | -0.1% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

# Conclusion

Experimental result on JCTVC-G552 has been verified. With the proposed simplification average loss by less than 0.1% is observed with LD/LD_Loco/LP/LP_Loco/RA/RA_Loco conditions.

Functionality of col-center for TMVP causes increase in complexity. The author recommend to adopt this simplification into HM-5.0.

# References

[1] “WD4: Working Draft 4 of High-Efficiency Video Coding”, JCTVC-F803, Jul 2011

[2] “Common test conditions and software reference configurations”, JCTVC-F900, Jul 2011

[3] “Description of Core Experiment 9: MV Coding and Skip/Merge Operations”, Jul 2011

[4] “Simplification of temporal motion vector (TMVP) candidate derivation for Merge and AMVP”, Nov 2011

# Patent rights declaration(s)

Sony Corp. does not have any current or pending patent rights relating to the technology described in this contribution.