| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G516 |
| --- | --- |

| Title: | Consideration on Spatial Motion Vector Prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kazushi Sato Sony City Osaki 9F 2-10-1 Osaki, Shinagawa, 141-8610 Tokyo, Japan | Tel: Email: | +81 50 3750 2741 Kazushi.Sato@jp.sony.com |
| Source: | Sony Corp. |  |  |

_____________________________

# Abstract

Under current HM specification motion information of topright PU is one of the candidates for spatial motion vector prediction.

When LCU-level parallel processing is applied for HEVC encoding/decoding, extraction of motion information of topright PU becomes bottleneck.

This contribution proposes not using motion information of topright PU at LCU boundaries for motion vector coding. Average losses with the proposed method is less than 0.2% for Y/U/V component with RA/RA_Loco/LD/LD_Loco conditions.

It is recommended that the proposed method be studied under the core experiment.

# Introduction

In HEVC the size of CU or PU varies within a sequence but the size of LCU is fixed. Therefore LCU-level parallel processing works well both with encoding and decoding as shown in Fig 1.1.

In Fig 1.1 both of processing of 2 LCU@t1 will be executed simultaneously with LCU-Level parallel processing.

However in the current HM specification motion information of topright PU is one of the candidates for spatial motion vector prediction, so at the LCU boundaries PUtr as shown in Fig 1.1 becomes bottleneck for this LCU-Level parallel processing.

Similar problem occurs with intra prediction. However, it can be avoided by prohibiting ipred modes that uses topright pixels as encoder issue. With motion vector coding case this can be avoided as encoder issue by prohibiting top PUs as a predictor candidate but it will cause big loss in coding efficiency.

Fig 1.1

# Proposed Algorithm

To solve the issue stated above, we propose not to use topright PUs for spatial motion vector prediction at the slice boundaries as shown in Fig 2.1. This is applied both with AMVP and Merge processes.

Fig 2.1

# Simulation Condition

HM-4.0 is the anchor for the experiment. On top of this anchor software the proposed method as described in the previous section is implemented.

To test the proposed method the following macros in “Typedef.h” should be set:

#define G516 1

RA/RA_Loco/LD/LD_Loco conditions as specified in [2] are tested with class A to E sequences.

# Simulation Result

Summary of the result is shown in Table 4.1.

Complete result is shown in JCTVC-G516.xls.

The author would like to thank JVCKenwood for crosschecking [3].

Average losses with the proposed method is less than 0.2% for Y/U/V component with RA/RA_Loco/LD/LD_Loco conditions. This loss may be compensated by defining spatial neighboring PUs for motion vector prediction at LCU boundaries as shown in Fig 4.1.

Table 4.1

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.2% | 0.0% | 0.1% | 0.2% | 0.2% | 0.2% |
| Class B | 0.2% | 0.1% | 0.2% | 0.2% | 0.2% | 0.2% |
| Class C | 0.1% | 0.0% | 0.1% | 0.1% | 0.1% | 0.0% |
| Class D | 0.1% | 0.0% | 0.1% | 0.0% | -0.1% | 0.1% |
| Class E |  |  |  |  |  |  |
| Class F |  |  |  |  |  |  |
| Overall | 0.1% | 0.0% | 0.1% | 0.1% | 0.1% | 0.1% |
|  | 0.1% | 0.0% | 0.1% | 0.1% | 0.1% | 0.1% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 101% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.2% | 0.2% | 0.1% | 0.2% | 0.2% | 0.0% |
| Class C | 0.1% | 0.1% | -0.1% | 0.1% | 0.2% | -0.1% |
| Class D | 0.1% | 0.1% | 0.1% | 0.1% | -0.2% | 0.1% |
| Class E | 0.3% | 0.6% | 0.6% | 0.2% | 0.2% | 0.5% |
| Class F |  |  |  |  |  |  |
| Overall | 0.1% | 0.2% | 0.1% | 0.1% | 0.1% | 0.1% |
|  | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 99% |  |  |

Fig 4.1

# Conclusion

This document proposes modification on spatial motion vector predictor derivation process to enable LCU-level parallel processing. Simulation result shows that average losses with the proposed modification at LCU boundaries as shown in Fig 2.1 is less than 0.2% for Y/U/V component with RA/RA_Loco/LD/LD_Loco conditions. This loss will be able to be compensated with modification shown in Fig 4.1.

It is recommended that the proposed method be studied under the core experiment.

# References

[1] “WD4: Working Draft 4 of High-Efficiency Video Coding”, JCTVC-F803, Jul 2011

[2] “Common test conditions and software reference configurations”, JCTVC-F900, Jul 2011

[3] “Non-CE9: Crosscheck for Sony's proposals (JCTVC-G516) by JVC KENWOOD”, Nov 2011

# Patent rights declaration(s)

Sony Corp. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).