| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G517 |
| --- | --- |

| Title: | CE2 subtest C.1: Harmonization of unified scan and NSQT |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xiaozhen Zheng, HiSilicon Yuan Yuan, Tsinghua Yun He, Tsinghua | Tel: Email: | zhengxiaozhen@huawei.com y-yuan08@mails.tsinghua.edu.cn |
| Source: | HiSilicon Technologies, Tsinghua Univeristy |  |  |

_____________________________

# Abstract

This document provides a harmonization solution of unified scan and non-square quadtree transform (NSQT). At the proposed solution, non-square to square reordering process for transform coefficient coding is removed. The experimental results show that no coding lost under common test condition. Encoding and decoding time for the proposed solution is almost same as HM4.0 anchor.

# Introduction

At Torino meeting, unified scan [1] has been adopted to HEVC working draft and HEVC Model version 4 (HM4). At the unified scan, backward direction scan, which is from last significant coefficient to the first coefficient, is applied to the significance map for CABAC coefficient coding. Therefore, forward zig-zag scan on 4x4 subblock was removed.

At the same meeting, NSQT [2] has also been adopted to HEVC working draft and HM4. At the NSQT structure, 32x8, 8x32, 16x4 and 4x16 transform are added to HEVC framework. Due to 4x4 subblock zig-zag scanning used at HM3, a reordering process of non-square coefficient array to square coefficient array is used prior CABAC coefficient coding. Then, square coefficient coding is used which is same as square transform coefficient coding process. At the decoder side, a reordering process of square data to non-square data is need after residual decoding.

Considering that 4x4 subblock zig-zag scanning has been removed from HEVC framework, reordering process from non-square to square data is not necessary anymore. This contribution provides a harmonization solution of unified scan and NSQT.

# Harmonization solution of unified scan and NSQT

## Scan for non-square transform

The general rule for the harmonization solution is to remove non-square to square data reordering process, and code non-square transform block as its shape. 32x8, 8x32, 16x4 and 4x16 non-square diagonal scan [3] are added CABAC residual process. A demonstration of non-square diagonal scan is shown at Fig. 1.

Figure 1: Illustration of 16x4 diagonal scan

## Derivation process of ctxIdxInc for significant_coeff_flag at non-square transform

Since non-square to square data reordering is used at current working draft, the derivation process of ctxIdxInc for "significant_coeff_flag" at 2Nx0.5N and 0.5Nx2N transform block is same as NxN transform block's process. At the harmonization solution, same procedure is used. The general rules for the derivation process of ctxIdxInc for "significant_coeff_flag" at 2Nx0.5N and 0.5Nx2N transform blocks are described as follows.

- 16x4, 4x16 transform blocks: same as 8x8 transform block's derivation process; sigCtx = 15..30; four neighboring coefficients share a context.

- 32x8, 8x32 transform blocks: same as 16x16 transform block's derivation process; sigCtx = 31..44; the update of context depends on the value of coefficients located at the right side and downside

## Derivation process of ctxIdxInc for last_significant_coeff_x and last_significant_coeff_y at non-square transform

Considering that the width and height of a non-square transform are different, the value of ctxIdxInc for last_significant_coeff_x and last_significant_coeff_y is chose separately. In detail, ctxIdxInc for last_significant_coeff_x is derived as follows:

- If the width is equal to 4, ctxIdxIncW =  lastCtx

- If the width is equal to 8, ctxIdxIncW =  lastCtx  +  3

- If the width is equal to 16, ctxIdxIncW =  lastCtx  +  8

- If the width is equal to 32, ctxIdxIncW =  lastCtx  +  15

ctxIdxInc for last_significant_coeff_y is derived as follows:

- If the width is equal to 4, ctxIdxIncH =  lastCtx

- If the width is equal to 8, ctxIdxIncH =  lastCtx  +  3

- If the width is equal to 16, ctxIdxIncH =  lastCtx  +  8

- If the width is equal to 32, ctxIdxIncH =  lastCtx  +  15

# Experimental results

The proposed method is implemented on HM4.0_rev.1354 software. The coding efficiency is evaluated in RA_HE, LD_B_HE and LD_P_HE configurations based on the common conditions defined in JCTVC-F900 [4]. The coding efficiency is measured by using BD-Rate. The experimental results are shown in Table 1.

Table 1

|  | Random Access HE |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | 0.0% | -0.1% | -0.2% |
| Class B | 0.0% | 0.1% | 0.0% |
| Class C | 0.0% | -0.1% | 0.0% |
| Class D | 0.0% | 0.0% | 0.1% |
| Class E |  |  |  |
| Class F | 0.0% | -0.1% | -0.1% |
| Overall | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% |
| Overall | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% |
| Enc Time[%] (without classF) | 101% |  |  |
| Enc Time[%] (with classF) | 99% |  |  |
| Dec Time[%] (without classF) | 101% |  |  |
| Dec Time[%] (with classF) | 99% |  |  |
|  | Low delay B HE |  |  |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | 0.0% | 0.2% | 0.2% |
| Class C | 0.0% | 0.0% | 0.1% |
| Class D | 0.0% | 0.1% | -0.3% |
| Class E | 0.1% | -0.3% | 0.6% |
| Class F | -0.2% | -0.1% | 0.0% |
| Overall | 0.0% | 0.0% | 0.1% |
|  | 0.0% | 0.0% | 0.1% |
| Overall | 0.0% | 0.0% | 0.1% |
|  | 0.0% | 0.0% | 0.1% |
| Enc Time[%] (without classF) | 101% |  |  |
| Enc Time[%] (with classF) | 98% |  |  |
| Dec Time[%] (without classF) | 101% |  |  |
| Dec Time[%] (with classF) | 98% |  |  |
|  | Low delay P HE |  |  |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | 0.0% | 0.2% | 0.4% |
| Class C | 0.0% | 0.0% | 0.0% |
| Class D | 0.1% | -0.1% | -0.6% |
| Class E | 0.0% | -0.5% | -0.2% |
| Class F | -0.3% | -0.2% | -0.2% |
| Overall | 0.0% | -0.1% | -0.1% |
|  | 0.0% | 0.0% | -0.1% |
| Overall | 0.0% | -0.1% | -0.1% |
|  | 0.0% | -0.1% | -0.2% |
| Enc Time[%] (without classF) | 101% |  |  |
| Enc Time[%] (with classF) | 99% |  |  |
| Dec Time[%] (without classF) | 101% |  |  |
| Dec Time[%] (with classF) | 99% |  |  |

# Conclusion

The proposed method at this document harmonizes unified scan and NSQT. The non-square to square data reordering process is removed. Meanwhile, no coding lost is observed under common test condition. 0.2% ~ 0.3% BD-bitrate reduction can be achieved for classF sequences under LD_B and LD_P configurations. The encoding time and decoding time for proposed method is almost same as HM4.0 anchor. It is recommended to incorporate the proposed harmonization solution to the next version of HEVC working draft and HM5.

# Reference

[1] J. Sole, R. Joshi, M. Karczewicz, “CE11: Unified scans for the significance map and coefficient level coding in high efficiency”, Doc. JCTVC-F288, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Torino, IT, July 2011.

[2] Y. Yuan (Tsinghua), X. Zheng (HiSilicon), X. Peng (USTC), J. Xu (Microsoft), I.-K Kim (Samsung), L. Liu (HiSilicon), Y. Wang, X. Cao (Tsinghua Univ.), C. Lai, J. Zheng (HiSilicon), Y. He (Tsinghua Univ.), H. Yu (Huawei), “CE2: Non-Square Quadtree Transform for symmetric and asymmetric motion partitions”, Doc. JCTVC-F412, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Torino, IT, July 2011.

[3] V. Sze, M. Budagavi, “CE11: Parallelization of HHI_TRANSFORM_CODING (Fixed Diagonal Scan from C227)”, Doc. JCTVC-F129, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Torino, IT, July 2011.

[4] Frank Bossen, “Common test conditions and software reference configurations”, Doc. JCTVC-F900, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Torino, IT, July 2011.

# Patent rights declaration(s)

Huawei Technologies, HiSilicon Technoligies and Tsinghua University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Appendix: Proposed change to WD text

### 7.3.10Rsidual coding CABAC syntax

| residual_coding_cabac( x0, y0, log2TrafoWidth, log2TrafoHeight, trafoDepth, scanIdx, cIdx ) { | Descriptor |
| --- | --- |
| last_significant_coeff_x | ae(v) |
| last_significant_coeff_y | ae(v) |
| n = 0 |  |
| xC = ScanOrder[ log2TrafoWidth − 2 ][ log2TrafoHeight − 2 ][ scanIdx ][ n ][ 0 ] |  |
| yC = ScanOrder[ log2TrafoWidth − 2 ][ log2TrafoHeight − 2 ][ scanIdx ][ n ][ 1 ] |  |
| while( ( xC != last_significant_coeff_x ) ¦¦ ( yC != last_significant_coeff_y ) ) { |  |
| significant_coeff_flag[ xC ][ yC ] | ae(v) |
| n++ |  |
| xC = ScanOrder[ log2TrafoWidth − 2 ][ log2TrafoHeight − 2 ][ scanIdx ][ n ][ 0 ] |  |
| yC = ScanOrder[ log2TrafoWidth − 2 ][ log2TrafoHeight − 2 ][ scanIdx ][ n ][ 1 ] |  |
| } |  |
| numSubsets = max( 1, ( 1 << ( log2TrafoWidth + log2TrafoHeight ) ) >> ( cIdx > 0 ? 6 : 4 )) |  |
| for( i = 0; i < numSubsets; i++ ) { |  |
| offset = i << 4 |  |
| xS = ScanOrder[ log2TrafoWidth − 2 ][ log2TrafoHeight − 2 ][ 0 ][ i ][ 0 ] << 2 |  |
| yS = ScanOrder[ log2TrafoWidth − 2 ][ log2TrafoHeight − 2 ][ 0 ][ i ][ 1 ] << 2 |  |
| for( n = 0; n < 16; n++ ) { |  |
| xOffset = n – (n >> 2 ) << 2 |  |
| yOffset = n >> 2 |  |
| if( significant_coeff_flag[ xS + xOffset ][ yS + yOffset ] ) |  |
| coeff_abs_level_greater1_flag[ n ] | ae(v) |
| } |  |
| for( n = 0; n < 16; n++ ) { |  |
| if( coeff_abs_level_greater1_flag[ n ] ) { |  |
| coeff_abs_level_greater2_flag[ n ] | ae(v) |
| if( coeff_abs_level_greater2_flag[ n ] ) |  |
| coeff_abs_level_minus3[ n ] | ae(v) |
| } |  |
| } |  |
| for( n = 0; n < 16; n++ ) { |  |
| xOffset = n – (n >> 2 ) << 2 |  |
| yOffset = n >> 2 |  |
| if( significant_coeff_flag[ xS + xOffset ][ yS + yOffset ] ) { |  |
| coeff_sign_flag[ n ] | ae(v) |
| transCoeffLevel[ x0 ][ y0 ][ trafoDepth ][ cIdx ][ n + offset ] = ( coeff_abs_level_minus3[ n ] + 3 ) * ( 1 − 2 * coeff_sign_flag[ n ] ) |  |
| } else |  |
| transCoeffLevel[ x0 ][ y0 ][ trafoDepth ][ cIdx ][ n + offset ] = 0 |  |
| } |  |
| } |  |
| } |  |

###### 9.3.3.1.1.3Derivation process of ctxIdxIncW for the syntax elements last_significant_coeff_x and ctxIdxIncH last_significant_coeff_y

Inputs to this process are the binIdx, the color component index cIdx, the transform block width log2TrafoSizeW and transform block height log2TrafoSizeH.

Output of this process is ctxIdxInc.

Table 9-1 – Specifcation of lastCtx[ i ]

| i | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| lastCtx | 0 | 1 | 2 | 3 | 3 | 4 | 4 | 5 | 5 | 5 | 5 | 6 | 6 | 6 | 6 | 7 |
| i | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 |  |
| lastCtx | 7 | 7 | 7 | 8 | 8 | 8 | 8 | 9 | 9 | 9 | 9 | 10 | 10 | 10 | 10 |  |

The vector lastCtx[ i ] is specified in Table 9-1. For the derivation of ctxIdxIncW and ctxIdxIncH, the following applies.

–	If log2TrafoSizeW is less than or equal to 2, ctxIdxIncW is derived as follows.

ctxIdxIncW =  lastCtx[ binIdx ]

–	Otherwise (log2TrafoSizeW is greater than 2), ctxIdxIncW is derived as follows.

ctxIdxIncW =  lastCtx[ ( 1 << ( log2TrafoSize – 1 ) ) + binIdx ]

When cIdx is greater than 0, ctxIdxIncW is modified as follows.

ctxIdxIncW =  ctxIdxIncW  +  19

–	If log2TrafoSizeH is less than or equal to 2, ctxIdxIncH is derived as follows.

ctxIdxIncH =  lastCtx[ binIdx ]

–	Otherwise (log2TrafoSizeH is greater than 2), ctxIdxIncH is derived as follows.

ctxIdxIncH =  lastCtx[ ( 1 << ( log2TrafoSize – 1 ) ) + binIdx ]

When cIdx is greater than 0, ctxIdxIncH is modified as follows.

ctxIdxIncH =  ctxIdxIncH  +  19

###### 9.3.3.1.1.4Derivation process of ctxIdxInc for the syntax element significant_coeff_flag

Inputs to this process are the color component index cIdx, the current coefficient scan position ( xC , yC ) , the transform block width log2TrafoSizeW and the transform block height log2TrafoSizeH.

Output of this process is ctxIdxInc.

The variable sigCtx depends on the current position ( xC, yC ), the transform block size and previsously decoded bins of the syntax element significant_coeff_flag. For the derivation of sigCtx, the following applies.

- log2TrafoSize = (log2TrafoSizeW + log2TrafoSizeH ) >> 1

- If log2TrafoSize is less than or equal to 3 and log2TrafoSizeW is equal to log2TrafoSizeH, sigCtx is derived as follows.

shift  	=  log2TrafoSize  = =  3 ? 1 : 0
sigCtx  =  ( shift  *  15 )  +  ( ( yC  >>  shift )  <<  2 )  +  ( xC  >>  shift )	(9-1)

- Otherwise, if log2TrafoSize is less than or equal to 3 and log2TrafoSizeW is larger than log2TrafoSizeH, sigCtx is derived as follows.

shift  	=  log2TrafoSize  = =  3 ? 1 : 0
sigCtx  =  ( shift  *  15 )  +  ( ( yC  >>  shift )  <<  3 )  +  ( xC  >>  shift )

- If log2TrafoSize is less than or equal to 3 and log2TrafoSizeW is less than log2TrafoSizeH, sigCtx is derived as follows.

shift  	=  log2TrafoSize  = =  3 ? 1 : 0
sigCtx  =  ( shift  *  15 )  +  ( ( yC  >>  shift )  <<  1 )  +  ( xC  >>  shift )

- Otherwise if xC + yC is less than 2, sigCtx is derived as follows.

sigCtx  =  31  +  ( yC  <<  1 )  +  xC		(9-2)

- Otherwise if xC + yC is less than 5, sigCtx is derived as follows.

temp  =  significant_coeff_flag[ xC + 1 ][ yC ]  +  significant_coeff_flag[ xC + 2 ][ yC ]  +  
		significant_coeff_flag[ xC ][ yC + 1 ]  +  significant_coeff_flag[ xC + 1 ][ yC + 1 ]  +  
		significant_coeff_flag[ xC ][ yC + 2 ]	(9-3)
sigCtx  =  34  +  Min( 4, temp )

- Otherwise (xC  +  yC is greater than 4), sigCtx is derived using previously decoded bins of the syntax element significant_coeff_flag as follows.

- The variable sigCtx is initialized as follows.

sigCtx  =  39		(9-4)

- When xC is less than ( 1 << log2TrafoSizeW ) − 1, the following applies.

sigCtx  =  sigCtx  +  significant_coeff_flag[ xC + 1 ][ yC ]	(9-5)

- When xC is less than ( 1 << log2TrafoSizeW ) − 1 and yC is less than ( 1 << log2TrafoSizeH ) − 1, the following applies.

sigCtx  =  sigCtx  +  significant_coeff_flag[ xC + 1 ][ yC + 1 ]	(9-6)

- When xC is less than ( 1 << log2TrafoSizeW ) − 2, the following applies.

sigCtx  =  sigCtx  +  significant_coeff_flag[ xC + 2 ][ yC ]	(9-7)

- When yC is less than ( 1 << log2TrafoSizeH ) − 1, the following applies.

sigCtx  =  sigCtx  +  significant_coeff_flag[ xC ][ yC + 1 ]	(9-8)

- When yC is less than ( 1 << log2TrafoSizeH ) − 2 and sigCtx is less than 43, the following applies.

sigCtx  =  sigCtx  +  significant_coeff_flag[ xC ][ yC + 2 ]	(9-9)

The context index increment ctxIdxInc is derived using the color component index cIdx and sigCtx as follows.

- If cIdx is equal to 0, ctxIdxInc is derived as follows.

ctxIdxInc  =  sigCtx		(9-10)

- Otherwise (cIdx is greater than 0), ctxIdxInc is derived as follows.

ctxIdxInc  =  44  +  sigCtx		(9-11)