| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G520r2 WG11 Number: m22084 |
| --- | --- |

| Title: | Non-CE11: Modified Context Derivation for last coefficient flag |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hisao Sasai Takahiro Nishi 1006 Kadoma, Kadoma, Osaka, Japan | Tel: Email: | +81 6 6900 9689 sasai.hisao@jp.panasonic.com |
| Source: | Panasonic Corporation |  |  |

_____________________________

# Abstract

This contribution presents a technique to reduce context size for last coefficient flag. In this contribution, a binarization process by truncated rice golomb code for last_coefficient_flag was proposed to define a suffix binarization part, then a context size reduction is proposed. The proposed solution was also implemented in HMv4 and their coding efficiency was evaluated. Experimental result shows the modification brought 16% (93 to 78) reduction of context size for last coefficient flags with less than 0.1% performance changes.

- Introduction

In this contribution, there are three aspects. First one is that a truncated Rice golomb code is used for suffix part of last coefficient flag. Second one is that reducing context size by using different rice parameter on each transform sizes. And third one is that bypass-concatenation for last coefficient flags.

- Proposed Solution

## Truncated Rice Golomb Coding for suffix part on last coefficient flag

In current HEVC[1], last coefficient flag is binarized by using truncated unary code for prefix part and fixed length code for suffix part except. The detail setting is as follows.

Table 1. last coefficient flag coding in HMv4.

| size | prefix-length | suffix-length |
| --- | --- | --- |
| 4x4 | 3 | 0 |
| 8x8 | 4 | 1 |
| 16x16 | 8 | 2 |
| 32x32 | 16 | 3 |

The fixed-length suffix part can be designated by truncated rice golomb code with rice parameter is 1, 2 and 3 for transform size 8x8, 16x16 and 32x32 respectively.

In this contribution, a clean definition of last coefficient flag coding is proposed as "truncated unary" for prefix part and "truncated rice golomb code" for suffix part.

## Rice parameter selection for each transform sizes

To reduce context size for last coefficient coding and improve throughput, the combination of prefix and suffix is changed as shown in Table 2.

Table 2. proposed binarization for last coefficient flag

| size | prefix-length | rice parameter |
| --- | --- | --- |
| 4x4 | 3 | - |
| 8x8 | 4 | 1 |
| 16x16 | 4 | 2 |
| 32x32 | 8 | 2 |

The current context indexes for each bin are assigned as follows;

4x4: 	 0, 1, 2
8x8:	 3, 4, 5, 5
16x16:	 6, 7, 8, 9, 10, 10, 11, 11
32x32:	12,13,14,15,16,16,16,16,17,17,17,17,18,18,18,18

In our method, following contexts are used;

4x4: 	 0, 1, 2
8x8:	 3, 4, 5, 5
16x16:	 6, 7, 8, 9
32x32:	10,11,12,13,14,14,15,15

The number of context can be reduced by 20% (93 to78).

## Changing coding order to concatenate bypass part

The suffix part of last coefficient coding is increased. Therefore, the bypass concatenation can be benefit to improve throughput [2].

Current coding order is prefix_x, suffix_x, prefix_y and suffix_y. To improve CABAC throughput, our roposed coding order is prefix_x, prefix_y, suffix_y and suffix_x by changing "suffix_x" position to the end. The coding order change has no significant changes.

- Results

The proposal method is implemented in HM4.0 and checked under the common test condition described in JCTVC-F900 [3]. The computing platform is Linux 64bits and the executables were compiled on 64-bit Linux with gcc 4.4.3. The simulation results will be updated when the simulation is finished.

BD BR and execution time is shown in Table 3.

Table 3. BD BR for our proposal on modified context derivation for last coefficient flag (high efficiency)

|  | All Intra HE |  |  | Random Access HE |  |  | Low delay B HE |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | 0.16% | 0.18% | 0.20% | 0.14% | 0.08% | 0.36% |  |  |  |
| Class B | 0.06% | 0.16% | 0.14% | 0.05% | 0.16% | 0.12% | 0.10% | 0.16% | -0.15% |
| Class C | 0.02% | 0.04% | 0.02% | 0.06% | -0.07% | -0.13% | 0.03% | -0.18% | -0.22% |
| Class D | 0.01% | 0.00% | 0.03% | 0.03% | -0.05% | -0.10% | -0.02% | 0.20% | -0.17% |
| Class E | 0.03% | 0.11% | 0.05% |  |  |  | 0.23% | -0.08% | 0.81% |
| Overall | 0.06% | 0.10% | 0.09% | 0.07% | 0.04% | 0.07% | 0.08% | 0.04% | 0.01% |
| Enc Time[%] | 100% |  |  | 100% |  |  | 101% |  |  |
| Dec Time[%] | 101% |  |  | 101% |  |  | 100% |  |  |

As additional result, BD BR and execution time for our proposal 2.1 and 2.2 is shown in Table 4.

Table 4. BD BR for our proposal on modified context derivation for last coefficient flag without context size reduction (high efficiency)

|  | All Intra HE |  |  | Random Access HE |  |  | Low delay B HE |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |  |  |  |
| Class B | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| Class C | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| Class D | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| Class E | 0.00% | 0.00% | 0.00% |  |  |  | 0.00% | 0.00% | 0.00% |
| Overall | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| Enc Time[%] | 100% |  |  | 100% |  |  | 101% |  |  |
| Dec Time[%] | 101% |  |  | 100% |  |  | 100% |  |  |

- Draft of WD Text

9.3.2.9 	Binarization process for last_significant_coefficient_x and last_significant_coefficient_y

Input to this process is a request for a binarization for the syntax element last_significant_coefficient_pos with pos being x or y and log2TrafoSize.
Output of this process is the binarization of the syntax element.
The variables cRiceParam and cTRMax are derived from as given in Table 9-.

Table 9-xx – Specifcation of cTRMax depending on cRiceParam

| log2TrafoSize | cRiceParam | cTRMax |
| --- | --- | --- |
| 2 | 0 | 3 |
| 3 | 1 | 4 |
| 4 | 2 | 4 |
| 5 | 2 | 8 |

The binarization of last_significant_coefficient_x and last_significant_coefficient_y is derived by invoking the TR binarization process as specified in subclause Error: Reference source not found with the variables cRiceParam and cTRMax as the inputs.

The prefix of the binarization is specified by invoking the TU binarization part of TR binarization. The suffix of the binarization is a rest of the binarization part.

Table 9-4 – Association of ctxIdx and syntax elements for each slice type in the initialisation process

|  | Syntax element | ctxIdxTable | Slice Type |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | I | P | B |
| residual_coding() | last_significant_coeff_x, last_significant_coeff_y |  | 0..2530 | 2631..5161 | 5262..7792 |

Table 9-5 – Values of variable m and n for last_significant_coefficient_x and last_significant_coefficient_y ctxIdx

| Initialisation variables | last_significant_coefficient_x and last_significant_coeff_y ctxIdx |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 119 | 1210 | 1113 | 1214 |
| m | 14 | 10 | 12 | 22 | 10 | 9 | 17 | 15 | 16 | 17 | 18 | 16 | 23 | 20 | 24 |
| n | 25 | 38 | 41 | 25 | 37 | 40 | 19 | 21 | 18 | 16 | 13 | 18 | -2 | -2 | -9 |
|  | 1315 | 1416 | 1517 | 18 | 1916 | 1720 | 2118 | 2219 | 2320 | 2421 | 2522 | 2623 | 2724 | 28 | 29 |
| m | 24 | 18 | 9 | 22 | 37 | 35 | 53 | 31 | 13 | 21 | 35 | 22 | 18 | 9 | 14 |
| n | -7 | 10 | 24 | -7 | 9 | 14 | -5 | 21 | 39 | 28 | 2 | 17 | 23 | 40 | 32 |
|  | 2530 | 2631 | 2732 | 2833 | 2934 | 3035 | 3136 | 3237 | 3338 | 3439 | 40 | 41 | 3542 | 3643 | 3744 |
| m | 7 | 12 | 12 | 13 | 21 | 24 | 23 | 13 | 22 | 29 | 29 | 27 | 20 | 25 | 30 |
| n | 44 | 35 | 40 | 47 | 16 | 10 | 15 | 34 | 10 | 10 | -8 | -5 | 7 | 13 | -8 |
|  | 3845 | 3946 | 4047 | 4148 | 49 | 4250 | 4351 | 4452 | 4553 | 4654 | 4755 | 4856 | 4957 | 5058 | 59 |
| m | 35 | 37 | 42 | 42 | 43 | 21 | 22 | 35 | 11 | 22 | 27 | -3 | 14 | 23 | 27 |
| n | -20 | -29 | -41 | -45 | -46 | 36 | 37 | 27 | 53 | 27 | 18 | 83 | 35 | 18 | 3 |
|  | 60 | 561 | 562 | 563 | 564 | 565 | 566 | 567 | 568 | 569 | 670 | 71 | 72 | 6173 | 6274 |
| m | 32 | 17 | 12 | 12 | 13 | 21 | 24 | 23 | 13 | 22 | 29 | 29 | 27 | 20 | 25 |
| n | -11 | 11 | 35 | 40 | 47 | 16 | 10 | 15 | 34 | 10 | 10 | -8 | -5 | 7 | 13 |
|  | 6375 | 6476 | 6577 | 6678 | 6779 | 80 | 6881 | 6982 | 7083 | 7184 | 7285 | 7386 | 7487 | 7588 | 7689 |
| m | 30 | 35 | 37 | 42 | 42 | 43 | 21 | 22 | 35 | 11 | 22 | 27 | -3 | 14 | 23 |
| n | -8 | -20 | -29 | -41 | -45 | -46 | 36 | 37 | 27 | 53 | 27 | 18 | 83 | 35 | 18 |
|  | 90 | 91 | 7792 | 93 | 94 | 95 | 96 | 97 | 98 | 99 | 100 | 101 | 102 | 103 | 104 |
| m | 27 | 32 | 17 |  |  |  |  |  |  |  |  |  |  |  |  |
| n | 3 | -11 | 11 |  |  |  |  |  |  |  |  |  |  |  |  |

| Table 9-6 – Syntax elements and associated types of binarization, maxBinIdxCtx, ctxIdxTable, and ctxIdxOffset |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| last_significant_coeff_x, last_significant_coeff_y | I | prefix and suffix as specified in subclause 4 | 26 |  | 0 |
|  | P |  | 26 |  | 3126 |
|  | B |  | 26 |  | 6252 |

#### Derivation process for ctxIdx

Inputs to this process are binIdx, maxBinIdxCtx, ctxIdxTable, and ctxIdxOffset.

Output of this process is ctxIdx.

Table 9-7 shows the assignment of ctxIdx increments (ctxIdxInc) to binIdx for all ctxIdxTable and ctxIdxOffset.

…

| Table 9-7 – Assignment of ctxIdxInc to binIdx for all ctxIdxTable and ctxIdxOffset values |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Syntax element | ctxIdxTable, ctxIdxOffset |  | binIdx |  |  |  |  |
|  |  |  | 0 | 1 | 2 | 3 | >=4 |
| last_significant_coeff_x, last_significant_coeff_y |  | 0 | 0..25 (subclause Error: Reference source not found) |  |  |  |  |
|  |  | 26 | 0..25 (subclause Error: Reference source not found) |  |  |  |  |
|  |  | 52 | 0..25 (subclause Error: Reference source not found) |  |  |  |  |

- Conclusion

In this contribution, a modified last coefficient coding method for CABAC is proposed. The proposed method can reduce context size by 16% (93 to 78). An experimental result shows no performance drop in average BD-rate. It is suggested to consider the inclusion of this modification for complexity reduction in the HM/WD of HEVC.

- References

- JCT-VC, “WD4: Working Draft 4 of High-Efficiency Video Coding”, JCTVC-F803, JCT-VC 6th Meeting, Torino, July, 2011.

- B. Shi, W. Zheng, H.-S. Lee, D.-X. Li, and M. Zhang, “Pipelined Architecture Design of H.264/AVC CABAC Real-Time Decoding,” in IEEE International Conference on Circuits and Systems for Communications, May 2008, pp. 492 –496.

- Frank Bossen, “Common test conditions and software reference configurations”, JCTVC-F900, Torino, July 2011.

# Patent rights declaration(s)

Panasonic Corporation may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).